CMTD – Configuring Monitoring and Troubleshooting Dialup Services

- Which two statements are true re: limiting VT access to router? (2)

Blocking outbound telnet on all physical interfaces prevents remote access

Since all VTY’s use telnet only one type of VTY ACL is necessary

You should always set identical restrictions on all virtual lines

Cisco routers support 4 simultaneous virtual sessions

- Which of the following two statements are true regarding 56kbps modems?

Speeds are limited to 53kbps by fcc regulations

Digital data is not converted back to analog data

Isdn framing of analog signals creates lower overhead

Out of band signaling increases available bandwidth

The download speed is faster than the upload speed

56kb modems are compatible with isdn but at a slower speed

- What is the function of the l2f protocol in virtual private networks?

User authentication

Network authentication

## Tunnelling link level protocols over higher protocols

Establishing multiple virtual paths to a remote destination

- VPDN enables service providers to?

Buy fewer routers

Increase bandwidth

Decrease broadcast traffic

#### Replace corporate dialup services

- What command is used to specify that any traffic causes an ISDN call?

Group dialer-ip all

Dialer-list protocol ip permit

Dialer traffic ip

- What is the command to require a password to access the console terminal?

Set pass console

Require password

Line 1 console

Line console 0

Enable password

- What are the three options for the dialer map command?

##### Name hostname

Chat-script

##### Ip address

Modem-script

System-script

##### Dial number

- Which command displays information about b1 and b2 channels?

##### Show int bri0 1 2

Show int isdn bri 2 1

Show int bri1 bri2

Show int bri1

- Configure dial backup for the indicated interface (1)

Backup dial interface-name

Backup dial interface interface-name

Dial-backup interface-name

Dial-backup interface interface-name

Backup interface interface-name

- Which two statements are true regarding the command telnet 1.2.3.4 2003

###### Establishes a reverse telnet connection

###### The data is sent to rotary group 3

The data is sent to line 1

The data is sent to rotary group 3 which is connected to a host or modem

- Which three functions does pat provide for cisco 700 series routers

Address translations

Ip address conservation

Remote host location on behalf of local clients

Firewall protection by hiding local clients

Downloads configuration to local clients

- Which command executes the menu named sales when the user jane logs in through any line

Username jane autocommand menu sales

###### Autocommand menu sales user jane

Autocommand menu    jane user

Username auto select user jane

- Which two characteristics apply to the pri but do not apply to the bri

PRI uses a csu/dsu

Uses a t1 for isdn

The channel is 56k

The speed is 1.544 or 2.048mb

- Regarding terminal connections, which one describes a reverse connection?

Incoming asynchronous line

Outgoing asynchronous line

Support for a remote host printing on the local network

Support for a remote host dialing into a asynchronous interface

- What command should you use to verify the current configuration that an access server line is using? ______Show line

- What are two options of the ISDN t1/e1 pri configuration command for framing?

Esf

crc4

Ppp

hdlc

- What two events can trigger a chat script to execute? (2)

Line reset

Dialer triggered by ddr

Interesting traffic

Routing information update

Tcp port address

- Why is ipx and spx spoofing important?

Spoofing enables to networks to be hidden from unauthorized users

Spoofing avoids constant updates that might keep expensive ddr lines in use

Spoofing allows clients to find less expensive routes to resources

Spoofing causes spx to respond as if it where ipx to cut down overhead

- How does xDSL achieve such a high data rate over the phone lines?

XDSL uses higher frequencies for data transmissions

Digital data is not converted back to analog at the service provider

Multiple Phone lines are used

.Statistical multiplexing is used.

- Which command allows you to troubleshoot your VPDN operation?

Show nas

###### Debug vpdn

Show vpdn events

###### Debug vpdn event

- You are configuring dial backup for primary links. Which command do you use to indicate the backup interface in the case of a primary link failure.

Backup ip interface-name

Backup dial interface-name

###### Backup interface interface-name

Backup dial interface interface-name

- which function on a cisco access server draws ibm 3270 screens by turning ibm directives into dumb terminal commands

ttycap

keymap

keymap-type

emulation

- What is the operation of a dhcp servers

Dhcp manage ip addresses and assign ip configuration parameters at client request

Dhcp servers hide internal addresses from the outside world

Dhcp servers keep a database of network routers that can be used by other routing protocols

Dhcp servers act on behalf of network clients to find routes to remote networks

- Assuming you have just configured an asynchronous interface below as a dial backup line, active the line when the load for the primary interface reaches 90% and deactive when the combined load in both directions reaches 40%.Type the command: ______backup load 90 40_________

- Which examples are two options that modem auto-configuration is necessary.

Modem using called ID

Modem reinitilization after failure

Configuring a modem from a central location

Modem configuration without using modem configuration commands

- You are configuring a static route on a router. You would like to configure it so that if the interface associated with the route goes down the route will still remain in the routing table. Which ip routing parmeter causes this?

Keep

Permanent

Continuous

Backup

- What is a profile and how is it used with a cisco 700 series router

A configuration customized for a specific remote device

Access privileges associated with users

Access privileges to routing services

Access privileges associated remote system

Access privileges to the local network

###### Configuration files uses can download to their router for quick setup

- Which three access servers are used in the access path integrated access solution

As5200

Cisco 3600

Cisco 2511

Cisco 1600

Cisco 901

- What is the key advantage of chap over pap?

Chap never sends a visible password

Chap authentication takes fewer steps

Pap has a higher overhead

If authentication fails at any time pap drops the call

- Which signal indicates that the DTE is available to accept a call?

Dsr up

Rts up

Cts up

Dtr up

- Type the complete command to display a list of servers discovered through sap advertisements? _______

Show servers

- In addition to viewing routing tables, what command can you use to determine a router is using the best path when forwarding a packet to a specific network?____trace___

- What is the purpose of a dialer rotary group?

Define interesting traffic

Define ddr connection
Configure dial backup connections

###### Apply a single interface configuration to mulitple interfaces

- Which two services are available through Cisco NAT?
Address resolution

Domain lookup

Tcp load distribution

Specific address filtration

Static address translation

- Which resource node services link layer protocol provided by cisco access servers is preferred because it has low overhead, available compression, multiple protocol support, good security options

PPP

ALL5

ARAP

SLIP

- Which are the two frame types for T1 lines?

Super frame

Extended superframe

Q39.1T

Dma

- How does NAT allow for more hosts than there are available ip addresses

Manipulating the ip authentication field

Never using the same address twice

Modifying tcp port numbers

Restricting the number of hosts allowed to communicate on the network simultaneously

- What is true regarding default routes?

They are always specified by an individual host

They always have static routes

They are not available for isdn

They must be configured at the end of each link

- Ppp defines which two authentication protocols?

Pip pap

Pop pap

Pap chap

Pop arp

- What is the effect of the following configuration line?

Dialer load-threshold 128 either

Another line will be dialed when inbound utilization reaches 128kbps

Another line will be dialed when outbound utilization reaches 128kbps

Another line will be dialed when bidirectional utilization reaches 128kbps

Another line will be dialed when bidirectional utilization reaches 50%

- Which command sets up modem autodiscovery?

Modem auto-discovery

#### Modem autoconfigure discovery

Autoconfigure modem discovery

- what prevents tunnelling of the link layer frames through a vpdn?

l2f

l2e

lte

ltf

- What characteristics describe remote node instead of remote control? (2)

Provides access to all network resources

Host end run at lan speed

Does not require a dedicated host at the remote site

Provides good performance with legacy applications

- What command associates an access list with a dialer group

Dialer-list listnum

Dialer-group list Access-list

Dialer-list    groupnum list listnum

Dialer-group protocol protocol-list Access-list

access-list dialer-group groupnumber

- Which of the following are ISDN reference points

###### The user reference point

The routing reference point

###### The terminal reference point

The data reference point

- What is the command to initiate the vpdn at the isp nas? (2)

Vpdn enable

Vpdn outgoing

Enable vpdn incoming

Enable vpdn outgoing

- What command will enable a dialer rotary group

Dialer group

Dialer interface

Interface dialer

Dialer rotary-group

- Why would the access list specified as access-list 101 deny igrp any any save money when used with other access lists on a ddr interface?

Only routing packets will bring up the line

Routing packets will not bring up the line

#### All traffic but igrp will bring up the line

No traffic will bring up the line

- Where all the logging messages directed by default?

###### Console terminal

Auxilary terminal

Tty1 auxilary port

Console port

- Which of the following scenarios is an example of    protocol translation?

Connect to the access server using telnet to another host use telnet

Connect to access server using telnet then use lat

Connect to an access server using Ppp and use chap to authenticate

Connect via tn3270 then telnet destination

- Which command allows the remote dial in client to enter its own ip address?

Async static-address

Async dynamic address

Remote ip

address ip unnumbered

- What process normally occurs when you use NAT

Your address is always translated to the same address

Your address is always translated to a different address

Your address is incremented round robin

Your address is dynamically choosen through a pool

- Clear dynamic NAT translation entries:

Clear ip nat translation

Clear ip nat translation*

- Create static route for all traffic to net 150.27.0.0 to go out BRI0: ____________

Ip route 150.270.0.0    255.255.0.0 bri0

- Which command will configure ddr to multiple destinations?

Dialer map interface

Dialer-map

Ddr-map

Interface dialer map

- 3 functions PAT provides:

address translation

ip address conservation

remote host on behalf of local clients

firewall

download config

- Where is logging    directed by default:

VTY1

# Console

Syslog host

There is no default

- Which feature of the Cisco IOS can be used to cut connection costs?

DDR

PPP Callback

Shut off router

IPX/SPX spoofing

- Which command associates an access list with a dialer group?

# Dialer-list group-number protocol protocol-name {permit|deny|list access-list-number}

Dialer-list group-number protocol-name {permit|deny|list access-list-number}

Dialer-list group-number protocol-name access-list {permit|deny}

- Which command would you use to troubleshoot VPDN operation?

Show nas

Debug VPDN incoming

Show VPDN event

# Debug vpdn event

- What is modem autoconfigure used for?

Initialization after failure

Configure remote modems

# Auto discovery of modem type

Modem configuration with using commands

- Framing for T1’s

# SF/ESF

PPP/HDLC

SLIP/PPP

PAP/CHAP

- What signal indicates DTE ready for call?

DSR

RTS

CTS

DTR

- What are 2 options for the ISDN T1/E1 PRI configuration command “framing”

Esf

Ppp

Hdlc

# Crc4

- Which of the following are ISDN reference points:

# User reference

Routing reference

# Terminal reference

Data reference

- Which two terminal services are provided by Cisco access servers?

telnet

slip

ISDN

# LAT

- What is the command to activate the line when the primary interface reaches 90%, and deactivate when the line reaches 40%: _____backup load 90 40____

- What is a profile and how is it used on a cisco 7000

A operating system dump used by Cisco TAC

# Portable config file used for quick setup

A configuration wizard that you may use as a starting point

- What does a VPDN allow you to do?

Establish a connection between routers when interesting traffic is present

- Dialer load-threshold 128 either

Another line will be brought up after 50%

- Display info on B1 and B2:

Show int bri 1

- How does NAT allow more hosts than there are available IP addresses?

Manipulating IP identification field

# Modify TCP port

Restricting the number of hosts allowed to communicate simultaneously

- Advantages of MPP

# Increased bandwidth

# Multiple physical interfaces can be cascaded

Supports strong encryption types

Multiple routes for redundancy

- Enter the command that sets up modem auto-discovery

# Modem autoconfigure

- Why 56k modems work

Uses ISDN signaling

# No analog conversion at CO

Uses frame relay packet switching technology

New FCC regulations allow higher frequencies

- What command specifies all IP traffic triggers an ISDN call

Dialer list 1 protocol ip permit

- What command executes menu named sales for Jane:

# Username jane autocommand menu sales

- Why is ipx/spx spoofing important?

Avoids DDR overuse

- What command will enable dial-on-demand routing on any asychronous interface?

Dialer in-band

Backup interface

Dialer dynamic routing

Dialer enable

- What command displays modem configuration during auto-configuration ?

# Debug line

Debug autoconf

Debug line field

Debug conf modem

- 3 features supported by T1 interface in AS5200?

Channelized T1

# PRI

Only single t1 line

# A single card w/Dual T1’s

# Up to 60 DS0 channels

- VPDN is a way for service providers to:

# Replace dedicated corporate dialup service

Buy fewer routers

Increase bandwidth

- Features which are available in remote node technology which are not present in remote control?

# All network resources are available

Host end runs lan speed

# No dedicated host

Good performance with legacy apps

- Features which are available in remote control technology which are not present in remote node?

# All network resources are available

# Host end runs lan speed

No dedicated host

# Good performance with legacy apps

- What does PPP use to establsih and config. Data link connections:

PPP

NCP

CHAP

LCP

- Describe a reverse terminal connection:

Incoming asynchronous line

# Outgoing asynchronous line

- Why is PPP callback used?

Bill consolidation, cost cutting.

Line selection and reliability

Ease of config

End user savings

- Overload is when NAT experiences:

more hosts than addresses

excessive CPU utilization

no more static translations

messages to fast for console

- Describe telnet 1.2.3.4 2003 (3)

# The command establishes a reserve telnet connection

# The data is sent to rotary groups

The data is sent to line 1 which is connected to a modem

# The data is sent to the rotary group 3 which is connected to a modem or host

- Configure a terminal with no default protocol:

# No transport protocol

Transport protocol none

# Transport default null

Transport disable

- Give an example of Protocol translation

###### LAT to telnet

Telnet to Telnet

PAP to CHAP

- When Cisco access servers allows another terminal to emulate an IBM 3270 terminal type what two types does the access server assume are attached (radio button):

tn 3270

wyse 300

vt100

vt200

- How does xDSL achieve such a rate

digital data not converted to analog

uses ISDN signaling over standard copper

- What permits tunneling of link layer frames with a VPDN:

LZF

LZE

L2E

# L2F

- What command initiates VPDN at ISP NAS

VPN enable, VPDN accept

VPN enable, VPDN incoming

Enable VPN, VPDN accept

# Enable VPN, VPDN incoming

- DHCP is used for:

Dynamically assigning a mac address to a computer

## Dynamically assigning an ip address to a computer

Dynamically booting the computer from the network

- Which of the following characteristics are associated with BRI’s?

16kb D channel

64kb D channel

# 2B+1D

23B+1D

- Which of the following characteristics are associated with PRI’s?

16kb D channel

64kb D channel

# 2B+1D

# 23B+1D

- Which concern is increasing likely to become more important if a network grows and wan links are added? (1)

The 16 hop limitation of IPX

### The amount of network broadcast traffic becoming signficant

The limitations of IPX addressing

Non routable novell protocols

- What 3 commands will create a connection to a remote host from the router exec prompt?telnet ip-addresspad hostnameconnect hostnamepad hostname You are configuring multilink PPP. Which command do you use to configure the dialer interface if using dialer profiles

Config ppp

Config multilink

Set multilink

### Ppp multilink