ACRC

- How many hosts/networks are available in the netmask 255.255.192.0

192 subnets / 255 hosts

192 subnets / 192 hosts

4 Subnets / 16382 hosts

# 2 Subnets / 16382 hosts

- How many hosts/subnets are available in the netmask 255.255.255.240

16 subnets / 32 hosts

16 subnets /16 hosts

32 subnets / 16 hosts

32 subnets / 32 hosts

- What is the purpose of a poison sap?

To flush the entire sap table

A poison sap is a type of IPX/RIP update

# To remove a single entry from a sap table

There is no such thing as a poison sap

- What command will show all interfaces with IPX configured?

Show interfaces ipx

# Show ipx interfaces

Display ipx

Show protocol ipx

- What command will display all the IPX servers learned?

## Show ipx servers

- How do you allocate more bandwidth to a specific protocol’s traffic using custom queueing

Use the Priority-list command

Use the set-queue-priority command

Atually neither of these choices have correct answer specified in ACRC mannual

Queue-list list-no queue queue-no byte-count byte-count-no.

- Identify the true statements about EIGRP (2)

EIGRP supports areas

# EIGRP supports VLSMs

EIGRP requires considerably more configuration than IGRP

EIGRP keeps all protocols in one table

# EIGRP only sends notifications to the systems that are affected by a change

- What command will display all the IPX routes on a router?

## Show ipx routes

- Which Cisco keyword(s) describes the 802.3 frame format(s)?

ARPA

# NOVELL-ETHER

SNAP

HDLC

## SAP

- What IOS features solve the problem of large routing tables? (2)

Compression

Route summarization

Incremental updates

Access Lists

Priority queuing

Bridging

- What command will show all frame relay map assignments currently configured?

Show frame-relay lmi

## Show frame-relay map

Show frame-relay interface

Show frame-relay dlci

- What is the SAP type for a IPX file server?

Answer: 4

- Identify the proper order poison sap handling?

[A] When the one-minute timer expires, and if the router hasn't received a new good metric for the service, the service is removed from the table

[B] The router marks the entry as poisoned and sets a one-minute timer.

[C] The router receives a poison SAP.

[D] The router immediately generates a poison SAP packet for this service out all other interfaces.

DCBA

BDCA

# CBDA

ABCD

- How does a Cisco router pick the server to include in a Get Nearest Server response?

The server with the lowest MAC address is selected

The server of the requested type with the lowest hopcount is considered the "nearest" server.

The server with the least amount of CPU utilization is selected

The server is randomly selected from the server SAP table

- Identify the valid IPX address(es)

1.00.1000.de78

10000.1010.1000.303F

1001.1001.1010

3cf.1003.0103.3030

- What is the maximum hop count of a IPX RIP packet?

#### Answer: 15

- What is the SAP type for an IPX print server?

Answer: 7

- How many hosts/subnetworks are available in the netmask 255.255.255.224

32 subnets / 8 hosts

30 subnets / 16 hosts

24 subnets / 24 hosts

12 subnets / 32 hosts

- Which command would you use to determine if the “bad hop count” is incrementing on routers running IPX? (when spanning tree is disabled)

Debug ipx

Debug ipx loop

Show ipx interfaces

## Show ipx traffic

- If your network uses RIP, but not all interfaces require RIP, how do you prevent distribution of routes without using access lists? (1)

Default Routes

Passive interface

Static routes

Routing update Filter

- Which 2 statements regarding the following configuration are true:

router eigrp 110

Network    2.0.0.0

Network 3.0.0.0

Line 1 defines EIGRP as a routing process

Line 2 defines EIGRP as a routing process

Line 2 causes all interfaces connected to send information about 2.0.0.0

All networks supporting EIGRP should be listed in each EIGRP router

- What command enables IPX on a router? __________

## Ipx routing

- How many class C’s can be summarized with the route 176.12.172.0/20

4

8

16

20

32

64

- Identify the two true statements regarding the command: source-bridge transparent 20 23 1 4

Virtual SRB Ring is defined as Ring 20

Pseudo ring 23 is the transparent domain

The    1 will remove the this Virtual interface from the Spanning Tree

- You have just completed an upgrade to the Cisco IOS version 10.3 , access control lists work exactly how you had them setup prior to the upgrade. You now add two additional access lists using the new functionality of the 10.3 operating system, you then are forced to perform a downgrade of the IOS version back to the previous version. What happens to the new Access list entries?

Original access list entries will work, two new ones will be misinterpreted

Original access list entries will be corrupt and will not work

All access lists will work as expected

- Identify the valid network/broadcast address range for the ip address 198.168.1.1/26 (1)

192.168.1.0/192.168.1.32

192.168.1.32/192.168.1.63

192.168.1.64/192.168.1.127

192.168.1.96/192.168.0.223

- Which of the following scenarios requires BGP routing (2):

Need to make decision based on source and destination of internal traffic within an AS

Connections to different Internet Service Providers

Security Concerns require that you must filter all but three networks from the Internet

The ISP you connect to uses BGP

- How would you control SAP traffic across a WAN link? (1)

Disable GNS on some netware servers

Increase the SPX watchdog

USE EIGRP for SAP udpates

- List three options on IP extended access lists which are not found on standard access lists? (3)

Session layer information

Host name

Destination IP

Application port #

User login name

Source network ip

Source host IP

- Approximately how much address space is wasted by using a netmask 255.255.224.0?

10%

25%

50%

75%

- Which statement about EIGRP is True? (4)

Keeps copies of its neighbors tables

Uses forward broadcasts to discover routers

Adjancies exist between (MR) master routers

Eigrp provides support for other network layer protocols such as IPX & IP

EIGRP keeps 1 table per protocol

EIGRP does not make periodic updates

- Which problems are associate with a high number of routers in a single OSPF area: (2)

Excess LSA traffic

Frequent table recalculation

Frequent Adjacencies table recalculation

More reachability errors

- When should you use a null interface instead of an access list? (1)

When no Access List options for require filtering exist

You want to use host names rather than IP addresses

You have a need to conserve CPU resources

You cannot filter non-routable protocols

- Which two address ranges are private:

192.167.0

172.16.0

172.68.0.0

192.168.0

- as IPX networks grow what becomes important?

Broadcasts

Limitations of IPX

Non-routable netware protocols

IPX 16 hop limit

- Actions which increase congestion: (3)

Adjusting preset timers

Increasing buffers on router

Increasing buffers on server

Filtering broadcasts

Filtering users and applications

- Identify the true statement about IPX ticks

In IOS 9.21 and later IPX ticks are used like a hop metric

# In IOS 9.21 and later IPX ticks are used as a bandwidth metric

To set the tick delay use the command “ipx delay #ticks”

To set the tick delay use the command “ipx tick-delay #ticks”

- Which command would you use to display the routing info. Field data of Token ring frames passing through the router?    ___________

debug rif    or show rif

- command to specify encapsulation to carry traffic through an IP tunnel: _

tunnel mode {aurp|cayman|dvmrp|eon|gre ip | nos}_

- Type the command to show status and parameters on all IPX interfaces: ______

show ipx interface__

- What address range is the ip address 191.168.1.1

Class A

# Class B

Class C

This is not a classful address

- Which of these three statements regarding OSPF are true (3)

Can span +15 hops

Sends full routing table on updates

Path can be based on throughput

Ring updates are multicast

Effectively replaces RTMP, IPX, RIP, and IP RIP

- When redistribution is configured, minimize the chance of selecting sub optimal paths?

Priority queue

Passive interface

Access-lists

Default route

- Which queue method provides automated allocation to all net traffic based on messages and conversations: custom queueing, FIFO, priority queuing, weighted fair queuing

Answer is authenticated by ACRC book

- OSPF is better than RIP in large networks because:

It uses less RAM on the router

No reachability limits

Less complex than RIP

Supports VLSMs

- What is an ABR responsible for?

Connecting multiple OSPF areas

- Which statements are ture about the following commands:

Interface BVI 30

Ip address 1.2.3.4 255.255.255.0

Bridge IRB

Bridge 30 route ip

## This configuration uses IRB

Line 1 creates VRI to rep. Group 30

Line 2 specifies IRB protocol

Line 3 specifies IRB protocol

## Line 4 enables the BVI to Accept and route ip

- Type the command that will set bridge 4 to use source routing briding from ring 22 to ring 33: _________

### Source-bridge 22    1 33

- True about non-routable protocols: (1)

no FCS in the header

No network layer addressing

Use broadcasts to determine best route

Should not be used with WAN links

- What is the command which verifies OSPF interfaces are configured in the proper areas and displays all the adjacent neighbor names: ______________Show ip OSPF interfaces

- Which two statements are true about the following configuration:

Interface ethernet 0

Ip address 148.19.1.77 255.255.255.0

Ip helper-address 148.19.90.255

Host 148.19.90.255 is a backup router for 148.19.1.77

Bootp requests on interface ethernet0 will be forwarded to network 148.19.90.0

All non routable protocol traffic will be forwarded to network 148.19.90.0

Netbios broadcasts fromn net 148.19.90.0 will be sent as direct broadcasts to 148.19.90.0

- What are the mandatory commands to configure OSPF?

Router ospf; network address-mask area-id

Router ospf pid; network adddress-mask area-id

Router ospf PID; network-address wildcard-mask area area-id

- What command would you enter to show all known IPX servers regardless of the interface they were learned from.

##### Show ipx servers

- If IP RIP routing is configured on a router but not all interfaces are attached to rip networks what can you use to prevent all rip routing updates being sent through selected interfaces without using access-lists?

## Passive interface

Default routes

Static routes

Route update filtering

- Order of transmission in priority queuing

Higher priority queues are empited before lower priority queues

All queues use a round robin method to effect

Time-division

- Correct OSPF Configuration

(It had many similar Example Configurations)

router ospf pid 100

network 10.1.1.1

network 192.213.0.0 0.0.255.255 0.0.0.0

network 128.213.1.1 0.0.0.0 23

What is wrong with this configuration?

the netmask specified is not in the proper format

# The word “area” is missing from the network statements

The area of 23 is invalid

# The word “pid” should not be present in the router ospf statement

The statement 128.213.1.1 0.0.0.0 is not valid

- What is the enable password used for?

To allow the current session to login to the router

To allow the current session to connect to the router

#### To allow the current session to reconfigure the router

- Type the full the command to display access list 101?:

show ip access-list 101

- What is wrong with the following ip extended access list configuration? (1)

Access-list 22 permit ip 120.33.44.55 0.0.0.255 0.0.0.0 255.255.255.255

Destination mask specifies a broadcast

The source mask is illegal

The mask for both destination and source are the wrong ip address class

##### The access list number is incorrect

There are too many parameters on the access list line.

- Which of the following is included in an ospf link state database?

### Information about a routers directly networks collective links

### Information about an entire internetwork topology

Information about a routers directly connected neighbors (Adjacecies database)

Information about the best path a router can use

- What is the command to show EIGRP routing tables? ________

show ip eigrp topology or show ip eigrp route

- How can a single IP represent many?

Default routes

Static routes

# Route summarization

Route expansion

- Besides viewing routing tables verify a router is using the best path to forward packets on network: ________

#### Traceroute

- Bridge types and characteristics

## Store and forward – does not forward error

Store and forward – low overhead

Cut through – does not forward errors

# Cut through – low overhead

- Which two of the following statements regarding these sample configuration are true

Router eigrp 99

Network 2.0.0.0

Network 3.0.0.0

The AS 99 is invalid

# The eigrp process is running in autonomous system 99

The network statements are missing the netmask statement

# The network 2.0.0.0 is including in autonomous system 99

- Which two of the following statements regarding these sample configuration are true

Router eigrp100

Network 10.0.0.0

Network 11.0.0.0

##### Line 1 defines eigrp as an ip routing process

##### The number following line indicates the routing process ID

Line 2 causes all interfaces connected    to the network to send enhanced igrp updates to the other eigrp routers

All networks supporting eigrp should be listed in each eigrp router

- Which two responses explain why ospf is better than rip in a large network? (2)

### Ospf has virtually no reachability limits

Ospf is Less complex rip

Ospf has fewer tables to manage

Ospf selects that best path using a metric that is based on bandwidth