Chillnz Notes

OSI Reference

Identify and describe the functions of each of the seven layers of the OSI reference model.

Application, Presentation, Session, Transport, Network, Data-link, Physical

Describe connection-oriented network service and connectionless network service, and identify the key differences between them.

Describe data link addresses and network addresses, and identify the key differences between them.

Identify at least 3 reasons why the industry uses a layered model.

To divide the interrelated aspects of network operation into less complex elements

To define standard interfaces for “plug-and-play” compatibility and multivendor integration

To enable engineers to specialize design and development efforts on modular function enabling new applications and services to be deployed without redesigning each lower layer.

To promote symmetry in the different internetwork modular functions so they interoperate

To prevent changes in one area from affecting other areas so each area can evolve more quickly

To divide the complexity of internetworking into discrete, more easily learned operation subsets.

Define and explain the 5 conversion steps of data encapsulation.

Build the data

Package data for end-to-end transport

Append network address in header

Append Local address in data-link header

Convert to bits for transmission

Define flow control and describe the three basic methods used in networking.

List the key internetworking functions of the OSI Network layer and how they are performed in a router.

WAN Protocols

Differentiate between the following WAN services: Frame Relay, ISDN/LAPD, HDLC, & PPP.

Recognize key Frame Relay terms and features.

List commands to configure Frame Relay LMIs, maps, and subinterfaces.

List commands to monitor Frame Relay operation in the router.

Identify PPP operations to encapsulate WAN data on Cisco routers.

13) State a relevant use and context for ISDN networking.

14) Identify ISDN protocols, function groups, reference points, and channels.

15) Describe Cisco's implementation of ISDN BRI.

IOS

16) Log into a router in both user and privileged modes.

> enable

17) Use the context-sensitive help facility.

?

ip ?

ip route ?

18) Use the command history and editing features.

show history

19) Examine router elements (RAM, ROM, CDP, show).

RAM – Stores running-configuration

NVRAM - Stores startup-configuration

ROM – Stores bootstrap and monitor system

Flash – Stores IOS

CDP – Cisco Discovery Protocol

# show cdp neighbors

(c)#  cdp holdtime [seconds]

(c)# no cdp run

(c-if)# no cdp enable

(c)# cdp timer [seconds]

# show cdp entry [device-name]

20) Manage configuration files from the privileged exec mode.

Write terminal

Write memory

21) Control router passwords, identification, and banner.

Enable secret – encrypted password

Enable password – unencrypted password

Hostname – sets host name

Prompt – changes router display name – does not change name

(c)# Banner motd – sets message-of-the-day banner

(c-if)# Description – descrives an interface

22) Identify the main Cisco IOS commands for router startup.

23) Enter an initial configuration using the setup command.

24) Copy and manipulate configuration files.

25) List the commands to load Cisco IOS software from: flash memory, a TFTP server, or ROM.

Copy flash tftp

Copy tftp flash

26) Prepare to backup, upgrade, and load a backup Cisco IOS software image.

Copy tftp flash

Copy flash tftp

Ctrl-shift-6 to cancel process

27) Prepare the initial configuration of your router and enable IP.

Network Protocols

28) Monitor Novell IPX operation on the router.

29) Describe the two parts of network addressing, then identify the parts in specific protocol address examples.

30) Create the different classes of IP addresses [and subnetting].

A - 00001-12625416777214B - 1000128-1916553465534C - 1100192-22316777214254D - 1110224-239E - 1111240-255

31) Configure IP addresses.

(c-if)# Ip address [ip-address] [mask]

# Term ip netmask-format {bitcount|decimal|hexadecimal} – sets format of mask for current session

(c-if)# ip netmask-format {bitcount|decimal|hexadecimal} – sets format of mask for specific line

(c)# ip host name [tcp-port-number] address [address] – host name-to-address mapping

(c)# ip name-server server–address1 [server–address2] [server–address6] – up to 6

(c)# ip domain-name [name of default domain] – DNS resolution

(c)# no ip domain-lookup – turn off console name resolution

(c)# ip domain-lookup – re-enable

32) Verify IP addresses.

Show ip interface – shows a summary of interface IP information.

Show hosts [host name] – shows cached host names

33) List the required IPX address and encapsulation type.

Novell IPX NameCisco IOS NameEthernt_802.3

Ethernt_802.2

Ethernt_II

Ethernt_SNAPNovell-ether

Sap

Arpa

SnapToken-Ring

Token-Ring_SNAPSap

SnapFDDI_SNAP

FDDI_802.2

FDDI_RawSnap

Sap

Novell-fddiDefaults:

Ethernet – novell-ether

Token Ring – sap

FDDI – snap

Serial – hdlc

34) Enable the Novell IPX protocol and configure interfaces.

Ipx routing [node] – Enables IPX routing, if no code address is specified, the router uses the MAC address of the interface.

Ipx maximum-paths [paths] – Enables load sharing; default is 1 (disabled)

Ipx route {network [mask}|default} {network.node|interface} [floating-static] – 10.3 and later – not sure

Ipx network [network] [ encapsulation encap-type [secondary]] – Defines primary and secondary networks; secondary will not be supported in future IOS releases.

Information from IPX

Show ipx interface – IPX status and parameters

Show ipx route – Routing table contents

Show ipx servers – IPX server list

Show ipx traffic – number and type of packets

Debug ipx routing activity – Information about RIP update packets

Debug ipx sap – Information about SAP update packets

SAP packet types (0for debug ipx sap)

0x1 – General query

0x2 – Gneral response

0x3 – Get nearest server request

0x4 – Get nearest server response

More configuration:

Ipx delay [number] – sets tick metrics; over rides defaults – LAN interfaces = one tick, WAN interfaces = six ticks.

Ipx input-sap-filter

Ipx output-sap-filter

Ipx router-sap-filter

Ipx route-caching – used to set IPX fast switching status; enabled by default

35) Identify the functions of the TCP/IP transport-layer protocols.

Provides flow control by sliding windows

Reliability provided by sequenced number and acknowledgements

TCP – Connection oriented – Responsible for breaking down messages into segments, reassembling them at the destination station, resending anything that is not received, and reassembled messages from segments.

TCP supplies a virtual circuit between end-user applications.

UDP – Connectionless and unacknowledged – Fast and efficient, does not divide data into pieces, reliability is handled by upper layer protocols.

TCP and UDP use ports to pass information to upper layers.  Conversations that do not involve an application with a well-known port number are assigned port numbers randomly from a range.

Ports:

Below 256 – public access

256- 1023 – assigned to companies for saleable applications

above 1023 – dynamically assigned by the host

It is possible to access the same port number multiple times, example: telnet.  A client is accessing port 23 on the host, telnet selects an unused port number above 1023 to represent the source port.  The destination stays at 23.

Sequence numbers are used to track the order of communication and to ensure that there are no missing pieces of data in a communication that requires multiple packets.

TCP Handshake:

The initial sequence number(ISN) is the starting number used when the TCP connection is established.

Synchronization is accomplished by exchanging segments carrying the ISNs and a control bit called SYN.

Synchronization requires that each side sends its own ISN and receive a confirmation and ISN from the other side of the connection.

Each side must confirm the ISNs in a specific order

A [] B SYN my sequence number is X

A [] B ACK your sequence number is X

A [] B SYN my sequence number is Y

A [] B ACK your sequence number is Y

Three-way handshake:

Device1: “I would like to talk to you”

Device2: “Good. I want to talk to you”

Device1: “Fine lets talk”

The three-way handshake is required because device2 has no way of knowing if the segment was an old one, so it must ask the sender to verify.

TCP is a peer-to-peer (balanced) communications method.

Windowing:

The purpose of windowing is to improve flow control and reliability.

The window size determines how much data the receiver can accept at once; With a window size of 1, there is a inefficient use of bandwidth.

TCP uses a flow control mechanism.

The TCP receiver reports a window to the sending TCP.

This window specifies the number of bytes that the receiving TCP is currently prepared to receive.

[WS=3 Send=1] Device1  Device2

[WS=3 Send=1] Device1  Device2

[WS=3 Send=1] Device1  Device2

[WS=2 ACK=4]  Device2  Device1

[WS=3 Send=4] Device1  Device2

[WS=3 Send=5] Device1  Device2

[WS=2 ACK=6]  Device2  Device1

TCP window sizes are variable during the time of the connection.

Each ACK indicates how many bytes the receiver can accept.

TCP also maintains a congestion window , which is normally the same size as the receiver’s window but is cut in half when a segment is lost  (i.e. when there is a congestion).

This approach permits the window to be expanded or contracted as necessary to manage buffer space and processing.

Windowing is RFC 793 and 813

Sequencing:

TCP numbers each segment so they can be re-assembled as the receiver.  If a segment is missing in the series or is they are not ACKed they are retransmitted.

TCP provides full-duplex communication and sequencing is done bi-directionally.

UDP:

UDP has no windowing or ACKs.

Applications-layer protocols must provide the reliability.

UDP is designed for applications that do not need to put sequences of segments together.

TFTP uses UDP and a checksum.  If the checksum does not match the file did not make it.

36) Identify the functions of the TCP/IP network-layer protocols.

IP provides connectionless, best-effort delivery routing of datagrams.  It is not concerned with the content of the datagram, instead it looks for a way to move the datagrams to their destination.

ICMP is in the network layer.

OSPF is contained in the network layer.

37) Identify the functions performed by ICMP.

To detect routing failures.

Excellent network design and troubleshooting tool

Used in PING

Used in TraceRoute

38) Configure IPX access lists and SAP filters to control basic Novell traffic.

Routing

39) Add the RIP routing protocol to your configuration.

40) Add the IGRP routing protocol to your configuration.

41) Explain the services of separate and integrated multiprotocol routing.

42) List problems that each routing type encounters when dealing with topology changes and describe techniques to reduce the number of these problems.

43) Describe the benefits of network segmentation with routers.

Network Security

44) Configure standard and extended access lists to filter IP traffic.

45) Monitor and verify selected access list operations on the router.

LAN Switching

46) Describe the advantages of LAN segmentation.

47) Describe LAN segmentation using bridges.

48) Describe LAN segmentation using routers.

49) Describe LAN segmentation using switches.

50) Name and describe two switching methods.

51) Describe full- and half-duplex Ethernet operation.

52) Describe network congestion problem in Ethernet networks.

53) Describe the benefits of network segmentation with bridges.

54) Describe the benefits of network segmentation with switches.

55) Describe the features and benefits of Fast Ethernet.

56) Describe the guidelines and distance limitations of Fast Ethernet.

57) Distinguish between cut-through and store-and-forward LAN switching.

58) Describe the operation of the Spanning Tree Protocol and its benefits.

59) Describe the benefits of virtual LANs.

60) Define and describe the function of a MAC address.

48-bit expressed as 12 hex digits

First six hex digits are the vendor code

Last six hex digits are administered by the vendor

Used to get a packet from one local device to another local device.

Three methods of obtaining the MAC

Address Resolution Protocol (ARP)

Node is notified of another station’s address with a hello packet

Addresses are assigned in a predictable way, as with DECnet.

TCP Segment Format

16163232466Source PortDest. PortSequence #Ack #HLENReservedCode Bits

1616160 or 32DataWindowChecksumUrgentOption

UDP Segment Format

16161616DataSource PortDest. PortLengthChecksum

IP Datagram

44816163138VERSHLENType of ServTotal LengthIDFlagsFrag OffsetTTL

8163232VariableVariableVariableProtocolHdr chksumSource IPDest IPIP OptionsPaddingData