Configuring Standard and Extended IP Access Lists

Sample Network:  You may wish to configure the network below.  You will first need to configure the network for dynamic or static routing.  (Notice that the IP addresses may have changed from the previous example.)

Example:  Permitting only a specific host from the 205.7.5.0 network onto the 192.5.5.0 network

LabA (config)#  access-list  10  permit  host  205.7.5.11

LabA (config)#  inter  e  0

LabA (config-if)#  ip access-group 10 out

Example:  Denying a specific host from the 205.7.5.0 network onto the 192.5.5.0 network

LabA (config)#  access-list  10  deny  205.7.5.11

LabA (config)#  access-list  10  permit  0.0.0.0  255.255.255.255

{Or  LabA(config)#  access-list  10  permit any}

LabA (config)#  inter  e  0

LabA (config-if)#  ip  access-group  10  out

Example:  Permitting only hosts from the 210.93.105.0 network onto the 192.5.5.0 network

LabA (config)#  access-list  10  permit  210.93.105.0

{Or  LabA (config)#  access-list 10 permit 210.93.105.0  0.0.0.255}

LabA (config)#  inter e 0

LabA (config-if)#  ip access-group 10 out

Example:  An access list that will permit telnets destined for the host 223.8.151.10 from the 195.5.5.0, but will prohibit all other telnets from the 195.5.5.0 network into the 223.8.151.0 network.  All other traffic is allowed to enter the 223.8.151.0 network.

LabA (config)#  access-list  101  permit  tcp  192.5.5.0  0.0.0.255  223.8.151.10  0.0.0.0  eq 23

LabA (config)#  access-list  101  deny  tcp  192.5.5.0  0.0.0.255  223.8.151.0  0.0.0.255  eq  23

LabA (config)#  access-list  101  permit  ip  0.0.0.0  255.255.255.255  0.0.0.0  255.255.255.255

LabA (config)#  interface e 0

LabA (config-if)#  ip access-group 101 in

Using the host keyword same as the wildcard mask 0.0.0.0:

LabA (config)#  access-list  101  permit  tcp  192.5.5.0  0.0.0.255  223.8.151.10  0.0.0.0  eq 23

replaced by

LabA (config)#  access-list  101  permit  tcp  192.5.5.0  0.0.0.255  host  223.8.151.10  eq 23

Using the any keyword is the same as the using 0.0.0.0 255.255.255.255

LabA (config)#  access-list  101  permit  ip  0.0.0.0  255.255.255.255  0.0.0.0  255.255.255.255

replaced by

LabA (config)#  access-list  101  permit  ip  any  any

Note:  Remember there is an implicit deny any (everything) at the end of an access list.