exercise

31

Conduction System 
of the Heart and Electrocardiography

Objectives

1.	To list and localize the elements of the intrinsic conduction, or nodal, system of the heart, and to describe how impulses are initiated and conducted through this system and the myocardium.

2.	To interpret the ECG in terms of depolarization and repolarization events occurring in the myocardium; and to identify the P, QRS, and T waves on an ECG recording using an ECG recorder or BIOPAC®.

3.	To calculate the heart rate, QRS interval, P–Q interval, and Q–T interval from an ECG obtained during the laboratory period.

4.	To define tachycardia, bradycardia, and fibrillation.

Materials

q	Apparatus A or B:*
A: ECG recording apparatus, electrode paste, 
alcohol swabs, cot or lab table, rubber straps

B: BIOPAC® Apparatus: BIOPAC® MP30 (or MP35) data acquisition unit, PC or Macintosh (Mac) computer with at least 4MB of RAM available above and beyond the operating system needs and any other programs that are running, wall transformer, serial cable, BIOPAC® Student Lab Software v3.0 or greater, electrode lead set, disposable vinyl electrodes, cot or lab table, and pillow.

q	Millimeter ruler

*The Instructor Guide provides instructions for use of PowerLab® equipment.

The Intrinsic Conduction System

Heart contraction results from a series of electrical potential changes (depolarization waves) that travel through the heart preliminary to each beat. Because cardiac muscle cells are electrically connected by gap junctions, the entire myo-cardium behaves like a single unit, a functional syncytium (sin-sih9shum).

The ability of cardiac muscle to beat is intrinsic—it does not depend on impulses from the nervous system to initiate its contraction and will continue to contract rhythmically even if all nerve connections are severed. However, two types of controlling systems exert their effects on heart activity. One of these involves nerves of the autonomic nervous system, which accelerate or decelerate the heartbeat rate depending on which division is activated. The second system is the intrinsic conduction system, or nodal system, of the heart, consisting of specialized noncontractile myocardial tissue. The intrinsic conduction system ensures that heart muscle depolarizes in an orderly and sequential manner (from atria to ventricles) and that the heart beats as a coordinated unit.

The components of the intrinsic conduction system include the sinoatrial (SA) node, located in the right atrium just inferior to the entrance to the superior vena cava; the atrioventricular (AV) node in the lower atrial septum at the junction of the atria and ventricles; the AV bundle (bundle of His) and right and left bundle branches, located in the interventricular septum; and the Purkinje fibers, essentially long strands of barrel-shaped cells called Purkinje myocytes, which ramify within the muscle bundles of the ventricular walls. The Purkinje fiber network is much denser and more elaborate in the left ventricle because of the larger size of this chamber (Figure 31.1).

The SA node, which has the highest rate of discharge, provides the stimulus for contraction. Because it sets the rate of depolarization for the heart as a whole, the SA node is often referred to as the pacemaker. From the SA node, the impulse spreads throughout the atria and to the AV node. This electrical wave is immediately followed by atrial contraction. At the AV node, the impulse is momentarily delayed (approximately 0.1 sec), allowing the atria to complete their contraction. It then passes through the AV bundle, the right and left bundle branches, and the Purkinje fibers, finally resulting in ventricular contraction. Note that the atria and ventricles are separated from one another by a region of electrically inert connective tissue, so the depolarization wave can be transmitted to the ventricles only via the tract between the AV node and AV bundle. Thus, any damage to the AV node-bundle pathway partially or totally insulates the ventricles from the influence of the SA node. Although autorhythmic cells are found throughout the heart, their rates of spontaneous depolarization differ. The nodal system increases the rate of heart depolarization and synchronizes heart activity.

Electrocardiography

The conduction of impulses through the heart generates electrical currents that eventually spread throughout the body. These impulses can be detected on the body’s surface and recorded with an instrument called an electrocardiograph. The graphic recording of the electrical changes (depolarization followed by repolarization) occurring during the cardiac cycle is called an electrocardiogram (ECG) (Figure 31.2). The typical ECG consists of a series of three recognizable waves called deflection waves. The first wave, the P wave, is a small wave that indicates depolarization of the atria immediately before atrial contraction. The large QRS complex, resulting from ventricular depolarization, has a complicated shape (primarily because of the variability in size of the two ventricles and the time differences required for these chambers to depolarize). It precedes ventricular contraction. The T wave results from currents propagated during ventricular repolarization. The repolarization of the atria, which occurs during the QRS interval, is generally obscured by the large QRS complex. The relationship between the deflection waves of an ECG and sequential excitation of the heart is shown in Figure 31.3.

It is important to understand what an ECG does and does not show: First, an ECG is a record of voltage and time—nothing else. Although we can and do infer that muscle contraction follows its excitation, sometimes it does not. Secondly, an ECG records electrical events occurring in relatively large amounts of muscle tissue (i.e., the bulk of the heart muscle), not the activity of nodal tissue which, like muscle contraction, can only be inferred. Nonetheless, abnormalities of the deflection waves and changes in the time intervals of the ECG are useful in detecting myocardial infarcts or problems with the conduction system of the heart. The 
P–Q interval represents the time between the beginning of atrial depolarization and ventricular depolarization. Thus, it typically includes the period during which the depolarization wave passes to the AV node, atrial systole, and the passage of the excitation wave to the balance of the conducting system. Generally, the P–Q interval is about 0.16 to 0.18 sec. A longer interval may suggest a partial AV heart block caused by damage to the AV node. In total heart block, no impulses are transmitted through the AV node, and the atria and ventricles beat independently of one another—the atria at the SA node rate and the ventricles at their intrinsic rate, which is considerably slower.

If the QRS interval (normally 0.08 sec) is prolonged, it may indicate a right or left bundle branch block in which one ventricle is contracting later than the other. The Q–T interval is the period from the beginning of ventricular depolarization through repolarization and includes the time of ventricular contraction (the S–T segment). With a heart rate of 70 beats/min, this interval is normally 0.31 to 0.41 sec. As the rate increases, this interval becomes shorter; conversely, when the heart rate drops, the interval is longer.

A heart rate over 100 beats/min is referred to as tachycardia; a rate below 60 beats/min is bradycardia. Although neither condition is pathological, prolonged tachycardia may progress to fibrillation, a condition of rapid uncoordinated heart contractions which makes the heart useless as a pump. Bradycardia in athletes is a positive finding; that is, it indicates an increased efficiency of cardiac functioning. Because stroke volume (the amount of blood ejected by a ventricle with each contraction) increases with physical conditioning, the heart can contract more slowly and still meet circulatory demands.

Twelve standard leads are used to record an ECG for diagnostic purposes. Three of these are bipolar leads that measure the voltage difference between the arms, or an arm and a leg, and nine are unipolar leads. Together the 12 leads provide a fairly comprehensive picture of the electrical activity of the heart.

For this investigation, four electrodes are used (Figure 31.4), and results are obtained from the three standard limb leads (also shown in Figure 31.4). Several types of phy-siographs or ECG recorders are available. Your instructor will provide specific directions on how to set up and use the available apparatus if standard ECG apparatus is used (Activity 1a). Instructions for use of BIOPAC® apparatus (Activity 1b) are provided on pages 336–342.

Understanding the Standard 
Limb Leads

As you might expect, electrical activity recorded by any lead depends on the location and orientation of the recording electrodes. Clinically, it is assumed that the heart lies in the center of a triangle with sides of equal lengths (Einthoven’s triangle) and that the recording connections are made at the vertices (corners) of that triangle. But in practice, the electrodes connected to each arm and to the left leg are considered to connect to the triangle vertices. The standard limb leads record the voltages generated in the extracellular fluids surrounding the heart by the ion flows occurring simultaneously in many cells between any two of the connections. A recording using lead I (RA–LA), which connects the right arm (RA) and the left arm (LA), is most sensitive to electrical activity spreading horizontally across the heart. Lead II (RA–LL) and lead III (LA–LL) record activity along the vertical axis (from the base of the heart to its apex) but from different orientations. The significance of Einthoven’s triangle is that the sum of the voltages of leads I and III equals that in lead II (Einthoven’s law). Hence, if the voltages of two of the standard leads are recorded, that of the third lead can be determined mathematically.

Once the subject is prepared, the ECG will be recorded first under baseline (resting) conditions and then under conditions of fairly strenuous activity. Finally, recordings will be made while the subject holds his or her breath or carries out deep breathing. The activity recordings and those involving changes in respiratory rate or depth will be compared to the baseline recordings, and you will be asked to determine the reasons for the observed differences in the recordings.

Activity 1a:

Recording ECGs Using 
a Standard ECG Apparatus

Preparing the Subject

1.	Place electrode paste on four electrode plates and position each electrode as follows after scrubbing the skin at the attachment site with an alcohol swab. Attach an electrode to the anterior surface of each forearm, about 5 to 8 cm (2 to 3 in.) above the wrist, and secure them with rubber straps. In the same manner, attach an electrode to each leg, approximately 5 to 8 cm above the medial malleolus (inner aspect of the ankle).

2.	Attach the appropriate tips of the patient cable to the electrodes. The cable leads are marked RA (right arm), LA (left arm), LL (left leg), and RL (right leg, the ground).

Making a Baseline Recording

1.	Position the subject comfortably in a supine position on a cot (if available), or sitting relaxed on a laboratory chair.

2.	Turn on the power switch and adjust the sensitivity knob to 1. Set the paper speed to 25 mm/sec and the lead selector to the position corresponding to recording from lead I 
(RA–LA).

3.	Set the control knob at the RUN position and record the subject’s at-rest ECG from lead I for 2 to 3 minutes or until the recording stabilizes. (You will need a tracing long enough to provide each student in your group with a representative segment.) The subject should try to relax and not move unnecessarily, because the skeletal muscle action potentials will also be picked up and recorded.

4.	Stop the recording and mark it “lead I.”

5.	Repeat the recording procedure for leads II (RA–LL) and III (LA–LL).

6.	Each student should take a representative segment of one of the lead recordings and label the record with the name of the subject and the lead used. Identify and label the P, QRS, and T waves. The calculations you perform for your recording should be based on the following information: Because the paper speed was 25 mm/sec, each millimeter of paper corresponds to a time interval of 0.04 sec. Thus, if an interval requires 4 mm of paper, its duration is 4 mm 3 0.04 sec/mm 5 0.16 sec.

7.	Compute the heart rate. Obtain a mm ruler and measure the distance from the beginning of one QRS complex to the beginning of the next QRS complex, and enter this value into the following equation to find the time for one heartbeat.

mm/beat 3 0.04 sec/mm 5  sec/beat

Now find the beats per minute, or heart rate, by using the figure just computed for seconds per beat in the following equation:

Beats/min 5 	1	 3 60 sec/min

( sec/beat)

Beats/min 5

Is the obtained value within normal limits?

Measure the QRS interval, and compute its duration.

Measure the Q–T interval, and compute its duration.

Measure the P–Q interval, and compute its duration.

Are the computed values within normal limits?

8.	At the bottom of this page, attach segments of the ECG recordings from leads I through III. Make sure you indicate the paper speed, lead, and subject’s name on each tracing. To the recording on which you based your previous computations, add your calculations for the duration of the QRS, P–Q intervals, and Q–T intervals above the respective area of tracing. Also record the heart rate on that tracing.

Recording the ECG while Running in Place

1.	Make sure the electrodes are securely attached to prevent electrode movement while recording the ECG.

2.	Set the paper speed to 25 mm/sec, and prepare to make the recording using lead I.

3.	Record the ECG while the subject is running in place for 3 minutes. Then have the subject sit down, but continue to record the ECG for an additional 4 minutes. Mark the recording at the end of the 3 minutes of running and at 1 minute after cessation of activity.

4.	Stop the recording. Compute the beats/min during the third minute of running, at 1 minute after exercise, and at 4 minutes after exercise. Record below:

beats/min while running in place

beats/min at 1 minute after exercise

beats/min at 4 minutes after exercise

5.	Compare this recording with the previous recording from lead I. Which intervals are shorter in the “running” recording?

Does the subject’s heart rate return to resting level by 4 minutes after exercise?

Recording the ECG during Breath Holding

1.	Position the subject comfortably in the sitting position.

2.	Using lead I and a paper speed of 25 mm/sec, begin the recording. After approximately 10 seconds, instruct the subject to begin breath holding and mark the record to indicate the onset of the 1-min breath-holding interval.

3.	Stop the recording after 1 minute and remind the subject to breathe. Compute the beats/minute during the 1-minute experimental (breath-holding) period.

Beats/min during breath holding:

4.	Compare this recording with the lead I recording obtained under resting conditions.

What differences are seen?

Attempt to explain the physiological reason for the differences you have seen. (Hint: A good place to start might be to check hypoventilation or the role of the respiratory system in acid-base balance of the blood.)

Activity 1b:

Electrocardiography Using BIOPAC®

In this activity, you will record the electrical activity of the heart under three different conditions: (1) while the subject is lying down, (2) after the subject sits up and breathes normally, and (3) after the subject has exercised and is breathing deeply.

Since the electrodes are not placed directly over the heart, artifacts can result from the recording of unwanted skeletal muscle activity. In order to obtain a clear ECG, it is important that the subject:

•	Remain still during the recording.

•	Refrain from laughing or talking during the recording.

•	When in the sitting position, keep arms and legs steady and relaxed.

Setting up the Equipment

1.	Connect the BIOPAC® unit to the computer and turn the computer ON.

2.	Make sure the BIOPAC® MP30 (or MP35) unit is OFF.

3.	Plug in the equipment as shown in Figure 31.5:

•	Electrode lead set — CH 2

4.	Turn the BIOPAC® MP30 (or MP35) unit ON.

5.	Place the three electrodes on the subject as shown in Figure 31.6, and attach the electrode leads according to the colors indicated. The electrodes should be placed on the medial surface of each leg, 5 to 8 cm (2 to 3 in.) superior to the ankle. The other electrode should be placed on the right anterior forearm 5 to 8 cm above the wrist.

6.	The subject should lie down and relax in a comfortable position. A chair or place to sit up should be available nearby.

7.	Start the BIOPAC® Student Lab program on the computer by double-clicking the icon on the desktop or by following your instructor’s guidance.

8.	Select lesson L05-ECG-1 from the menu, and click OK.

9.	Type in a filename that will save this subject’s data on the computer hard drive. You may want to use subject last name followed by ECG-1 (for example, SmithECG-1), then click OK.

Calibrating the Equipment

•	Examine the electrodes and the electrode leads to be certain they are properly attached.

1.	The subject must remain still and relaxed. With the subject in a still position, click Calibrate. This will initiate the process whereby the computer will automatically establish parameters to record the data.

2.	The calibration procedure will stop automatically after eight seconds.

3.	Observe the recording of the calibration data, which should look similar to the data in Figure 31.7.

•	If the data look very different, click Redo Calibration and repeat the steps above.

•	If the data look similar, proceed to the next section.

Recording Segment 1: Subject Lying Down

1.	To prepare for the recording, remind the subject to remain still and relaxed while lying down.

2.	When prepared, click Record and gather data for 20 seconds. At the end of 20 seconds, click Suspend.

3.	Observe the data, which should look similar to the data in Figure 31.8.

•	If the data look very different, click Redo and repeat the steps above. Be certain to check attachment of the electrodes and leads, and remind the subject not to move, talk, or laugh.

•	If the data look similar, move on to the next recording segment.

Recording Segment 2: After Subject Sits Up, with Normal Breathing

1.	Inform the subject to be ready to sit up in the designated location. With the exception of breathing, the subject should try to remain motionless after assuming the seated position. If the subject moves too much during recording after sitting up, unwanted skeletal muscle artifacts will affect the recording.

2.	When prepared, instruct the subject to sit up. Immediately after the subject assumes a motionless state, click Resume, and the data will begin recording.

3.	At the end of 20 seconds, click Suspend to stop recording.

4.	Observe the data, which should look similar to the data in Figure 31.9.

•	If the data look very different, click Redo and repeat the steps above. Be certain to check attachment of the electrodes, and do not click Resume until the subject is motionless.

•	If the data look similar, move on to the next recording segment. Note: For start of next segment, ignore the program instructions at the bottom of the computer screen.

Recording Segment 3: After Subject Exercises, with Deep Breathing

1.	Remove the electrode pinch connectors from the electrodes on the subject.

2.	Have the subject do a brief round of exercise, such as jumping jacks or running in place for 1 minute, in order to elevate the heart rate.

3.	As quickly as possible after the exercise, have the subject resume a motionless, seated position and reattach the pinch connectors. Once again, if the subject moves too much during recording, unwanted skeletal muscle artifacts will affect the data. After exercise, the subject is likely to be breathing deeply, but otherwise should remain as still as possible.

4.	Immediately after the subject assumes a motionless, seated state, click Resume, and the data will begin recording. Record the ECG for 60 seconds in order to observe post-exercise recovery.

5.	After 60 seconds, click Suspend to stop recording.

6.	Observe the data, which should look similar to the data in Figure 31.10.

•	If the data look very different, click Redo and repeat the steps above. Be certain to check attachment of the electrodes and leads, and remember not to click Resume until the subject is motionless.

7.	When finished, click Done. Remove the electrodes from the subject.

8.	A pop-up window will appear. To record from another subject, select Record data from another subject and return to step 5 under Setting up the Equipment. If continuing to the Data Analysis section, select Analyze current data file and proceed to step 2 of the Data Analysis section.

Data Analysis

1.	If just starting the BIOPAC® program to perform data analysis, enter Review Saved Data mode and choose the file with the subject’s ECG data (for example, SmithECG-1).

2.	Use the following tools to adjust the data in order to clearly view and analyze four consecutive cardiac cycles:

•Click on the magnifying glass in the lower right corner of the screen (near the I-beam cursor box) to activate the zoom function. Use the magnifying glass cursor to click on the very first waveforms until there are about four seconds of data represented (see horizontal time scale at the bottom of the screen).

•	Select the Display menu at the top of the screen, and click on Autoscale Waveforms. This function will adjust the data for better viewing.

•	Click the Adjust Baseline button. Two new buttons will appear; simply click on these buttons to move the waveforms Up or Down so they appear clearly in the center of the screen. Once they are centered, click Exit.

3.	Set up the first two pairs of channel/measurement boxes at the top of the screen by selecting the following channels and measurement types from the drop-down menus (note that Channel 2 contains the ECG data):

Channel	Measurement

CH 2	deltaT

CH 2	bpm

Analysis of Segment 1: Subject Lying Down

1.	Use the arrow cursor and click the I-beam cursor box on the lower right side of the screen to activate the “area selection” function.

2.	First measure deltaT and bpm in Segment 1 (approximately seconds 0–20). Using the I-beam cursor, highlight from the peak of one R-wave to the peak of the next R-wave, as shown in Figure 31.11.

3.	Observe that the computer automatically calculates the deltaT and bpm for the selected area. These measurements represent the following:

deltaT (difference in time): computes the elapsed time between the beginning and end of the highlighted area.

bpm (beats per minute): computes the beats per minute when highlighting from the R-wave of one cycle to the R-wave of another cycle.

4.	Record this data in the chart below under R to R Sample 1 (round to the nearest 1⁄100 second and 1⁄10 beat per minute).

5.	Using the I-beam cursor, highlight two other pairs of R to R areas in this segment and record the data in the Segment 1 chart on page 339.

6.	Calculate the means and ranges of the data in this table.

7.	Next, use the zoom, Autoscale Waveforms, and Adjust Baseline tools described above to focus in on one ECG waveform within Segment 1. See the example in Figure 31.12.

8.	Once a single ECG waveform is centered for analysis, click on the I-beam cursor box on the lower right side of the screen to activate the “area selection” function.

9.	Using the highlighting function and deltaT computation, measure the duration of every component of the ECG waveform. Refer to Figure 31.2b and Table 31.1 for guidance in highlighting each component.

10.	Highlight each component of one cycle. Observe the elapsed time, and record this data under Cycle 1 in the chart to the right.

11.	Scroll along the horizontal axis at the bottom of the data to view and analyze two additional cycles in Segment 1. Record the elapsed time for every component of Cycle 2 and Cycle 3 in the chart to the right.

12.	Calculate the means for the three cycles of data and record in the data chart to the right.

Analysis of Segment 2: Subject Sitting Up and Breathing Normally

1.	Scroll down on the horizontal time bar until you reach the data for Segment 2 (the 20–40 second period). A marker with “after sitting up” should denote the beginning of this data.

2.	As was done in the Analysis of Segment 1 section, use the I-beam tool to highlight and measure the deltaT and bpm between three different pairs of R-waves in this segment, and record the data in the Segment 2 chart below.

Analysis of Segment 3: After Exercise with Deep Breathing

1.	Scroll down on the horizontal time bar until you reach the data for Segment 3 (the 40–60 second period). A marker with “deep breathing” should denote the beginning of this data.

2.	As before, use the I-beam tool to highlight and measure the deltaT and bpm between three pairs of R-waves in this segment, and record the data in the Segment 3 chart above.

3.	Using the instructions provided in numbers 8 and 9 under the Analysis of Segment 1 section, highlight and observe the elapsed time for each component of one cycle, and record this data under Cycle 1 in the Elapsed time chart below.

4.	Scroll along the horizontal axis at the bottom of the data to view and analyze two other cycles in Segment 3. Record the elapsed time for each component of Cycle 2 and Cycle 3 in the Elapsed time chart below.

5.	Calculate the means for each component in Segment 3.

6.	When finished, Exit the program.

Compare the average deltaT times and average bpm between the data in Segment 1 (lying down) and the data in Segment 3 (after exercise). Which is greater in each case?

What is the relationship between elapsed time (deltaT) between R-waves and the heart rate?

Is there a change in heart rate when the subject makes the transition from lying down (Segment 1) to a sitting position (Segment 2)?

Examine the average duration of each of the ECG components in Segment 1 and Segment 3. In the chart just below, record the average values of each component. Draw a circle around those measures that fit within the normal range.

Compare the QT intervals in the data while the subject is at rest versus after exercise; this interval corresponds closely with the duration of contraction of the ventricles. Describe and explain any difference.

Compare the duration in the period of time from the end of each T wave to the next R-spike while the subject is at rest versus after exercise. This interval corresponds closely with the duration of contraction of the ventricles. Describe and explain any difference.

Which, if any, of the components in the Average Duration chart to the left demonstrates a significant difference from the normal value? Based on the events of the cardiac cycle and their representation in an ECG, what abnormality might the data suggest?

n

#	Conduction System of the Heart and Electrocardiography#

Measure	Channel	R to R 	R to R 	R to R 	Mean	Range
		Sample 	Sample 	Sample
		1	2	3

deltaT	CH 2

bpm	CH 2				#	Exercise 31

Table 31.1	Boundaries of each 
ECG ComponentFeature	Boundaries

P wave	Start of P deflection to return to 
	isoelectric line

P–R interval	Start of P deflection to start of Q 
	deflection

P–R segment	End of P wave to start of Q deflection

QRS 	Start of Q deflection to S return to 
complex	isoelectric line

S–T segment	End of S deflection to start of T wave

Q–T interval	Start of Q deflection to end of T wave

T wave	Start of T deflection to return to 
	isoelectric line

End T to 	End of T wave to next R spike
next R

Figure 31.1  The intrinsic conduction system of the heart. Dashed-line arrows indicate transmission of the impulse from the SA node through the atria. Solid arrows indicate transmission of the impulse from the SA node to the AV node via the internodal pathway.

Figure 31.2  The normal electrocardiogram. 
(a) Regular sinus rhythm. (b) Waves, segments, and intervals of a normal ECG.#	Exercise 31

Figure 31.3  The sequence of excitation of the heart related to the deflection waves of an ECG tracing.	Conduction System of the Heart and Electrocardiography#

Figure 31.4  ECG recording positions for the standard limb leads.				#	Exercise 31							Conduction System of the Heart and Electrocardiography#

Figure 31.5  Setting up the BIOPAC® unit. Plug the electrode lead set into Channel 2.

Figure 31.6  Placement of electrodes and the appropriate attachment of electrode leads by color.

Figure 31.7  Example of calibration data.#	Exercise 31

igure 31.8  Example of ECG data while the subject is lying down.

Figure 31.9  Example of ECG data after the subject sits up and breathes normally.

Figure 31.10  Example of ECG data after the subject exercises.	Conduction System of the Heart and Electrocardiography#

Figure 31.11  Example of highlighting from 
R-wave to R-wave.deltaT and bpm Samples for Segment 1

Figure 31.12  Example of a single ECG waveform with the first part of the P wave highlighted.Elapsed time for ECG components in Segment 1 (seconds)

Component	Cycle 1	Cycle 2	Cycle 3	Mean

P wave

P–R interval

P–R segment

QRS
complex

S–T segment

Q–T interval

T wave

End T to 
next R				deltaT and bpm Samples for Segment 2

Measure	Channel	R to R 	R to R 	R to R 	Mean	Range
		Sample 	Sample 	Sample
		1	2	3

deltaT	CH 2

bpm	CH 2					Conduction System of the Heart and Electrocardiography#deltaT and bpm Samples for Segment 3

Measure	Channel	R to R 	R to R 	R to R 	Mean	Range
		Sample 	Sample 	Sample
		1	2	3

deltaT	CH 2

bpm	CH 2				Elapsed time for ECG Components in Segment 3 (in seconds)

Component	Cycle 1	Cycle 2	Cycle 3	Mean

P wave

P–R interval

P–R
segment

QRS
complex

S–T 
segment

Q–T interval

T wave				#	Exercise 31Average Duration for ECG Components

ECG 	Normal 	Segment	Segment 
Component	Duration 	1	3
	(seconds)	(lying 	(post 
		down)	exercise)

P wave	0.06–0.11

P–R interval	0.12–0.20

P–R segment	0.08

QRS complex	Less than
	0.12

S–T segment	0.12

Q–T interval	0.31–0.41

T wave	0.16

End of T to 	varies	
next R