Respiratory System Physiology

Objectives

1.	To define the following (and be prepared to provide volume figures if applicable):

inspiration	expiratory reserve volume

expiration	inspiratory reserve volume

tidal volume	minute respiratory volume

vital capacity

2.	To explain the role of muscles and volume changes in the mechanical process of breathing.

3.	To demonstrate proper usage of a spirometer.

4.	To explain the relative importance of various mechanical and chemical factors in producing respiratory variations.

5.	To describe bronchial and vesicular breathing sounds.

6.	To explain the importance of the carbonic acid–bicarbonate buffer system in maintaining blood pH.

Materials

q	Model lung (bell jar demonstrator)

q	Tape measure

q	Stethoscope

q	Alcohol swabs

q	Spirometer

q	Disposable cardboard mouthpieces

q	Nose clips

q	Table (on chalkboard) for recording class data

q	Disposable autoclave bag

q	Battery jar containing 70% ethanol solution

q	Apparatus A or B:

A: Physiograph, pneumograph, and recording attachments for physiograph

B: BIOPAC® Apparatus: BIOPAC® MP30 (or MP35) data acquisition unit, PC or Macintosh (Mac) computer with at least 4MB of RAM available above and beyond the operating system needs and any other programs that are running, BIOPAC® Student Lab Software v3.0 or greater, wall transformer, serial cable, BIOPAC® airflow transducer, BIOPAC® calibration syringe, disposable mouthpiece, nose clip, and bacteriological filter.

q	Paper bag

q	0.05 M NaOH

q	Phenol red in a dropper bottle

q	100-ml beakers

q	Straws

q	Concentrated HCl and NaOH in dropper bottles

q	250- and 50-ml beakers

q	Plastic wash bottles containing distilled water

q	Graduated cylinder (100 ml)

q	Glass stirring rod

q	Animal plasma

q	pH meter (standardized with buffer of pH 7)

q	Buffer solution (pH 7)

q	0.01 M HCl

q	Human Respiratory System videotape*

PhysioEx™ 5.0 Computer Simulation on page P-1

*Available to qualified adopters from Benjamin Cummings.

†The Instructor’s Guide provides instructions for use of PowerLab® equipment.

Mechanics of Respiration

Pulmonary ventilation, or breathing, consists of two phases: inspiration, during which air is taken into the lungs, and expiration, during which air passes out of the lungs. As the inspiratory muscles (external intercostals and diaphragm) contract during inspiration, the size of the thoracic cavity increases. The diaphragm moves from its relaxed dome shape to a flattened position, increasing the superoinferior volume. The external intercostals lift the rib cage, increasing the anteroposterior and lateral dimensions (Figure 37A.1). Because the lungs adhere to the thoracic walls like flypaper because of the presence of serous fluid in the pleural cavity, the intrapulmonary volume (volume within the lungs) also increases, lowering the air (gas) pressure inside the lungs. The gases then expand to fill the available space, creating a partial vacuum that causes air to flow into the lungs—constituting the act of inspiration. During expiration, the inspiratory muscles relax, and the natural tendency of the elastic lung tissue to recoil acts to decrease the intrathoracic and intrapulmonary volumes. As the gas molecules within the lungs are forced closer together, the intrapulmonary pressure rises to a point higher than atmospheric pressure. This causes gases to flow from the lungs to equalize the pressure inside and outside the lungs—the act of expiration.

Activity 1:

Operating the Model Lung

Observe the model lung, which demonstrates the principles involved in gas flows into and out of the lungs. It is a simple apparatus with a bottle “thorax,” a rubber membrane “diaphragm,” and balloon “lungs.”

1.	Go to the demonstration area and work the model lung by moving the rubber diaphragm up and down. The balloons will not fully inflate or deflate, but notice the relative changes in balloon (lung) size as the volume of the thoracic cavity is alternately increased and decreased.

2.	Check the appropriate columns in the chart concerning these observations in the Exercise 37A Review Sheet at the back of this book.

3.	Simulate a pneumothorax. Inflate the balloon lungs by pulling down on the diaphragm. Ask your lab partner to let air into the bottle “thorax” by loosening the rubber stopper.

What happens to the balloon lungs?

4.	After observing the operation of the model lung, conduct the following tests on your lab partner. Use the tape measure to determine his or her chest circumference by placing the tape around the chest as high up under the armpits as possible. Record the measurements in inches in the appropriate space for each of the conditions below.

Quiet breathing:

Inspiration  Expiration

Forced breathing:

Inspiration  Expiration

Do the results coincide with what you expected on the basis

of what you have learned thus far?

How does the structural relationship between the balloon-lungs and bottle-thorax differ from that seen in the human lungs and thorax?

n

Respiratory Sounds

As air flows in and out of the respiratory tree, it produces two characteristic sounds that can be picked up with a stethoscope (auscultated). The bronchial sounds are produced by air rushing through the large respiratory passageways (the trachea and the bronchi). The second sound type, vesicular breathing sounds, apparently results from air filling the alveolar sacs and resembles the sound of a rustling or muffled breeze.

Activity 2:

Auscultating Respiratory Sounds

1.	Obtain a stethoscope and clean the earpieces with an alcohol swab. Allow the alcohol to dry before donning the stethoscope.

2.	Place the diaphragm of the stethoscope on the throat of the test subject just below the larynx. Listen for bronchial sounds on inspiration and expiration. Move the stethoscope down toward the bronchi until you can no longer hear sounds.

3.	Place the stethoscope over the following chest areas and listen for vesicular sounds during respiration (heard primarily during inspiration).

•	At various intercostal spaces.

•	At the triangle of auscultation (a small depressed area of the back where the muscles fail to cover the rib cage; located just medial to the inferior part of the scapula).

•	Under the clavicle. n

Diseased respiratory tissue, mucus, or pus can produce abnormal chest sounds such as rales (a rasping sound) and wheezing (a whistling sound). l

Respiratory Volumes and Capacities_Spirometry

A person’s size, sex, age, and physical condition produce variations in respiratory volumes. Normal quiet breathing moves about 500 ml of air in and out of the lungs with each breath. As you have seen in the first activity, a person can usually forcibly inhale or exhale much more air than is exchanged in normal quiet breathing. The terms given to the measurable respiratory volumes are defined next. These terms and their normal values for an adult male should be memorized.

Tidal volume (TV): Amount of air inhaled or exhaled with each breath under resting conditions (500 ml).

Inspiratory reserve volume (IRV): Amount of air that can be forcefully inhaled after a normal tidal volume inhalation (3100 ml).

Expiratory reserve volume (ERV): Amount of air that can be forcefully exhaled after a normal tidal volume exhalation (1200 ml).

Vital capacity (VC): Maximum amount of air that can be exhaled after a maximal inspiration (4800 ml).

VC 5 TV 1 IRV 1 ERV

An idealized tracing of the various respiratory volumes and their relationships to each other is shown in Figure 37A.2.

Respiratory volumes will be measured with an apparatus called a spirometer. There are two major types of spirometers, which give comparable results—the handheld dry, or wheel, spirometers (such as the Wright spirometer illustrated in Figure 37A.3) and “wet” spirometers, such as the Phipps and Bird spirometer and the Collins spirometer (which is available in both recording and nonrecording varieties). The somewhat more sophisticated wet spirometer consists of a plastic or metal bell within a rectangular or cylindrical tank that air can be added to or removed from (Figure 37A.4). The outer tank contains water and has a tube running through it to carry air above the water level. The floating bottomless bell is inverted over the water-containing tank and connected to a volume indicator.

In nonrecording spirometers, an indicator moves as air is exhaled, and only expired air volumes can be measured directly. By contrast, recording spirometers allow both inspired and expired gas volumes to be measured.

Activity 3:

Measuring Respiratory Volumes

The steps for using a nonrecording spirometer and a wet recording spirometer are given separately below.

Using a Nonrecording Spirometer

1.	Before using the spirometer, count and record the subject’s normal respiratory rate. The subject should face away from you as you make the count.

Respirations per minute:

Now identify the parts of the spirometer you will be using by comparing it to the illustration in Figure 37A.3 or 37A.4a. Examine the spirometer volume indicator before beginning to make sure you know how to read the scale. Work in pairs, with one person acting as the subject while the other records the data of the volume determinations. Reset the indicator to zero before beginning each trial.

Obtain a disposable cardboard mouthpiece. Insert it in the open end of the valve assembly (attached to the flexible tube) of the wet spirometer or over the fixed stem of the handheld dry spirometer. Before beginning, the subject should practice exhaling through the mouthpiece without exhaling through the nose, or prepare to use the nose clips (clean them first with an alcohol swab). If you are using the handheld spirometer, make sure its dial faces upward so that the volumes can be easily read during the tests.

2.	The subject should stand erect during testing. Conduct the test three times for each required measurement. Record the data where indicated on this page, and then find the average volume figure for that respiratory measurement. After you have completed the trials and computed the averages, enter the average values on the table prepared on the chalkboard for tabulation of class data,* and copy all averaged data onto the Exercise 37A Review Sheet.

3.	Measuring tidal volume (TV). The TV, or volume of air inhaled and exhaled with each normal respiration, is approximately 500 ml. To conduct the test, inhale a normal breath, and then exhale a normal breath of air into the spirometer mouthpiece. (Do not force the expiration!) Record the volume and repeat the test twice.

trial 1:  ml  trial 2:  ml

trial 3:  ml  average TV:  ml

4.	Compute the subject’s minute respiratory volume (MRV) using the following formula:

MRV 5 TV 3 respirations/min 5  ml/min

5.	Measuring expiratory reserve volume (ERV). The ERV is the volume of air that can be forcibly exhaled after a normal expiration. Normally it ranges between 700 and 1200 ml.

Inhale and exhale normally two or three times, then insert the spirometer mouthpiece and exhale forcibly as much of the additional air as you can. Record your results, and repeat the test twice again.

trial 1:  ml	trial 2:  ml

trial 3:  ml	average ERV:  ml

ERV is dramatically reduced in conditions in which the elasticity of the lungs is decreased by a chronic obstructive pulmonary disease (COPD) such as emphysema. Since energy must be used to deflate the lungs in such conditions, expiration is physically exhausting to individuals suffering from COPD. l

6.	Measuring vital capacity (VC). The VC, or total exchangeable air of the lungs (the sum of TV 1 IRV 1 ERV), normally ranges from 3600 ml to 4800 ml.

Breathe in and out normally two or three times, and then bend forward and exhale all the air possible. Then, as you raise yourself to the upright position, inhale as fully as possible. It is important to strain to inhale the maximum amount of air that you can. Quickly insert the mouthpiece, and exhale as forcibly as you can. Record your results and repeat the test twice again.

trial 1:  ml	trial 2:  ml

trial 3:  ml	average VC:  ml

7.	The inspiratory reserve volume (IRV), or volume of air that can be forcibly inhaled following a normal inspiration, can now be computed using the average values obtained for TV, ERV, and VC and plugging them into the equation:

IRV 5 VC 2 (TV 1 ERV)

Record your average IRV:  ml

The normal IRV is substantial, ranging from 1900 to 3100 ml. How does your computed value compare?

Steps 8–10, which provide common directions for use of both nonrecording and recording spirometers, continue on page 406 after the wet recording spirometer directions.

Using a Recording Spirometer

1.	In preparation for recording, familiarize yourself with the spirometer by comparing it to the equipment illustrated in Figure 37A.4b.

2.	Examine the chart paper, noting that its horizontal lines represent milliliter units. To apply the chart paper to the recording drum, lift the drum retainer and then remove the kymograph drum. Wrap a sheet of chart paper around the drum, making sure that the right edge overlaps the left. Fasten it with tape, and then replace the kymograph drum and lower the drum retainer into its original position in the hole in the top of the drum.

3.	Raise and lower the floating bell several times, noting as you do so that the ventilometer pen moves up and down on the drum. This pen, which writes in black ink, will be used for recording and should be adjusted so that it records in the approximate middle of the chart paper. This adjustment is made by repositioning the floating bell using the reset knob on the metal pulley at the top of the spirometer apparatus. The other pen, the respirometer pen, which records in red ink, will not be used for these tests and should be moved away from the drum’s recording surface.

4.	Record your normal respiratory rate. Clean the nose clips with an alcohol swab. While you wait for the alcohol to air dry, count and record your normal respiratory rate.

Respirations per minute:

5.	Recording tidal volume. After the alcohol has air dried, apply the nose clips to your nose. This will enforce mouth breathing.

Open the free-breathing valve. Insert a disposable cardboard mouthpiece into the end (valve assembly) of the breathing tube, and then insert the mouthpiece into your mouth. Practice breathing for several breaths to get used to the apparatus. At this time, you are still breathing room air.

Set the spirometer switch to SLOW (32 mm/min). Close the free-breathing valve, and breathe in a normal manner for 2 minutes to record your tidal volume—the amount of air inspired or expired with each normal respiratory cycle. This recording should show a regular pattern of inspiration-expiration spikes and should gradually move upward on the chart paper. (A downward slope indicates that there is an air leak somewhere in the system—most likely at the mouthpiece.) Notice that on an apparatus using a counterweighted pen, such as the Collins–9L Ventilometer shown in Fig. 37A.4b, inspirations are recorded by upstrokes and expirations are recorded by downstrokes.*

6.	Recording vital capacity. To record your vital capacity, take the deepest possible inspiration you can and then exhale to the greatest extent possible—really push the air out. The recording obtained should resemble that shown in Figure 37A.5. Repeat the vital capacity measurement twice again. Then turn off the spirometer and remove the chart paper from the kymograph drum.

7.	Determine and record your measured, averaged, and corrected respiratory volumes. Because the pressure and temperature inside the spirometer are influenced by room temperature and differ from those in the body, all measured values are to be multiplied by a BTPS (body temperature, atmospheric pressure, and water saturation) factor. At room temperature, the BTPS factor is typically 1.1 or very close to that value. Hence, you will multiply your measured values by 1.1 to obtain your corrected respiratory volume values. Copy the averaged and corrected values onto the Exercise 37A Review Sheet.

•	Tidal volume (TV). Select a typical resting tidal breath recording. Subtract the millimeter value of the trough (exhalation) from the millimeter value of the peak (inspiration). Record this value below as measured TV 1. Select two other TV tracings to determine the TV values for the TV 2 and TV 3 measurements. Then, determine your average TV and multiply it by 1.1 to obtain the BTPS-corrected average TV value.

measured TV 1:  ml	average TV:  ml

measured TV 2:  ml	corrected average TV:

measured TV 3:  ml		 ml

Also compute your minute respiratory volume (MRV) using the following formula:

MRV 5 TV 3 respirations/min 5  ml/min

•	Inspiratory capacity (IC). In the first vital capacity recording, find the expiratory trough immediately preceding the maximal inspiratory peak achieved during vital capacity determination. Subtract the milliliter value of that expiration from the value corresponding to the peak of the maximal inspiration that immediately follows. For example, according to Figure 37A.5, these values would be

6600 2 3650 5 2950 ml

Record your computed value and the results of the two subsequent tests on the appropriate lines below. Then calculate the measured and corrected inspiratory capacity averages and record.

measured IC 1:  ml	average IC:  ml

measured IC 2:  ml	corrected average IC:

measured IC 3:  ml		 ml

•	Inspiratory reserve volume (IRV). Subtract the corrected average tidal volume from the corrected average for the inspiratory capacity and record below.

IRV 5 corrected average IC 2 corrected average TV

corrected average IRV  ml

•	Expiratory reserve volume (ERV). Subtract the number of milliliters corresponding to the trough of the maximal expiration obtained during the vital capacity recording from milliliters corresponding to the last normal expiration before the VC maneuver is performed. For example, according to Figure 37A.5, these values would be

3650 ml 2 2050 ml 5 1600 ml

Record your measured and averaged values (three trials) below.

measured ERV 1:  ml	average ERV:  ml

measured ERV 2:  ml	corrected average ERV:

measured ERV 3:  ml	 ml

•	Vital capacity (VC). Add your corrected values for ERV and IC to obtain the corrected average VC. Record below and on the Exercise 37A Review Sheet.

corrected average VC:  ml

[Now continue with step 8 (below) whether you are following the procedure for the nonrecording or recording spirometer.]

8.	Figure out how closely your measured average vital capacity volume compares with the predicted values for someone your age, sex, and height. Obtain the predicted figure either from Table 37A.1 (male values) or Table 37A.2 (female values). Notice that you will have to convert your height in inches to centimeters (cm) to find the corresponding value. This is easily done by multiplying your height in inches by 2.54.

Computed height:  cm

Predicted VC value (obtained from the appropriate table):

ml

Use the following equation to compute your VC as a percentage of the predicted VC value:

% of predicted VC 5 averaged measured VC 3 100

predicted value

% predicted VC value:  %

Figure 37A.2 on page 401 is an idealized tracing of the respiratory volumes described and tested in this exercise. Examine it carefully. How closely do your test results compare to the values in the tracing?

9.	Computing residual volume. A respiratory volume that cannot be experimentally demonstrated here is the residual volume (RV). RV is the amount of air remaining in the lungs after a maximal expiratory effort. The presence of residual air (usually about 1200 ml) that cannot be voluntarily flushed from the lungs is important because it allows gas exchange to go on continuously—even between breaths.

Although the residual volume cannot be measured directly, it can be approximated by using one of the following factors:

For ages 16–34    Factor 5 1.250

For ages 35–49    Factor 5 1.305

For ages 50–69    Factor 5 1.445

Compute your predicted RV using the following equation:

RV 5 VC 3 factor

10.	Recording is finished for this subject. Before continuing with the next member of your group:

•	Dispose of used cardboard mouthpieces in the autoclave bag.

•	Swish the valve assembly (if removable) in the 70% ethanol solution, then rinse with tap water.

•	Put a fresh mouthpiece into the valve assembly (or on the stem of the handheld spirometer). Using the procedures outlined above, measure and record the respiratory volumes for all members of your group. n

Forced Expiratory Volume (FEVT) Measurement

While not really diagnostic, pulmonary function tests can help the clinician distinguish between obstructive and restrictive pulmonary diseases. (In obstructive disorders, like chronic bronchitis and asthma, airway resistance is increased, whereas in restrictive diseases, such as polio and tuberculosis, total lung capacity declines.) Two highly useful pulmonary function tests used for this purpose are the FVC and the FEVT.

The FVC (forced vital capacity) measures the amount of gas expelled when the subject takes the deepest possible breath and then exhales forcefully and rapidly. This volume is reduced in those with restrictive pulmonary disease. The FEVT (forced expiratory volume) involves the same basic testing procedure, but it specifically looks at the percentage of the vital capacity that is exhaled during specific time intervals of the FVC test. FEV1, for instance, is the amount exhaled during the first second. Healthy individuals can expire 75% to 85% of their FVC in the first second. The FEV1 is low in those with obstructive disease.

Activity 4:

Measuring the FVC and FEV1

Directions provided here for the FEVT determination apply only to the recording spirometer.

1.	Prepare to make your recording as described for the recording spirometer, steps 1–5 on page 404.

2.	At a signal agreed upon by you and your lab partner, take the deepest inspiration possible and hold it for 1 to 2 seconds. As the inspiratory peak levels off, your partner is to change the drum speed to FAST (1920 mm/min) so that the distance between the vertical lines on the chart represents a time of 1second.

3.	Once the drum speed is changed, exhale as much air as rapidly and forcibly as possible.

4.	When the tracing plateaus (bottoms out), stop recording and determine your FVC. Subtract the milliliter reading in the expiration trough (the bottom plateau) from the preceding inhalation peak (the top plateau). Record this value.

FVC:  ml

5.	Prepare to calculate the FEV1. Draw a vertical line intersecting with the spirogram tracing at the precise point that exhalation began. Identify this line as line 1. From line 1, measure 32 mm horizontally to the left, and draw a second vertical line. Label this as line 2. The distance between the two lines represents 1 second, and the volume exhaled in the first second is read where line 2 intersects the spirogram tracing. Subtract that milliliter value from the milliliter value of the inhalation peak (at the intersection of line 1), to determine the volume of gas expired in the first second. According to the values given in Figure 37A.6, that figure would be 3400 ml (6800 ml 2 3400 ml). Record your measured value below.

Milliliters of gas expired in second 1:  ml

6.	To compute the FEV1 use the following equation:

FEV1 5 volume expired in second 1 3 100%

FVC volume

Record your calculated value below and on the Exercise 37A Review Sheet.

FEV1:  % of FVC n

Using the Pneumograph 
to Determine Factors Influencing Rate and Depth of Respiration*

The neural centers that control respiratory rhythm and maintain a rate of 12 to 18 respirations/min are located in the medulla and pons. On occasion, input from the stretch receptors in the lungs (via the vagus nerve to the medulla) modifies the respiratory rate, as in cases of extreme overinflation of the lungs (Hering-Breuer reflex).

Death occurs when medullary centers are completely suppressed, as from an overdose of sleeping pills or gross overindulgence in alcohol, and respiration ceases completely. l

Although the nervous system centers initiate the basic rhythm of breathing, there is no question that physical phenomena such as talking, yawning, coughing, and exercise can modify the rate and depth of respiration. So, too, can chemical factors such as changes in oxygen or carbon dioxide concentrations in the blood or fluctuations in blood pH. Changes in carbon dioxide blood levels seem to act directly on the medulla control centers, whereas changes in pH and oxygen concentrations are monitored by chemoreceptor regions in the aortic and carotid bodies, which in turn send input to the medulla. The experimental sequence in this section is designed to test the relative importance of various physical and chemical factors in the process of respiration.

The pneumograph, an apparatus that records variations in breathing patterns, is the best means of observing respiratory variations resulting from physical and chemical factors. The chest pneumograph is a coiled rubber hose that is attached around the thorax. As the subject breathes, chest movements produce pressure changes within the pneumograph that are transmitted to a recorder. BIOPAC® uses an airflow transducer to record breathing movements.

The instructor will demonstrate the method of setting up the recording equipment and discuss the interpretation of the results. Work in pairs so that one person can mark the record to identify the test for later interpretation. Ideally, the student being tested should face away from the recording apparatus to prevent voluntary modification of the record.

Activity 5:

Visualizing Respiratory Variations

Using the Physiograph-Pneumograph Apparatus

1.	Attach the pneumograph tubing firmly, but not restrictively, around the thoracic cage at the level of the sixth rib, leaving room for chest expansion during testing. If the subject is female, position the tubing above the breasts to prevent slippage during testing. Set the pneumograph speed at 1 or 2, and the time signal at 10-second intervals. Record quiet breathing for 1 minute with the subject in a sitting position.

Record breaths per minute:

2.	Make a vital capacity tracing: Record a maximal inhalation followed by a maximal exhalation. This should correlate to the vital capacity measurement obtained earlier and will provide a baseline for comparison during the rest of the pneumograph testing. Stop recording, and mark the graph appropriately to indicate: tidal volume, expiratory reserve volume, inspiratory reserve volume, and vital capacity (the total of the three measurements). Also mark, with arrows, the direction the recording stylus moves during inspiration and during expiration.

Measure in mm the height of the vital capacity recording. Divide the vital capacity measurement average recorded on the Exercise 37A Review Sheet by the millimeter figure to obtain the volume (in milliliters of air) represented by 1 mm on the recording. For example, if your vital capacity reading is 4000 ml and the vital capacity tracing occupies a vertical distance of 40 mm on the pneumograph recording, then a vertical distance of 1 mm equals 100 ml of air.

Record your computed value:  ml air/mm

3.	Record the subject’s breathing as he or she performs activities from the list below. Make sure the record is marked accurately to identify each test conducted. Record your results on the Exercise 37A Review Sheet.

talking	swallowing water

yawning	coughing

laughing	lying down

standing	doing a math problem
  (concentrating)

running in place

4.	Without recording, have the subject breathe normally for 2 minutes, then inhale deeply and hold his or her breath for as long as he or she can.

Time the breath-holding interval:  sec

As the subject exhales, turn on the recording apparatus and record the recovery period (time to return to normal breathing—usually slightly over 1 minute):

Time of recovery period:  sec

Did the subject have the urge to inspire or expire during breath holding?

Without recording, repeat the above experiment, but this time exhale completely and forcefully after taking the deep breath. What was observed this time?

Explain the results. (Hint: The vagus nerve is the sensory nerve of the lungs and plays a role here.)

5.	Have the subject hyperventilate (breathe deeply and forcefully at the rate of 1 breath/4 sec) for about 30 seconds.* Record both during and after hyperventilation. How does the pattern obtained during hyperventilation compare with that recorded during the vital capacity tracing?

Is the respiratory rate after hyperventilation faster or slower than during normal quiet breathing?

6.	Repeat the above test, but do not record until after hyperventilating. After hyperventilation, the subject is to hold his or her breath as long as he or she can. Can the breath be held for a longer or shorter time after hyperventilating?

7.	Without recording, have the subject breathe into a paper bag for 3 minutes, then record his or her breathing movements.

During the bag-breathing exercise the subject’s partner should watch the subject carefully for any untoward reactions.

Is the breathing rate faster or slower than that recorded during normal quiet breathing?

After hyperventilating?

8.	Run in place for 2 minutes, and then have your partner determine the length of time that you can hold your breath.

Length of breath-holding:  sec

9.	To prove that respiration has a marked effect on circulation, conduct the following test. Have your lab partner record the rate and relative force of your radial pulse before you begin.

Rate:  beats/min	Relative force:

Inspire forcibly. Immediately close your mouth and nose to retain the inhaled air, and then make a forceful and prolonged expiration. Your lab partner should observe and record the condition of the blood vessels of your neck and face, and again immediately palpate the radial pulse.

Observations:

Radial pulse:  beats/min	Relative force:

Explain the changes observed.

Dispose of the paper bag in the autoclave bag. Keep the pneumograph records to interpret results and hand them in if requested by the instructor. Observation of the test results should enable you to determine which chemical factor, carbon dioxide or oxygen, has the greatest effect on modifying the respiratory rate and depth. n

Measuring Respiratory Volumes Using BIOPAC®

In this activity, you will measure respiratory volumes using the BIOPAC® airflow transducer. An example of these volumes is demonstrated in the computer-generated spirogram in Figure 37A.7. Since it is not possible to measure residual volume (RV) using the airflow transducer, assume that it is 1.0 liter for each subject, which is a reasonable estimation. It is also important to estimate the predicted vital capacity of the subject for comparison to the measured value. A rough estimate of the vital capacity in liters (VC) of a subject can be calculated using the following formulas based on height in centimeters (H) and age in years (A).

Male VC      5 (0.052)H 2 (0.022)A 2 3.60

Female VC 5 (0.041)H 2 (0.018)A 2 2.69

Because there are many other factors besides height and age that influence vital capacity, it should be assumed that measured values 20% lesser or greater than the calculated predicted value are considered normal.

Setting up the Equipment

1.	Connect the BIOPAC® unit to the computer and turn the computer ON.

2.	Make sure the BIOPAC® MP30 (or MP 35) unit is OFF.

3.	Plug in the equipment as shown in Figure 37A.8.

•	Airflow transducer—CH 1

4.	Turn the BIOPAC® MP30 (or MP35) unit ON.

5.	Place a clean bacteriological filter onto the end of the BIOPAC® calibration syringe as shown in Figure 37A.9. Since the subject will be blowing through a filter, it is necessary to use a filter for calibration also.

6.	Insert the calibration syringe and filter assembly into the airflow transducer on the side labeled Inlet.

7.	Start the BIOPAC® Student Lab program on the computer by double-clicking the icon on the desktop or by following your instructor’s guidance.

8.	Select lesson L12-Lung-1 from the menu, and click OK.

9.	Type in a filename that will save this subject’s data on the computer hard drive. You may want to use subject last name followed by Lung-1 (for example, SmithLung-1), then click OK.

Calibrating the Equipment

There are two precautions that must be followed:

•	The airflow transducer is sensitive to gravity, so it must be held directly parallel to the ground during calibration and recording.

•	Do not hold onto the airflow transducer when it is attached to the calibration syringe and filter assembly—the syringe tip is likely to break. See Figure 37A.10 for the proper handling of the calibration assembly.

1.	Make sure the plunger is pulled all the way out. While the assembly is held in a steady position parallel to the ground, click Calibrate and then OK after you have read the alert box. This part of the calibration will terminate automatically with an alert box ensuring that you have read the onscreen instructions and those indicated in step 2, below.

2.	The final part of the calibration will involve simulating five breathing cycles using the calibration syringe. A single cycle consists of:

•	Pushing the plunger in (taking one second for this stroke).

•	Waiting for two seconds.

•	Pulling the plunger out (taking one second for this stroke).

•	Waiting two seconds.

Remember, the airflow transducer must be held directly parallel to the ground during calibration and recording.

4.	When ready to perform this second stage of the calibration, click Yes. After you have completed five cycles, click End Calibration.

5.	Observe the data, which should look similar to that in Figure 37A.11.

•	If the data look very different, click Redo Calibration and repeat the steps above.

•	If the data look similar, gently remove the calibration syringe, leaving the air filter attached to the transducer. Proceed to the next section.

Recording the Data

Follow these procedures very precisely because the airflow transducer is very sensitive. Hints to obtaining the best data:

•	Always insert air filter on, and breathe through, the transducer side labeled Inlet.

•	Keep the airflow transducer upright at all times.

•	The subject should not look at the computer screen during the recording of data.

•	The subject must keep a nose clip on throughout the experiment.

1.	Insert a clean mouthpiece into the air filter that is already attached to the airflow transducer. Be sure that the filter is attached to the Inlet side of the airflow transducer.

2.	Write the name of the subject on the mouthpiece and air filter. For safety purposes, each subject must use their own air filter and mouthpiece.

3.	The subject should now place the nose clip on the nose (or hold the nose very tightly with finger pinch), wrap the lips tightly around the mouthpiece, and begin breathing normally through the airflow transducer, as shown in Figure 37A.12.

4.	When prepared, the subject will complete the following unbroken series with nose plugged and lips tightly sealed around the mouthpiece:

•	Take three normal breaths (1 breath 5 inhale 1 exhale).

•	Inhale as much air as possible, then exhale back to normal breathing.

•	Take three normal breaths.

•	Exhale as much air as possible, then inhale back to normal breathing.

•	Take three normal breaths.

5.	When the subject is prepared to proceed, click Record on the first normal inhalation and proceed. When the subject finishes the last exhalation at the end of the series, click Stop.

6.	Observe the data, which should look similar to that in Figure 37A.13.

•	If the data look very different, click Redo and repeat the steps above. Be certain that the lips are sealed around the mouthpiece, the nose is completely plugged, and the transducer is upright.

•	If the data look similar, proceed to step 7.

7.	When finished, click Done. A pop-up window will appear.

•	Click Yes if you are done and want to stop recording.

•	To record from another subject select Record from another Subject and return to step 1 under Recording the Data. Note that you will not need to redo the calibration procedure for the second subject.

•	If continuing to the Data Analysis section, select Analyze current data file and proceed to step 2 of the Data Analysis section.

Data Analysis

1.	If just starting the BIOPAC® program to perform data analysis, enter Review Saved Data mode and choose the file with the subject’s Lung data (for example, SmithLung-1).

2.	Observe how the channel numbers are designated: CH 1— Airflow; CH 2—Volume.

3.	To set up the display for optimal viewing, hide CH 1—Airflow. To do this, hold down the control key (PC) or option key (Mac) while using the cursor to click on channel box 1 (the small box with a 1 in the upper left of the screen).

4.	To analyze the data, set up the first pair of channel/ measurement boxes at the top of the screen by selecting the following channel and measurement type from the drop-down menu:

Channel	Measurement

CH 2	p-p

5.	Use the arrow cursor and click on the I-beam cursor box on the lower right side of the screen to activate the “area selection” function. Using the activated I-beam cursor, highlight the region containing the first three breaths as shown in Figure 37A.14.

6.	Observe that the computer automatically calculates the p-p for the selected area. This measurement, calculated from the data by the computer, represents the difference in value between the highest and lowest values in the selected area.

7.	For the first three breaths that are highlighted, the p-p measurement represents the tidal volume (in liters). Record this value in the chart below.

8.	Use the I-beam cursor to measure the IRV. Highlight from the peak of maximum inhalation to the peak of the last normal inhalation just before it (see Figure 37A.7 for an example of IRV). Observe and record the p-p value in the chart (to the nearest 1/100 liter).

9.	Use the I-beam cursor to measure the ERV. Highlight from the trough of maximum exhalation to the trough of the last normal exhalation just before it (see Figure 37A.7 for an example of ERV). Observe and record the p-p value in the chart (to the nearest 1/100 liter).

10.	Lastly, use the I-beam cursor to measure the VC. Highlight from the trough of maximum exhalation to the peak of maximum inhalation (see Figure 37A.7 for an example of VC). Observe and record the p-p value in the chart (to the nearest 1/100 liter).

11.	When finished, Exit the program.

Using the measured data, calculate the following capacities:

Use the formula in the introduction of this activity (page 412) to calculate the predicted vital capacity of the subject based on height and age.

Predicted VC:  liters

How does the measured vital capacity compare to the predicted vital capacity?

Describe why height and weight might correspond with a subject’s VC.

What other factors might influence the VC of a subject?

n

Role of the Respiratory System in Acid-Base 
Balance of Blood

As you have already learned, pulmonary ventilation is necessary for continuous oxygenation of the blood and removal of carbon dioxide (a waste product of cellular respiration) from the blood. Blood pH must be relatively constant for the cells of the body to function optimally. The carbonic acid– bicarbonate buffer system of the blood is extremely important because it helps stabilize arterial blood pH at 7.4 6 0.02.

When carbon dioxide diffuses into the blood from the tissue cells, much of it enters the red blood cells, where it combines with water to form carbonic acid (Figure 37A.15):

Carbonic anhydrase

H2O 1 CO2   enzyme present in RBC  Æ H2CO3

Some carbonic acid is also formed in the plasma, but that reaction is very slow because of the lack of the carbonic anhydrase enzyme. Shortly after it forms, carbonic acid dissociates to release bicarbonate (HCO32) and hydrogen (H1) ions. The hydrogen ions that remain in the cells are neutralized, or buffered, when they combine with hemoglobin molecules. If they were not neutralized, the intracellular pH would become very acidic as H1 ions accumulated. The bicarbonate ions diffuse out of the red blood cells into the plasma, where they become part of the carbonic acid–bicarbonate buffer system. As HCO32 follows its concentration gradient into the plasma, an electrical imbalance develops in the RBCs that draws Cl2 into them from the plasma. This exchange phenomenon is called the chloride shift.

Acids (more precisely, H1) released into the blood by the body cells tend to lower the pH of the blood and to cause it to become acidic. On the other hand, basic substances that enter the blood tend to cause the blood to become more alkaline and the pH to rise. Both of these tendencies are resisted in large part by the carbonic acid–bicarbonate buffer system. If the H1 concentration in the blood begins to increase, the H1 ions combine with bicarbonate ions to form carbonic acid (a weak acid that does not tend to dissociate at physiological or acid pH) and are thus removed.

H1 1 HCO32 Æ H2CO3

Likewise, as blood H1 concentration drops below what is desirable and blood pH rises, H2CO3 dissociates to release bicarbonate ions and H1 ions to the blood. The released H1 lowers the pH again. The bicarbonate ions, being weak bases, are poorly functional under alkaline conditions and have little effect on blood pH unless and until blood pH drops toward acid levels.

H2CO3 Æ H1 1 HCO32

In the case of excessively slow or shallow breathing (hypo-ventilation) or fast deep breathing (hyperventilation), the amount of carbonic acid in the blood can be greatly modified—increasing dramatically during hypoventilation and decreasing substantially during hyperventilation. In either situation, if the buffering ability of the blood is inadequate, respiratory acidosis or alkalosis can result. Therefore, maintaining the normal rate and depth of breathing is important for proper control of blood pH.

Activity 6:

Demonstrating the Reaction 
Between Carbon Dioxide (in 
Exhaled Air) and Water

1.	Fill a beaker with 100 ml of distilled water.

2.	Add 5 ml of 0.05 MNaOH and five drops of phenol red. Phenol red is a pH indicator that turns yellow in acidic solutions.

3.	Blow through a straw into the solution.

What do you observe?

What chemical reaction is taking place in the beaker?

4.	Discard the straw in the autoclave bag. n

Activity 7:

Observing the Operation 
of Standard Buffers

1.	To observe the ability of a buffer system to stabilize the pH of a solution, obtain five 250-ml beakers and a wash bottle containing distilled water. Set up the following experimental samples:

Beaker 1:
(150 ml distilled water)	pH

Beaker 2:
(150 ml distilled water and 
1 drop concentrated HCl)	pH

Beaker 3:
(150 ml distilled water and 
1 drop concentrated NaOH)	pH

Beaker 4:
(150 ml standard buffer solution
[pH 7] and 1 drop concentrated HCl)	pH

Beaker 5:
(150 ml standard buffer solution
[pH 7] and 1 drop concentrated NaOH)	pH

2.	Using a pH meter standardized with a buffer solution of pH 7, determine the pH of the contents of each beaker and record above. After each and every pH recording, the pH meter switch should be turned to STANDBY, and the electrodes rinsed thoroughly with a stream of distilled water from the wash bottle.

3.	Add 3 more drops of concentrated HCl to beaker 4,

stir, and record the pH:

4.	Add 3 more drops of concentrated NaOH to beaker 5,

stir, and record the pH:

How successful was the buffer solution in resisting pH changes when a strong acid (HCl) or a strong base (NaOH) was added?

n

Activity 8:

Exploring the Operation of the Carbonic Acid–Bicarbonate 
Buffer System

To observe the ability of the carbonic acid–bicarbonate buffer system of blood to resist pH changes, perform the following simple experiment.

1.	Obtain two small beakers (50 ml), animal plasma, graduated cylinder, glass stirring rod, and a dropper bottle of 0.01 M HCl. Using the pH meter standardized with the buffer solution of pH 7.0, measure the pH of the animal plasma. Use only enough plasma to allow immersion of the electrodes and measure the volume used carefully.

pH of the animal plasma:

2.	Add 2 drops of the 0.01 M HCl solution to the plasma; stir and measure the pH again.

pH of plasma plus 2 drops of HCl:

3.	Turn the pH meter switch to STANDBY, rinse the electrodes, and then immerse them in a quantity of distilled water (pH 7) exactly equal to the amount of animal plasma used. Measure the pH of the distilled water.

pH of distilled water:

4.	Add 2 drops of 0.01 M HCl, swirl, and measure the pH again.

pH of distilled water plus the two drops of HCl:

Is the plasma a good buffer?

What component of the plasma carbonic acid–bicarbonate buffer system was acting to counteract a change in pH when HCl was added?

n

##	Exercise 37A

Figure 37A.1  Rib cage 
and diaphragm positions during breathing. (a) At the end of a normal inspiration; chest expanded, diaphragm depressed. (b) At the end of a normal expiration; chest depressed, diaphragm elevated.							Respiratory System Physiology#

Figure 37A.2  Spirographic record for a healthy young adult male.			#	Exercise 37A

Figure 37A.3  The Wright handheld dry spirometer. Reset to zero prior to each test.									*Note to the Instructor: The format of class data tabulation can be similar to that shown here. However, it would be interesting to divide the class into smokers and nonsmokers and then compare the mean average VC and ERV for each group. Such a comparison might help to determine if smokers are handicapped in any way. It also might be a good opportunity for an informal discussion of the early warning signs of bronchitis and emphysema, which are primarily smokers’ diseases.		Respiratory System Physiology#

Figure 37A.4  Wet spirometers. (a) The Phipps and Bird wet spirometer. (b) The Collins-9L wet recording 
spirometer.#	Exercise 37A												*If a Collins survey spirometer is used, the situation is exactly opposite: Upstrokes are expirations and downstrokes are inspirations.	Respiratory System Physiology#

Figure 37A.5  A typical spirometry recording of tidal volume, inspiratory capacity, expiratory reserve volume, and vital capacity. At a drum speed of 32 mm/min, each vertical column of the chart represents a time interval of 1 minute. (Note that downstrokes represent exhalations and upstrokes represent inhalations.)							#	Exercise 37A											Respiratory System Physiology#	Predicted Vital Capacities for Males

Height in centimeters

Age	146	148	150	152	154	156	158	160	162	164	166	168	170	172	174	176	178

16	3765	3820	3870	3920	3975	4025	4075	4130	4180	4230	4285	4335	4385	4440	4490	4540	4590

18	3740	3790	3840	3890	3940	3995	4045	4095	4145	4200	4250	4300	4350	4405	4455	4505	4555

20	3710	3760	3810	3860	3910	3960	4015	4065	4115	4165	4215	4265	4320	4370	4420	4470	4520

22	3680	3730	3780	3830	3880	3930	3980	4030	4080	4135	4185	4235	4285	4335	4385	4435	4485

24	3635	3685	3735	3785	3835	3885	3935	3985	4035	4085	4135	4185	4235	4285	4330	4380	4430

26	3605	3655	3705	3755	3805	3855	3905	3955	4000	4050	4100	4150	4200	4250	4300	4350	4395

28	3575	3625	3675	3725	3775	3820	3870	3920	3970	4020	4070	4115	4165	4215	4265	4310	4360

30	3550	3595	3645	3695	3740	3790	3840	3890	3935	3985	4035	4080	4130	4180	4230	4275	4325

32	3520	3565	3615	3665	3710	3760	3810	3855	3905	3950	4000	4050	4095	4145	4195	4240	4290

34	3475	3525	3570	3620	3665	3715	3760	3810	3855	3905	3950	4000	4045	4095	4140	4190	4225

36	3445	3495	3540	3585	3635	3680	3730	3775	3825	3870	3920	3965	4010	4060	4105	4155	4200

38	3415	3465	3510	3555	3605	3650	3695	3745	3790	3840	3885	3930	3980	4025	4070	4120	4165

40	3385	3435	3480	3525	3575	3620	3665	3710	3760	3805	3850	3900	3945	3990	4035	4085	4130

42	3360	3405	3450	3495	3540	3590	3635	3680	3725	3770	3820	3865	3910	3955	4000	4050	4095

44	3315	3360	3405	3450	3495	3540	3585	3630	3675	3725	3770	3815	3860	3905	3950	3995	4040

46	3285	3330	3375	3420	3465	3510	3555	3600	3645	3690	3735	3780	3825	3870	3915	3960	4005

48	3255	3300	3345	3390	3435	3480	3525	3570	3615	3655	3700	3745	3790	3835	3880	3925	3970

50	3210	3255	3300	3345	3390	3430	3475	3520	3565	3610	3650	3695	3740	3785	3830	3870	3915

52	3185	3225	3270	3315	3355	3400	3445	3490	3530	3575	3620	3660	3705	3750	3795	3835	3880

54	3155	3195	3240	3285	3325	3370	3415	3455	3500	3540	3585	3630	3670	3715	3760	3800	3845

56	3125	3165	3210	3255	3295	3340	3380	3425	3465	3510	3550	3595	3640	3680	3725	3765	3810

58	3080	3125	3165	3210	3250	3290	3335	3375	3420	3460	3500	3545	3585	3630	3670	3715	3755

60	3050	3095	3135	3175	3220	3260	3300	3345	3385	3430	3470	3500	3555	3595	3635	3680	3720

62	3020	3060	3110	3150	3190	3230	3270	3310	3350	3390	3440	3480	3520	3560	3600	3640	3680

64	2990	3030	3080	3120	3160	3200	3240	3280	3320	3360	3400	3440	3490	3530	3570	3610	3650

66	2950	2990	3030	3070	3110	3150	3190	3230	3270	3310	3350	3390	3430	3470	3510	3550	3600

68	2920	2960	3000	3040	3080	3120	3160	3200	3240	3280	3320	3360	3400	3440	3480	3520	3560

70	2890	2930	2970	3010	3050	3090	3130	3170	3210	3250	3290	3330	3370	3410	3450	3480	3520

72	2860	2900	2940	2980	3020	3060	3100	3140	3180	3210	3250	3290	3330	3370	3410	3450	3490

74	2820	2860	2900	2930	2970	3010	3050	3090	3130	3170	3200	3240	3280	3320	3360	3400	3440

Courtesy of Warren E. Collins, Inc., Braintree, Mass.180	182	184	186	188	190	192	194

4645	4695	4745	4800	4850	4900	4955	5005

4610	4660	4710	4760	4815	4865	4915	4965

4570	4625	4675	4725	4775	4825	4875	4930

4535	4585	4635	4685	4735	4790	4840	4890

4480	4530	4580	4630	4680	4730	4780	4830

4445	4495	4545	4595	4645	4695	4740	4790

4410	4460	4510	4555	4605	4655	4705	4755

4375	4425	4470	4520	4570	4615	4665	4715

4340	4385	4435	4485	4530	4580	4625	4675

4285	4330	4380	4425	4475	4520	4570	4615

4250	4295	4340	4390	4435	4485	4530	4580

4210	4260	4305	4350	4400	4445	4495	4540

4175	4220	4270	4315	4360	4410	4455	4500

4140	4185	4230	4280	4325	4370	4415	4460

4085	4130	4175	4220	4270	4315	4360	4405

4050	4095	4140	4185	4230	4275	4320	4365

4015	4060	4105	4150	4190	4235	4280	4325

3960	4005	4050	4090	4135	4180	4225	4270

3925	3970	4010	4055	4100	4140	4185	4230

3890	3930	3975	4020	4060	4105	4145	4190

3850	3895	3940	3980	4025	4065	4110	4150

3800	3840	3880	3925	3965	4010	4050	4095

3760	3805	3845	3885	3930	3970	4015	4055

3730	3770	3810	3850	3890	3930	3970	4020

3690	3730	3770	3810	3850	3900	3940	3980

3640	3680	3720	3760	3800	3840	3880	3920

3600	3640	3680	3720	3760	3800	3840	3880

3560	3600	3640	3680	3720	3760	3800	3840

3530	3570	3610	3650	3680	3720	3760	3800

3470	3510	3550	3590	3630	3670	3710	3740

Table 37A.1#	Exercise 37A	Predicted Vital Capacities for Females

Height in centimeters

Age	146	148	150	152	154	156	158	160	162	164	166	168	170	172	174	176	178

16	2950	2990	3030	3070	3110	3150	3190	3230	3270	3310	3350	3390	3430	3470	3510	3550	3590

17	2935	2975	3015	3055	3095	3135	3175	3215	3255	3295	3335	3375	3415	3455	3495	3535	3575

18	2920	2960	3000	3040	3080	3120	3160	3200	3240	3280	3320	3360	3400	3440	3480	3520	3560

20	2890	2930	2970	3010	3050	3090	3130	3170	3210	3250	3290	3330	3370	3410	3450	3490	3525

22	2860	2900	2940	2980	3020	3060	3095	3135	3175	3215	3255	3290	3330	3370	3410	3450	3490

24	2830	2870	2910	2950	2985	3025	3065	3100	3140	3180	3220	3260	3300	3335	3375	3415	3455

26	2800	2840	2880	2920	2960	3000	3035	3070	3110	3150	3190	3230	3265	3300	3340	3380	3420

28	2775	2810	2850	2890	2930	2965	3000	3040	3070	3115	3155	3190	3230	3270	3305	3345	3380

30	2745	2780	2820	2860	2895	2935	2970	3010	3045	3085	3120	3160	3195	3235	3270	3310	3345

32	2715	2750	2790	2825	2865	2900	2940	2975	3015	3050	3090	3125	3160	3200	3235	3275	3310

34	2685	2725	2760	2795	2835	2870	2910	2945	2980	3020	3055	3090	3130	3165	3200	3240	3275

36	2655	2695	2730	2765	2805	2840	2875	2910	2950	2985	3020	3060	3095	3130	3165	3205	3240

38	2630	2665	2700	2735	2770	2810	2845	2880	2915	2950	2990	3025	3060	3095	3130	3170	3205

40	2600	2635	2670	2705	2740	2775	2810	2850	2885	2920	2955	2990	3025	3060	3095	3135	3170

42	2570	2605	2640	2675	2710	2745	2780	2815	2850	2885	2920	2955	2990	3025	3060	3100	3135

44	2540	2575	2610	2645	2680	2715	2750	2785	2820	2855	2890	2925	2960	2995	3030	3060	3095

46	2510	2545	2580	2615	2650	2685	2715	2750	2785	2820	2855	2890	2925	2960	2995	3030	3060

48	2480	2515	2550	2585	2620	2650	2685	2715	2750	2785	2820	2855	2890	2925	2960	2995	3030

50	2455	2485	2520	2555	2590	2625	2655	2690	2720	2755	2785	2820	2855	2890	2925	2955	2990

52	2425	2455	2490	2525	2555	2590	2625	2655	2690	2720	2755	2790	2820	2855	2890	2925	2955

54	2395	2425	2460	2495	2530	2560	2590	2625	2655	2690	2720	2755	2790	2820	2855	2885	2920

56	2365	2400	2430	2460	2495	2525	2560	2590	2625	2655	2690	2720	2755	2790	2820	2855	2885

58	2335	2370	2400	2430	2460	2495	2525	2560	2590	2625	2655	2690	2720	2750	2785	2815	2850

60	2305	2340	2370	2400	2430	2460	2495	2525	2560	2590	2625	2655	2685	2720	2750	2780	2810

62	2280	2310	2340	2370	2405	2435	2465	2495	2525	2560	2590	2620	2655	2685	2715	2745	2775

64	2250	2280	2310	2340	2370	2400	2430	2465	2495	2525	2555	2585	2620	2650	2680	2710	2740

66	2220	2250	2280	2310	2340	2370	2400	2430	2460	2495	2525	2555	2585	2615	2645	2675	2705

68	2190	2220	2250	2280	2310	2340	2370	2400	2430	2460	2490	2520	2550	2580	2610	2640	2670

70	2160	2190	2220	2250	2280	2310	2340	2370	2400	2425	2455	2485	2515	2545	2575	2605	2635

72	2130	2160	2190	2220	2250	2280	2310	2335	2365	2395	2425	2455	2480	2510	2540	2570	2600

74	2100	2130	2160	2190	2220	2245	2275	2305	2335	2360	2390	2420	2450	2475	2505	2535	2565

Courtesy of Warren E. Collins, Inc., Braintree, Mass.180	182	184	186	188	190	192	194

3630	3670	3715	3755	3800	3840	3880	3920

3615	3655	3695	3740	3780	3820	3860	3900

3600	3640	3680	3720	3760	3800	3840	3880

3565	3605	3645	3695	3720	3760	3800	3840

3530	3570	3610	3650	3685	3725	3765	3800

3490	3530	3570	3610	3650	3685	3725	3765

3455	3495	3530	3570	3610	3650	3685	3725

3420	3460	3495	3535	3570	3610	3650	3685

3385	3420	3460	3495	3535	3570	3610	3645

3350	3385	3425	3460	3495	3535	3570	3610

3310	3350	3385	3425	3460	3495	3535	3570

3275	3310	3350	3385	3420	3460	3495	3530

3240	3275	3310	3350	3385	3420	3455	3490

3205	3240	3275	3310	3345	3380	3420	3455

3170	3205	3240	3275	3310	3345	3380	3415

3130	3165	3200	3235	3270	3305	3340	3375

3095	3130	3165	3200	3235	3270	3305	3340

3060	3095	3130	3160	3195	3230	3265	3300

3025	3060	3090	3125	3155	3190	3225	3260

2990	3020	3055	3090	3125	3155	3190	3220

2950	2985	3020	3050	3085	3115	3150	3180

2920	2950	2980	3015	3045	3080	3110	3145

2880	2920	2945	2975	3010	3040	3075	3105

2845	2875	2915	2940	2970	3000	3035	3065

2810	2840	2870	2900	2935	2965	2995	3025

2770	2805	2835	2865	2895	2925	2955	2990

2735	2765	2800	2825	2860	2890	2920	2950

2700	2730	2760	2795	2820	2850	2880	2910

2665	2695	2725	2755	2780	2810	2840	2870

2630	2660	2685	2715	2745	2775	2805	2830

2590	2620	2650	2680	2710	2740	2765	2795

Table 37A.2	Respiratory System Physiology#				*Note to the Instructor: This exercise may also be done without using the recording apparatus by simply having the students count the respiratory rate visually.#	Exercise 37A

Figure 37A.6  A recording of the forced vital capacity (FVC) and forced expiratory volume (FEV) or timed vital capacity test.	Respiratory System Physiology#																						*A sensation of dizziness may develop. (As the carbon dioxide is washed out of the blood by overventilation, the blood pH increases, leading to a decrease in blood pressure and reduced cerebral circulation.) The subject may experience a lack of desire to breathe after forced breathing is stopped. If the period of breathing cessation—apnea—is extended, cyanosis of the lips may occur.		#	Exercise 37A

Figure 37A.7  Example of a computer-generated spirogram.	Respiratory System Physiology#

Figure 37A.8  Setting up the BIOPAC® MP30 (or MP35) unit. Plug the airflow transducer into Channel 1.

Figure 37A.9  Placement of the calibration syringe and filter assembly onto the airflow transducer for calibration.

Figure 37A.10  Proper handling of the calibration assembly.#	Exercise 37AFigure 37A.11  Example of calibration data.

Figure 37A.13  Example of pulmonary data.

Figure 37A.12  Proper equipment setup for recording data.	Respiratory System Physiology#Pulmonary Measurements

Volumes	Measurement
(liters)

Tidal volume (TV)

Inspiratory reserve volume (IRV)

Expiratory reserve volume (ERV)

Vital capacity (VC)

Residual volume (RV)	1.00 (assumed)Figure 37A.14  Highlighting data for the first three breaths.							Calculated Pulmonary Capacities

Capacity	Formula	Calculation (liters)

Inspiratory capacity 	5 TV 1 IRV
(IC)

Expiratory capacity 	5 TV 1 ERV
(EC)

Functional residual 	5 ERV 1 RV
capacity (FRC)

Total lung capacity 	5 TV 1 RV 1
(TLC)		IRV 1 ERV		#	Exercise 37A

Figure 37A.15  Oxygen release and carbon dioxide loading at the tissues.	Respiratory System Physiology#