Document: CITTESTA

Name: Cisco Internetwork Troubleshooting Answers

The following are my answers, if you’ve got a different answer you might be right. Find a wrong answer and receive a $5 credit towards the CiscoTests CCIE program.

1. When troubleshooting a problem connecting a modem to a router which of the following is a useful diagnostic step?

Answer: Issue the show line command to check modem state

2. The follow lines are part of the output from a show interfaces command:

Ethernet 0 is up, Line protocol is up

Hardware is MCI Ethernet aa00.00400.0134 BIA 0000.0c00.4365

Ethernet address 131.0.8.11 subnet 255.255.255.0

Which hardware address will this station respond to

*Aa00.00400.0134

3. What does the SNAP mean in the following packet

* subnetwork access protocol

Capturing appletalk packets with your sniffer with your    encounter the following counter: 279 frames accepted

*the router has received 279 packets that met the capture filter criteria

Which problem will cause an increasing number of transitions occurred when you do a show interface token ring

The ring is repeatedly going down and coming back up

Which tool is best suited for testing signals across a PC / Modem interface

Breakout box

After capturing a variety of IPX traffic from your network how can you view only the packets using the 802.3 frame type.

*configure a display filter

What tool is useful for certification of your Lan infrastructure

cable tester

What command monitors RTMP route aging    acquisition advertisements

I don’t’ think this question has a correct answer from the answers listed, this is the way it appears verbatim on the test.

Debug apple-routing

You have noticed a few of your IPX clients on a serverless segment are having a problem connecting to their router since you’ve updated your router from an older IOS version to the newest version. What action can you take to resolve this problem

*increase the ipx-gns-response-delay time ont the router

What command displays the entries in the routing table

*Show ip route

What can be derived from the following section of a    packet:

*This is a    routing information packet

While capturing ip traffic with your network sniffer

You get the message 2279 frames accept what does this mean?

*the analyzer can display 2279 well formed packets that meet the capture filter criteria

Examine the graphic by clicking on the exhibit:

*Excessive high cpu utilization

What command can you use to obtain a routers firmware version?

*Show version

What is an advantage of a connectionless protocol?

*less network traffic

what is a common term cisco systems uses when referring to the frame type novell describes as ethernet 802.3

choose (1)

*novell-ether

What problem is likely to result in a users inability to see zones or services outside their own network?

phase 1 / phase ii incompability

results of a show controllers token command on one of your routers includes the information

“bridge capab1e”

show controllers token is the command

*the interface is bridge capable but not configured for bridging

What command can you use to obtain a routers configuration for your tech support engineer

* Show running-config

What does IPX internal network command do?

*Assigns the address that is advertised by NLSP and IPX wan on all router interfaces

show interfaces fddi 3/0 displays an upstream neighbor value of 0000 0 f8 0000 what is the signifcance of this address

the identity of the upstream neighbor is unknown

What which step of the troubleshooting process contains identifying a set of symptoms and associated causes?

*defining the problem

Which services are not available through Cisco online? (Choose two)

*Cisco Documentation

*Ordering Information

When trying to isolate and resolve a problem on a token ring network you issue a “show controllers token” one part of the result is listed below

Tx-beacon 150/0

After making changes you believe you solved the problem you issue the show controllers token command again and get the following result:

Tx-becaon 20/150

Trying the command later gives you following results

Tx-becoan 0/170

What was the result of your attempts to correct the problem?

- *beaconing has stopped

which tool can be used to verify cable length?

- *tdr

Which field in the partial display of the packet listed below indicates available space for incoming data below:

- *tcp: tcp window = 512

what is the source of a common novell network error referred to as “Configuration mistmatch”

- *servers or routers have assigned different network addresses to a common network

what command reports the discover of new zones?

*Debug apple zip

when diagnosing a problem the output of the show interfaces serial command includes the following line:

serial s0 is administratively down, line protocol is down

what does this mean?

*the interface has been taken down by an administrator

Which cisco product provides an RMON console    to analyze traffic and enable proactive management of switched internetworks

Traffic director

Regarding the following frame which statements are true? (choose 2)

Ipx: ipx header

Ipx: no align

Ipx: checksum = ffff

Ipx: length = 224

Ipx: transport control = 00

Ipx: 0000 = reservered ipx

Ipx: ….. 0000 = hop count

Ipx : dest network.node = 1000.fffffffffffff socket = 1106    (sap)

Ipx: src network.node = 100.02.60 c2 fc 79 socket = 1106 (sap)

* This is a broadcast packet

* This is uses a novell ether frame

An IP ping command displays a series of five exclamation points in the output. What is the significance of the exclamation points.

*five replies were received

Use the sample sniffer    internetwork analyzer sample decode, select the attributes that characterize this communciation (choose 3)

I believe it was a dump from a breakout box, it had the following columns:

Summary	delta	pdst/source	Dce	Dte	Hdlc	sabm		p/f = 1

Select the attributes that characterize this communication

*The data is using token ring framing

*The stations are using hdlc framing (given because hdlc appears in the dump)

*this is a serial communication

Which of the following protocols are always connectionless (choose 3)

- *IP

- *Arp

- *802.3

what is the show buffers command used for?

- *Showing trends that indicate the link is a bottleneck

What counter in the show interfaces serial command output indicates a modem or line problem exists?

*carrier transitions

what type of device is typically used to test physical connectivity?

*digital multimeter

Which tool is most useful for analysis of redesign, reconfiguration, or stress testing?

*simulation and modeling tools

what type of protocol requires the application to request retransmission of lost, missing or corrupt packets?

*connection oriented

which step follows isolating a problem isolating a problem to a specific device (router)

*swapping out the router and observe the result

Where should you first look for errors when troubleshooting a serial wan problem?

*interface operation with the show interface command

Where are the output of the debug and system error messages sent by default?

*console

What is the troubleshooting engine?

*a customer engineered tool that uses case based reasoning to solve common problems

A show interfaces serial command indicates a high number of interface resets, what is the significance of the interface resets counter?

*there have been several attempts to restart the interface with the protocol down but the interface up

Which command should you use on an appletalk network if you suspect problems and want to monitor neighbors becoming reachable or unreachable and interfaces coming up or down.

*debug apple events

When should you use the write core command?

*To obtain a memory image of a malfunctioning router

when troubleshooting a wan connection what command displays hardware type and version information?

*Show controller

The output of show buffers indicates an increasing number of trends under very big buffers what does this indicate?

*outgoing frames are being truncated because the very big buffer    size is too small

You have implementing an action plan to resolve a problem. What should your next action be if your previous action did not solve the problem?

*undue the changes that didn’t work

what is the purpose of the debug arp command?
*Determines if the router is sending and receiving arp requests?

How do you use the show processes command to see when processes are causing an excessive load on the router?

*issue the command at select time throughout the workday to note the variations in the values over time

If certain ip data is being unintentionally being filtered by one or more router interfaces the likely cause of the problem is:

*ip access list

which type of troubleshooting tool is commonly used to display packet data?

*Network monitor

which command restricts logging to the console only?

*no logging on

if users can access some hosts on a local and remote host on some segment but not others what is the likely cause of the problem?

*addressing and subnet mask problems

regarding the following show interfaces command output which of the following statements is true?

2200000 packets input, 200000000 bytes 0 no buffer

192000 broadcasts, 134 runts, 0 giants

3 input errors, 390 crc, 0 frame, 0 overrun, 0 ignore, 0 abort

0 input packets with dribble condition detected

23000230 packets output 30239203 bytes 0 no buffers

output packets: 436000 collisions

4 choices:

*the network is experiencing excessive collisions

what command should you use to display debugging information about ip packets?

*debug ip packet

which of the following    error logging methods produces the lowest overhead

*syslog server

which device commonly uses the rmon mib for    internetwork troubleshooting

protocol analzyer

*network monitor

global statistics screen of a internetwork anaylzer says that there is a large amount of SNMP traffic. What is the signfiicance of SNMP?

* The management overhead consititues a large portion of the link traffic

What will be indicated in the show interfaces command output if more than 3 consecutive keepalives have been missed by the e0 interface?

*Ethernet 0 is    up, line protocol down

Which is the possible cause of a host being unable offnet hosts through a router

*no default gateway specified

which of the following problems is likely to cause certain tcp/ip protocols to be routed but not others?

*Misconfigured access lists

what fact can be arrived from the following decoded packet
full dump includes: dlc header info

it’s a broadcast packet fffffffff

definitally an arp packet

You have to pick the correct address.

What tool is used to verify a optical fiber installation?

*Otdr

What is the purpose of the debug ip icmp command?

*display if the router is sending or receiving    icmp messages

The show Ipx traffic displays the bad hop count increasing and the routes disappear and reappear often. Which of the problems is likely a source of the symptoms?

*routes unpredicatably appearing and disappearing

Which cisco product works with sun net manager and hp openview?

*cisco works