CLSC

Cisco Lan Switch Configuration

- Type the command which will display the full route cache for the IP protocol

Show ip route cache

- True/False: You may specify both the maximum entires, and the expiration for the route cache.

# True

False

Reason: Using the "ipx route-cache max-size" and the "ipx route-cache inactivity-timeout" command you may specify the both the maximum size and the inactivity timeout for the route cache

- Which type of switching would you use at an ISP to charge back customers based on bandwidth use?

Netflow

Reason: Netflow switching supports the ability to capture accounting statistics for a wide variety of purposes such as network analysis and planning, accounting, and billing.

- Which of the following scenarios support NetFlow switching?

ISL/VLAN

ATM

# IP encapsulated traffic

Frame Relay with more than one input access control list on the interface

ATM LANE

Reason: NetFlow switching is supported on IP and IP-encapsulated traffic over all interface types and encapsulations, except for ISL/VLAN, ATM, Frame Relay interfaces (when more than one input access control list is used on the interface), and ATM LANE.

- Which of the following things are considering a NetFlow network flow?

Source ip address

Destination ip address

Time to Live (TTL)

Source port number

Destination port number

# Packet Latency

# Protocol type

Type of service

Input interface

Output interface

- Which are the two versions of the Netflow protocol

1,2

1,5

2,5

there is only one version of the netflow protocol

Reason: Version 1 was the initial release, version 5 was a later enhancement to add Border Gateway Protocol and flow sequence number. Version 2 through 4 were not released.    NetFlow uses UDP datagrams, which a sequence number for sanity checking.

- Which statement is the proper command to configure an interface on a Cisco 7500 series router.

Interface type slot/port-adapter/port

Interface type slot/port

Interface type port-adapter/slot/port

Interface type port/slot

Reason: interface type slot/port-adapter/port on a Cisco 7500, use interface type slot/port on a Cisco 7200 series router.

- Which command will display the NetFlow switching statistics for the IP protocol?

Show ip route flow

- When should you use the command “ip route-cache same-interface”

When you do not have a VIP card installed in the router

On a partially meshed media such as Frame Relay

When you have multiple protocols configured on a single interface

Whenever you get the “need for speed”

Reason: Although having fast switching on the same ip interface is generally not recommended (because it may interfere with redirection) it is useful when you have a partially meshed media such as frame relay.

- Type the statement which disables fast switching for the IP protocol.

No ip route-cache

Reason: ip route-cache enables high speed route caching (typically enabled by default), no ip route-cache disables route caching.

- What protocols must an interface be running to have fast switching enabled by default?

Appletalk

Netbios

IPX

IP

Reason: Fast switcing is enabled by default on all interfaces that support it. Appletalk, Banyan, DecNet, IPX, IOS CLNS, XNS all support fast switching.

- Which statement(s) are true about Random Early Detection?

Traffic is prioritized based on the type of service (TOS).

This feature is only available if you have a VIP card installed

This feature is available on T3, OC-3, and ATM interfaces

This feature is available on any interface faster than 10mbps

Reason: Random Early Detection is designed for congestion avoidance. Traffic is prioritized based on Type of Service (TOS), and is only available on T3, OC-3, and ATM.

- Which statement describes FIFO Queuing

- Packets are forwarded in the same order they arrive at the interface

- Packets are forwarded based on an assigned priority. The administrator must create priority lists and groups to define rules for assigning packets to a priority queue

- Allocates a percentage of bandwidth for a specified traffic by creating protocol queue lists and custom queue lists.

- Provides automatic traffic priority management, low bandwidth sessions have priority over high priority sessions.

Reason: FIFO stands for First In, First Out.

- Which statement describes Weighted Fair Queuing

- Packets are forwarded in the same order they arrive at the interface

- Packets are forwarded based on an assigned priority. The administrator must create priority lists and groups to define rules for assigning packets to a priority queue

- Allocates a percentage of bandwidth for a specified traffic by creating protocol queue lists and custom queue lists.

- Provides automatic traffic priority management, low bandwidth sessions have priority over high priority sessions.

Reason: Each session is assigned a weight, the higher the weight the lower the priority.

- Which type of queuing is the default for a serial interface connected to a 56kbps circuit?

Weighted Fair Queueing

Priority Queuing

FIFO Queuing

Custom Queuing

Reason: Weighted Fair Queuing is the default for all interfaces slower than 2.048mbps

- Which statement describes Custom Queuing

- Packets are forwarded in the same order they arrive at the interface

- Packets are forwarded based on an assigned priority. The administrator must create priority lists and groups to define rules for assigning packets to a priority queue

- Allocates a percentage of bandwidth for a specified traffic by creating protocol queue lists and custom queue lists.

- Provides automatic traffic priority management, low bandwidth sessions have priority over high priority sessions.

- Which statement describes Priority Queueing

- Packets are forwarded in the same order they arrive at the interface

- Packets are forwarded based on an assigned priority. The administrator must create priority lists and groups to define rules for assigning packets to a priority queue

- Allocates a percentage of bandwidth for a specified traffic by creating protocol queue lists and custom queue lists.

- Provides automatic traffic priority management, low bandwidth sessions have priority over high priority sessions.

- Which command will enable NetFlow switching on a Cisco 7500?

Protocol route-cache flow

No protocol route-cache

Protocol route-cache optimum

Protocol route-cache distributed

Reason: Protocol route-cache flow enables NetFlow switching,    No protocol route-cache enables process switching, Protocol route-cache optimum enables optimum switching, Protocol route-cache distributed enables distributed switching

- Which type of switching is the slowest?

Optimum switching

#### Distributed Switching

Netflow Switching

#### Process Switching

Reason: Process switching is the slowest because it does not use a route cache. To enable rpceoss switching use the command “no protocol route-cache”

- Which type of switching cannot be used on a Cisco 7200 because it requires second generation VIP cards?

Optimum switching

#### Distributed Switching

Netflow Switching

Process Switching

Reason: Distribute switching requires a second generation VIP line cards. To enable distributed switching use the command “protocol route-cache distributed”

- True/False: The Cisco 7500 has an integrated Route/Switch Processor (RSP) which uses route caching to forward packets?

#### TRUE

FALSE

Reason: the Cisco 7500 has an integrated RSP which uses route caching to remember which port(s) a packet should be forwarded to.

- True/False: The Cisco 7500 also uses Versatile Interface Processors (VIP), which are RISC based processors that hand all traffic up to the Route Switch Processor (RSP) to be forwarded?

TRUE

#### FALSE

Reason: the VIP card makes switching decisions locally using the route cache, thereby decreasing the load on the RSP.

- Which term accurately describes distributed switching? (1)

Putting high end routers on the core of the network, and workgroup switches on the distribution area

Using a combination of workgroup switches on the core and distribution portions of the network

#### Using Virtual Interface Processors to reduce load on the main Route Switch Processor

Using multiple Route Switch Processors to reduce load on the Virtual Interface Processors

Reason: Virtual interface processors make switching decisions locally based on the route cache received from the Route Switch Processor.

- True/False: Multiple VIP cards can be installed in one router?

TRUE

FALSE

Reason: You can have multiple VIP cards in a single router to improve performance.

- Which of the following can be used to make routing decisions

Link speed

Media type

Protocol

Distance

current load

Reason: A routing decisions can be based on a variety of information such as link speed, topological distance, and protocol.

- How many routing tables must a router maintain?

One master routing table for all protocols

One for each protocol

One shared routing table for all protocols, then one individual routing table for each protocol

One read/write master table, then one read only table for each protocol

Reason: A router maintains one separate routing table for each protocol on the router.

- Which of the following are Cisco basic switching paths?

Process Switching

Fast Switching

Optimum Switching

Distributed Switching

NetFlow Switching

Reason: Cisco supports Process Switching, Fast Switching, Optimum Switching, Distributed Switching and NetFlow switching.

- When statement describes Process Switching?

A. Received packets are copied into the system buffer    then the router looks up the layer 3 network address in the routing table and initializes the fast-switch cache.. The RSP computes the CRC.

B. The first packet is copied into packet memory and the destination network or host is found in the fast-switching cache. The interface processor computes the CRC.

C. Enabled by default on a 7500. Similar to fast switching, however it must be disabled for debugging.

D. Switching occurs on the VIP which maintains a copy of the routing cache, switch performance improves linearly with the number of VIP cards installed. Cannot be used on a Cisco 7200.

E. Allows you to collect information about accounting, billing, and chargeback for network and application resource utilization.

- When statement describes Optimum Switching?

A. Received packets are copied into the system buffer    then the router looks up the layer 3 network address in the routing table and initializes the fast-switch cache.. The RSP computes the CRC.

B. The first packet is copied into packet memory and the destination network or host is found in the fast-switching cache. The interface processor computes the CRC.

C. Enabled by default on a 7500. Similar to fast switching, however it must be disabled for debugging.

D. Switching occurs on the VIP which maintains a copy of the routing cache, switch performance improves linearly with the number of VIP cards installed. Cannot be used on a Cisco 7200.

E. Allows you to collect information about accounting, billing, and chargeback for network and application resource utilization.

- When statement describes Fast Switching?

A. Received packets are copied into the system buffer    then the router looks up the layer 3 network address in the routing table and initializes the fast-switch cache.. The RSP computes the CRC.

B. The first packet is copied into packet memory and the destination network or host is found in the fast-switching cache. The interface processor computes the CRC.

C. Enabled by default on a 7500. Similar to fast switching, however it must be disabled for debugging.

D. Switching occurs on the VIP which maintains a copy of the routing cache, switch performance improves linearly with the number of VIP cards installed. Cannot be used on a Cisco 7200.

E. Allows you to collect information about accounting, billing, and chargeback for network and application resource utilization.

- When statement describes Netflow Switching?

A. Received packets are copied into the system buffer    then the router looks up the layer 3 network address in the routing table and initializes the fast-switch cache.. The RSP computes the CRC.

B. The first packet is copied into packet memory and the destination network or host is found in the fast-switching cache. The interface processor computes the CRC.

C. Enabled by default on a 7500. Similar to fast switching, however it must be disabled for debugging.

D. Switching occurs on the VIP which maintains a copy of the routing cache, switch performance improves linearly with the number of VIP cards installed. Cannot be used on a Cisco 7200.

E. Allows you to collect information about accounting, billing, and chargeback for network and application resource utilization.

- When statement describes Distributed Switching?

A. Received packets are copied into the system buffer    then the router looks up the layer 3 network address in the routing table and initializes the fast-switch cache.. The RSP computes the CRC.

B. The first packet is copied into packet memory and the destination network or host is found in the fast-switching cache. The interface processor computes the CRC.

C. Enabled by default on a 7500. Similar to fast switching, however it must be disabled for debugging.

D. Switching occurs on the VIP which maintains a copy of the routing cache, switch performance improves linearly with the number of VIP cards installed. Cannot be used on a Cisco 7200.

E. Allows you to collect information about accounting, billing, and chargeback for network and application resource utilization.

- Which ASIC on both line modules and the supervisor engine provides arbitration to the switching bus among ports and among lines?

SAGE

SAINT

# SAMBA

PHOENIX

- What are the conditions of a password a catalyst 3000?

# You can only change the password from the console

# ESC-DEL to clear

The password must be at least 6 digits

# Must be the same as the catalyst stack

- Which two applications need queuing? (2)

Consistent utilization

Video Conferencing

Lack of Buffers on hosts

A congested network

Bursty traffic

- What is the purpose of VTP?

Allows Virtual Terminal sesions

# Maps trunking ports together

A required protocol for all VLANs

- Type the command which displays supervisor modules that provides network interface information

# Sh module

- True about RMON:

# Four groups: stats, history, alarm, event

Rmon’s can be maintained with software such as CiscoWorks or TrafficDirector

RMON is in EARL ASIC

# Will allow you to monitor/mirror traffic to another port

- Which statement is true regarding 802.10 VLANs:

Defines multiple protocol data unit

# VLAN ID is required

# Header includes clear header & protected header

Clear header replicates the source address contained in the MAC

- Which switch processor uses system software that governs the general control of the hardware, its config and diagnostic routines:

NCP

NMP

# MCP

LCP

- Which of the following are valid switch forwarding methods (3)

Cut-through

Cut-through with Fast-forward

Cut-through with fast frag

## Store and forward

Hybrid Store and forward

- Which 5000 component is similar to a learning bridge

LTL

CBL

Arbiter

# EARL

- Identify the two true statements regarding the Cisco supervisor engine options:

C5500 uses only original supervisor engine

# Supervisor II supports 2 100mb links

# Supervisor II is faster than Supervisor I

# Supervisor 2 Supports 1.2gb and 3.6gb backplane

- Which two tasks should you perform to configure VLANs and LAN emulation.

VLAN Admin parameters

VTP ports

# ATM & LANE

# VLAN ports

- Which of the following statements are true?

#### Demand nodes should always be placed as close as possible to their resource nodes

Resource nodes should be grouped together for optimum performance

Demand nodes are always workstations

Resource nodes should be placed on segments with other resource nodes

- Which statements are true regarding the C3000

2 slots for modules

7 slots for modules

160mbps to 560mbps

40mbs to 700mbps

- LAN Emulation Components: BUS & LES (and how they interact)

Transaction Order?

1. Client A sends a LE-ARP to LES

2. Client A intiates Data direct VCC to B

3. Bus forwards intial data frames to B

321

123

231

213

- What Command assigns port 2/1 to VLAN 610

# Set vlan 610 2/1

- Set up a management domain

# Set vtp

Set domain

Set managemetn

Set client

- How many 10base T ports on a 1912:

8

12

20

48

- True regarding serial connections through supervisor engine:

Port is synchronous

Use straight through cable to DTE

# Use straight through cable to DCE

# DCE is DB25

- Transceiver has a max distance of 1.2 miles (2km):

# Multimode

Singlemode (ATM & FE)

Singlemode FDDI

CAT 5

- Which command do you use to enable a LES and a LANE bus for the ELAN:

Lane elane name

Lane bus elan-name

Lane les bus elan-name

Lane les bus elan protocol name

# Lane server-bus elan-name

- Switch applications:

# Server cluster

Workstation cluster

10/100mbps workgroups

desktop manager

- Which two rules are true for allocating resources

##### Local resources should be placed nearest to those users which need them the most

##### Local resources should be on their own dedicated segment to enable full 100mbps throughput

Two demand segments should not be configured to share one resource segment

A five segment network with 4 demand nodes should be configured at the same speed that would make the switched network unneccessary

- LANE multimode fiber module

# Source is a laser

# Source is a led

# Max distance is 2km

Max distance is 10km

- Out of bound management examples:

console

telnet

asynchronous modem connection

snmp

- Correct order of steps for tracing frame through 5000

- bus arbiter issues a grant which signals SAINT ASIC to initate data transfer

- Ethernet port receives buffer, checks FCS

- SAINT ASIC ports request to bus arbiter to transfer frame

- A frame is transferred across switch backplane

3214

4123

2314

1324

- Describe half duplex

Can transmit both directions at the same time

Each station has a full send and receive channel

###### Similar to a one way bridge

It uses time division to determine which computer gets priority

- Which of the following routers has full supervisor redunandcy with a supervisor II installed?
	5000

5007

5500

- Allocating resource rules

Resource nodes should be grouped together in workgroups away from demand node traffic

Demand nodes should be located as close to their primary resource nodes as possible

Demand nodes should always have their own dedicated segments

*Note: According to Cisco if you have 3 demand segments accessing 1 resource segment it is the same as having them all in the same collision domain.

- You have configured multilink PPP, and an outbound dialer interface, what command do you use to let the dialer profile know that it is part of a multilink group: _show dialer____

- Which two statements are true regarding APART?

##### Disabling apart increases throughput

After disabling apart earl ceases to provide packet forwarding

##### After disabling apart earl continues to provide packet forwarding

Disabling apart decreases throughput

- Which vlan technology is a standard protocol on a catalyst 5000 that allows you to map trunking protocols together to create a integrated vlan implementation across a user defined management domain

Isl

Lane

##### Vtp

Vtam

- Which statements are true regarding vlan trunk protocol?

Vtp information can be distributed to all stations throughout the network including servers, routers, and switches that participate as a vlan configurator

VTP provides static reporting for added vlans across the network

VTP provides a manual mapping scheme going across mixed media backbones

VTP provides auto intelligence for configuring switches across the network

- What is the correct order for tracing a frames progress through a catalyst 5000?

- the bus arbiter issues a grant to local arbiter which at the appropriate time which at the appropraite time notifies the –saint asic to initiate data transfer

- the ethernet port fully receives buffers and checks the frame fcs

- the saint asic posts a request to the bus arbiter to transmit the frame across the switching bus

- the frame is transmitted across the switching backplane all ports receive the frame and store it in their input buffers

1234

2314

3142

4321

- Which statements are true regarding the store and forward switching methods?

In store and forward switching the switch receives the full frame before beginning to transmit it.

The switch checks the destination address as soon as it receives the header and begins to forward the frame immediately

##### Latency through the switch varies with frame length

Latency remains constant regardless of frame length

- Which two statements are true regarding out of bound management

- works in conjunction with ppp

- works in conjunction with slip

- the out of bound management features can be used to provide telnet access to a catalyst switch CLI

- out of bounds management can be used to establish snmp management sessions on an snmp management platform

- which two tasks should you perform to configure vlan and lan emulation

configure vlan administrative parameters

configure vtp ports

configure atm and lane

configure vlan ports

- which command displays the supervisor module that provides network interface information

show arp

show int

show module

show netstat

- which two statements are true regarding lan emulation components

1.the bus is responsible for handling both broadcasts and multicasts

2    the bus registers and resolves all mac address to atm addresses using the lane emulation address resolution protocol

3.when a device on the elan has data to send to another device on the elan the sending requests the atm address of the atm address of the destination from the bus

4.the les manages the stations that make up the elan

- how to use the port priority menu

it is used to setup stp priorities for each port

it is used to view and change stp path associated with each port

use it to setup stp priorities for each port

use it to setup and change the stp path associate with each port

- which two statements are true regarding the password on a catalyst 3000?

the switch must have the same password as the stack

must have a different password than the stack

you can delete the password by pressing the sys-req button on the back panel during boot

you can delete the 3000 password by pressing esc+del

- client a has data to send to client b, which order do the transactions occur?

client a sends a le-arp request to the les

client a initiates data direct vcc to client b

bus forwards initial data frames to client b

client a sends flush to bus

1,2,3,4

- which command do you use to enable a lec lane emulation client for the first elan?

lane client ethernet vlan number elan-name

enable lec vlan number elan-name

lane ethernet vlan# elan-name

enable ethernet lec vlan number elan-name

- which two statements are true regarding the supervisor engine

the supervisor engine must be installed in slot 4

the supervisor engine contains the network management process

the supervsior engine is the main system process in the switch

the supervisor engine contains a layer 3 switching engine

- which two commands allow you to overwrite or erase the switch parameters

set

show

clear

erase

- which switching mode eliminates the chances of fowarding collision fragments?

##### Store and forward

Fast forward

fragement free

Fast forward

Fast frag and forward

- Which two statements are true regarding the 2820

##### Two high speed expansion slots

Fore high speed expansion slots

##### Supports shared memory architecture with a 3mb packet buffer

Up to 8 vlans allowing ports to be grouped together

- Which ASIC is on the supervisor III engine and is a gigabit bridge used to create the crossbar fabric?

##### Phoenix

Sage

Saint

Samba

- Which catalyst 5000 switch components is similar to the function of a learning bridge or content addressable memory

Ltl

Cbl

Arbiter

Earl

- Which two statements regarding 802.10 vlans are true? (2)

##### The 802.10 said identifies traffic as belonging to a particular vlan

the 6 byte said allows for 4.29 billion distinct lans

the 802.10 said field is used as a vlan id

the fddi 802.10 said are associated by the catalyst 5000 ethernet vlan’s to create multiple broadcast domains

- which transceiver type has a maximum distance of 1.2 miles between stations?

##### Multimode

Single mode atm fe

Single mode Fddi

Category 5

- Which two statements are true regarding frame tagging (2)

Frame tagging is a technique is a technique used to identify frames based on user defined offsets

##### Frame tagging assigns a unique user id to each frame

A filtering table is developed for each switch

A unique identifier is placed in each frame as it is forwarded through the switching fabric

- Which two statements regarding demand and resource nodes are true?

##### Demand nodes include client applications and terminal services

Resource nodes include personal computers and workstations

##### Network traffic occurs primarly between demand and resource nodes

Network traffic occurs mainly between resource and resource nodes

- Which two statements are true regarding cat 5000 switch leds

##### Leds flash during startup and turn green once initialization is complete

##### An orange light can indicate a problem on some modules

A purple led indicates failure

Leds flashing during startup indiciate a lan failure

- Which switch processor primarily communicates    information between nmp and the line module communication processors distributed on the catalyst 5000 line modules.

Ncp

Nmp

##### Mcp

Lcp

- Which two statements are true regarding the lane single mode fiber module?

##### The optical    source is a laser

The optical source is a led

The maximum distance is 2km

The maximum span is 10km

- What are the two types of atm addresses?

##### Icd

Pop

Mcp

##### Dcc

Nfp

- Which catalyst 5000 product has full supervisor redundancy with a supervisor ii installed

5000

5002

5200

5500

- which statement(s) are true regarding the embedded rmon agent

the embedded rmon mib supports 4 groups: the statistics, history, alarm, and event group

rmon lets you monitor traffic from across a vlan to a single port for analysis

rmon solutions which include vlan director, traffic director, cisco view deliver a management system for growing switch networks

the rmon is contained in the earl asic

- which two statements are true regarding configuring fddi port parameters

the menu for fddi configuration will be available only if you have the fddi module installed

to display the fddi port configuration press F on the main menu

the ring status is show at the right of the menu indicates if the module has attached to the ring

the status has three possible states operation, non-operational, pending

- which two statements regarding remote resources are true?

Remote conversation takes place between a demand node and resource node located in different collision domains

A remote conversation takes place between a demand node and resource node    located in the same collision domain

The measurement of throughput in a remote conversation is determined by the amount of transactions which can take place

A switch with multiple segments enables multiple remote conversations between nodes in the same collision domain

- Which statement is true regarding half duplex ethernet operation

Half duplex is similar to a large bus with many passengers

## Half duplex is similar to a narrow one way bridge

Half duplex is the same as Simplex broadcasting

- Which two are switch applications?

##### Server cluster

Workstation switch cluster

10/100 workgroups

desktop manager

- which command sets up a management domain?

##### Set vtp

Set domain

Set management

Set client

- Which 3 statements are true regarding the following configuration lines (3)

Access list 800 deny aa bb 01

Access list 800 permit -1 –1

Ethernet 0

Access group 800 in

- this is a standard ipx access filter

- this is an extended ipx access filter

- the –1 is wildcard matching a packet type

- the bb entry refers to the network bb protocol 01

- traffic from aa to bb will not be filtered on interface ethernet0

- traffic from aa to bb with be filtered on interface ethernet0

- which routing protocol supports multiple network layer    routed protocols.

OSPF

RIP 1

RIP 2

# EIGRP

- Type the command to display the routing information field data of token ring frames passing through the router?

Debug rif I think show rif is also possible

- You want to configure to an interface access list    that filters all traffic except for stations on network 133.22.0.0. Which of the following access lists will accomplish this task?

Access list 1 deny all except 133.22.0.0

Access-list 1 permit 133.22.0.0

### Access-list 1 permit 133.22.0.0 0.0.255.255

Access-list 1 permit 0.0.0.0 133.22.0.0

Access-list 1 permit 133.22.255.255

- Type the command which will setup the bridge to use source route briding from ring 22 to ring 33

### Source-bridge 33 4 22

- Which of the following exhibits show the correct OSPF configuration?

#Example A:

Router ospf 100

Network 10.0.0.0

Network 209.76.25.0

Example B:

Router ospf pid 100

Network 10.0.0.0

Network 209.76.25.0

Example C:

Router ospf 100

Network 10.0.0.0 area 10

Network 209.76.25.0 area 10

Example D:

Router ospf 100

Network 10.0.0.0 0.0.0.255 area 10

Network 209.76.25.0 0.255.255.255 area 10

- Which two statements concerning source route bridging are true?

Source station first sends an address recognized broadcast to see which ring the dest. Resides on

To obtain routes from remote stations the source sends an all routes or a single route explorer frame

Destination end stations reverse the direction of the source frame that located it, sending it back along the same path

The source station always waits for all responses to select the path with the fewest bridges

- If you need to verify that a specific routed was selected as the designated router and what timer intervals were configured what show command could you use? (type it in)show ip ospf interface

- What two possible problems can occur when a large number of routers operating in a single ospf area?

Excess lsa traffic

Frequent table recalculation

Frequent adjancies recalculation

More Reachability errors

- What type of configuration is used when implementing a null interface?

### A static route

A access-list

A serial interface

A virtual terminal

- Which two of the following conditions require the use of BGP to connect to an ISP

Need to make routing decisions based on the source destination of internet traffic from within a remote AS

Identical connections have been setup to separate ISP’s for load balancing and redundancy

Security concerns require that all traffic from the internet is filtered except for three networks

The isp you connect to uses BGP

- Type the complete command to specify the encapsulation to carry traffic through and ip tunnel?

- What command to you use to determine which eigrp routers have adjacencies with each other

### Show ip eigrp neighbors

- Which cisco feature solves convergence metric reachability problems?

Compression over wans

### Route summararization

Incremental updates

Scalable protocol support

- You have decided to use private addresses, which statements are true regarding implementation considerations?

Routers that connect to external networks should be setup with the appropriate packet forwarding filters at the local end of link in order to prevent the leaking of private ip addresses

Routers that connect to external networks should be setup with the appropriate packet forwarding filters at both ends of the link.

You should filter any private address networks from inbound routing information in order to prevent ambiguous routing situations

You should filter any private address networks from outbound routing information in order to prevent ambiguous routing situations

### Answers are authenticated by book

- Router r1 uses a subnet mask of 255.255.255.0 and sits on a boundary of area 0 and area 1 based on the sample router configuration:

Router ospf 76

Network 145.12.32.0 0.0.15.255 area 1

Network 145.12.96.0 0.0.15.255 area 0

Area 0 range 145.12.96.0 255.255.224.0

Area 1 range 145.12.32.0 255.255.224.0

### An interface on this router with the address 145.12.32.124 is in area 1

All networks within the range 145.12.64.0 to 145.12.95.0 will be summarized from area 0 into area 1

### All networks within the range 145.12.32.0 to 145.12.63.0 will be summarized from area 1 into area 0

Area 0 can act as a stub or transit area for routes including networks in the range 145.12.96.0 to 255.255.224.0

The effect of the 4th line is reduce the number of route table entries

- Which three statements are true regarding the sample configuration:

Interface bbi 30

Ipaddress 133.44.8.0 255.255.255.0

Bridge irb

Bridge 30 route ip

This is an integrated routing and bridging configuration

### Line 1 creates a virtual routed interface to represent bridge group 30

Line 3 specifies that the irb protocol will be bridged

### Line 4 enables the bbi to accept and route ip packets for bridge group 30

### Line 2 specifies an ip address for a bridged interface

- What is the effect of the following priority configuration command?

Priority-list 4 default low

All protocols defined on the current interface will be set to low priority

Traffic to the default router on the remote network receives low priority queuing

Each queue defined in priority list 4 receives low (20) number of buffers by default

Packets meeting no other priority list conditions are assigned to queue which has low priority

- To minimize the amount of routing information an internal ospf router receives and manages what area type should you use?

External area

Backbone area

### Stub area

Totally stubby area

- How do you allocate more bandwidth to a protocols traffic using custom queuing

Use the set-queue-priority command

### Increase the byte count of a queue

Modify the round robin polling register

Assign the protocol to use a queue with a lower number

Assign the protocol to use a queue with a higher number

- Which statements best show how converstation is determined for queueing and priority purposes?

### Source destination MAC address

### Source destination network address

LLC header    information

### Frame dlci value

TCP sequence and acknowledgement    numbers

IP options and information

### Source destination port or socket numbers

Qos/Tos Values (not specified in the answers list)

- Type the command that will display priority queuing information

______show queueing or show queueing priority___

- Which four statements concerning bridge type and characteriscs are    true? (4)

Transparent bridging is define as the IEEE 802.1d standard

Translating bridges is necessary when the end nodes use different operating systems

Concuurent routing and bridging moves traffic in a device between routed interfaces or between bridge interfaces never a routed bridge

Source route bridging was development for use of token ring

Source route station placed a MAC address of default router in each    frame header

Source route translational    bridging commonly connects ethernet and token ring LANs

- Which two defaults are true regarding a variable length subnet mask?

### The vlsm allows more hierarhcial levels within an addressing plan

Companies are locked in implenting in a single subnet with a nic number in their entire network

### There is a greater capibilty to use route summarization

RIP1 network support    multiple subnet    networks per network address

- Mandatory commands to configure ospf are:

Router ospf process id network address route card area area-id

Router ospf network-address wildcard area area-id

Router ospf area-id network-address wildcard-address network area

Router ospf network address

Router ospf process id interface address wildcard-mask area area-id

- Which of the three following are scalable routing protocols? (3)

### Ospf

Appletalk

Ipx

### Nlsp

### Eigrp

Tcp/ip

- Which two addresses can be summarized into the address 152.25.16.0/20

152.25.16.0/24 , 152.25.17.0/24

152.25.31.0/24, 152.25.32.0/24

152.25.17.0/24, 152.25.31.0/24

- which two statements about the following configuration are true?

Source-bridge transparent 20 23 1 4

Virtual ring is defined as 20

Psuedo ring #23 establishes the transparent domain

The 1 will remove this bridge from the spanning tree root contention

4 is the speed of the interface

- which two of the following indicate a need for queueing (2)

congestion due to inadequate bandwidth

video conferencing

peak network traffic causes application errors

a lack of buffers is causing the router to drop packets

### Congestion occurs when bursty traffic exceeds wan link capacity

- You are asked to interconnect networks that can use different routing protocols, one network uses igrp one uses eigrp, you connect the networks using redistribution at the boundary routers. If the routers were to receive route information from the igrp and ospf networks, what is the protocol it would select the route from.

Igrp because its hybrid

### Igrp has a better administrative distance

Ospf because it has a better seed metric

Ospf because it is a link state protocol

- Where must the ip helper address statement be placed?

On all interfaces which are to receive local broadcasts

On the serial interfaces you wish to reduce broadcast traffic on

On interfaces that are to receive client broadcasts which need to be forwarded

On that support non routable protocols which need to be forwarded

- Which of the following will reduce the amount of routing information sent across a serial link (1)

### Configure static entries in routing table

Configure dial on demand routing

Prioritize traffic

Implement queuing on the router

- What is the effect of the following configuration commands?

Access-list 9 permit 220.88.99.0 line vty 0 4

access class 9 in

Only stations on network    220.88.99.0 can have access to the system console

### All stations on network 220.88.99.0 can establish sessions on virtual ports 0 through 4

All stations on network 220.88.99.0 will be allowed to establish virtual sessions on any port

No console access will be granted to any stations from network 220.88.99.0

- You would like enable IPX watchdog. Which command shows you how to do that?

Ipx spoof

Watchdog spoof

Ipx route cache

Route-cache

### Ipx watchdog-spoof

Ipx spoof

- Which statement is true for non-routable protocols such as Netbios, LAT, and MOP

No datalink address

Not associated with any media type

### Cannot be routed because they have no network layer address

### They are never used over wan links

- Which of the following will configure static route to the network 209.76.25.0 out interface ethernet 0?

#### Ip route 209.76.25.0 255.255.255.0 ethernet0 10

Route ip 209.76.25.0 netmask 255.255.255.0

Ip route 209.76.25.0 255.255.255.0 interface ethernet0 10

- What is the command to show all access lists?: _____show access-list____

- Enable SPX spoofing

ipx spx-spoof

- What command displays priority queueing?: __________

# Show priority-queuing, or show queueing, or show queueing priority

- Which queue method provides automated allocation to all net traffic based on messages and conversations: custom queueing, FIFO, priority queuing,

weighted fair queuing

- At what level of the network heirachy are reliability and availability most important?

Core

Distribution

Access

Campus

- Order of transmission in priority queuing

Higher priority queues are empited before lower priority queues

All queues use a round robin method to effect

Time-division sequenced

- There was a long question about VTP

- There was a long question about queuing methods?

Allocated based on messages and conditions was one of the choices

- There was a long question about FDDI port parameters? The choices were Menu, Status, Options

- There was a long question about Frame Tagging

- There was a long question about ATM Addresses? The choices were ICD, POP, NCP, DCC, NFP

- There was a long question about Cut-through techniques? The choices were: store & forward, fast forward, store & free, fast frag & free.