# Chillnz Cram Overview

# OSI Layers: It allows changes in one layer to occur without changing other layers, It clarifies what general function is to be done rather than how to do it.

Application –file, print, message, database, application svcs. identifying communication partners, determining resource availability, and synchng sending & receiving applictns, Eg: SMB, NCP, Telnet, FTP, WWW and SMTP, File Transfer Access and Mangt (FTAM), Virtual Term Proto (VTP), and Common Mangt Info Proto (CMIP).

Presentation-Negotiating data xfer Syntax, Data encryption, compression, translation, coding and conversion functions. Eg. ASCII, EBCDIC, JPEG, GIF, TIFF, MPEG, PICT, MIDI, QuickTime, 
Session–Dialog Control-establishes, manages, maintains, and terminates sessions between applications, communication sessions consist of service requests and service responses that occur between applications located in different network devices. Eg. NFS (sun), SQL, RPC, X window, ASP, Zone Info Proto (ZIP), and Session Control Proto (SCP).
Transport-End-End conn, hides info from upper layer, gives transparent data transfer, segments and reassembles data into data streams. responsible for both reliable and unreliable end-to-end logical conn. does flow control, windowing, acknowledgement, multiplexing, virtual circuit management, session establishment and error checking and recovery.

Eg. TCP, UDP, SPX, Name Binding Protocol (NBP), Nwlink, NetBeui, ATP, NBP. 
Network-Routing-uses logical addressing to provide routing and related functions that allow multiple data links to be combined into an internetwork. supports both connection-oriented and connectionless service from higher-layer protocols. protocols are typically routing protocols. BGP, OSPF, DLC, DecNet, ICMP,    RIP, NLSP,    IGRP, EIGRP, Packets and datagrams are sent across this layer.

Data Link-Framing-putting 1’s & 0’s in to a logical group, provides reliable transmission of data across a physical medium. specifies different network and protocol characteristics, including physical addressing, network topology, error notification, sequencing of frames, and flow control. two sublayers known as the Media Access Control (MAC) and the Logical Link Control (LLC) layer. LLC- Provides flexibility to upper & lower protos. Respon timing, flow control, supports both conn less &    conn oriented svcs used by upper protocols. MAC-respon for framing, access to physical net media.Eg. ARP.    DLL implementations are LAN or WAN specs. LAN imples- Ethernet/IEEE 802.3, Fast Ethernet, FDDI, and Token Ring/IEEE 802.5,WAN imples.-Frame Relay, LAPB, SDLC, HDLC, X.25, SLIP, PPP.

MAC Add. : 6 bytes ( 3 bytes- vendor code, 3 bytes- Serial No.)

Ethernet_II -PreAmb8-DA6-SA6-Type2(upper layer proto ID)-Data-FCS 4 (Cyclic Redundancy Checksum).

802.2_Ethernet-PreAmb8-DA6-SA6-Length-DSAP-SSAP(802.2 LLC header)-Data-FCS.(MAC & LLC)

802.3_Ethernet-PreAmb8-DA6-SA6-Length-Data-FCS (MAC) (CSMA/CD-IEEE-802.3) can’t identify up layer protos.

Physical-defines Bit synch & Physical topology, Specs define such as voltage levels, timing of voltage changes, physical data rates, max    trans distances, and the physical conntrs be used. Eg. FDDI, SONET, ISDN. (Stds: v.24, v.35, x..21, G.703 ),    LAN-imples-Ethernet/IEEE 802.3, Fast Ethernet, FDDI, and Token Ring/IEEE 802.5.

WAN-imples- High-Speed Serial Interface (HSSI),    SMDS Interface Protocol (SIP), and X.21bis.

10Base2-ThinNet-RG58-BNC-185 Mtrs,                        100BaseT4-100MB,Cat 3/4/5, 802.3

10Base5-ThickNet-RG8-DIX/AUI-500Mts,              100 BaseTx-Cat5-STP-100 Mtrs.(Fast Enet over STP/802.3),

10BaseT-Cat3/4/5-100Mts.                                                                    100Base Fx-100MB,2 pr OFC 802.3

10/100Base FL-OFC-2 Km,                                                                  100Base VG Any LAN-OFC, 802.3 (Cisco does not support it)

100BaseT-    CSMA/CD, 802.3u,    Fast Enet,

DOD Model : Proc / Appl : Telnet, FTP, LPD SNMP/ FTP, SMTP, NFS, X window,    Host-Host : TCP,UDP,

Internet : ICMP, BootP, ARP, RARP/IP,    Net Acc : Ether Net, Fast Enet, Token Ring, FDDI.

Tcp/udp: port Nos. 0-255:used for public appl, 255-1023: for comp to use their appl; >1023 for upper layers sessions with other Hosts. Tcp :    virtual ckt, conn oriented, sequenced, ackd, reliable, high o/h,    Udp:    unsequenced, unreliable, connless, low o/h. ftp>tcp21, telnet>tcp23, smtp>tcp25, tftp>udp69,    snmp>udp161, tcp>ip6, udp>ip17

Data Flow :  User info is converted to Data(7,6,5), Data    converted to Segments(4),    Segments converted    to    Packets or datagrams(3), Packets and datagrams are converted to frames(2),    Frames are converted    to    bits(1).

ICMP Inet Contr Mess Proto: Alerts routers if : Destn unreach, Buffer full, Hops-demise, ping uses ICMP+IP,

sends    redirects    back    to    origi    router.      SNMP: Simple Net Mngmt Proto- used to manage & Monitor Net,

RARP server- gives IP add against MAC add to diskless comp, BootP- gives IP add & boots diskless comp/w stns.

ARP- layer3 proto, Source & Destn H/w add keeps changing, but IP add remains same as pkt travels thro’ routers.

LAN Segmentation: breaking up the collision domains by decreasing the number of workstations per segment.

Full Duplex Ethernet : needs full duplex NIC cards, Loop back and collision disabled, 2 x 10/100 MB paths, software driver supporting two simultaneous data paths, Adherence to Enet Stds.

Fast Ethernet (100MB)- provides 10    times the bandwidth of older 10bastT Ethernet. Must have Cat5 cable, no longer than 100 meters, and Fast Ethernet NIC’s and Hubs/Switches.

Gateway–used for comm between diff    NOS Eg. NT & IBM SNA,takes the pkt strips off the old protocol and repackages for the receiving network- proto translation (Trans, sessio, prestn, appli).

Router-Filters by both HW add & IP add. strip off data link sourc & dest add & recreates it. Provides flow control, manageability, Explicit pkt life time control, more functionality, multi paths, segments net, links unlike phys media, stops broadcasts, no un routable Net protos ,    Brouter- will act as router and bridge based on protocols. (DLL, Network).

Bridge-Segs net, filters traff by MAC add, regenerates signal at pkt level, reliability, availability, scalability, manageability, latency, pass broadcasts, no unlike med (DLL,mac sub). Switch-Duplex, filters trafiic on MAC add, pass broadcasts, (DLL) Repeater- regenerates signals for tx, pass broadcasts, no unlike phys media. (Physical).

Hubs- one tx, all other rx, simplex, no addressing(Physical).Multiplexer- for multiplexing links.(Physical)

Repeater reduces dist for 100BaseFx due to delays, but extends 100BaseTx dist. Class1:Translational rptr supports both 100baseX and 100baset4 signalling. Delay=140 bit times, Class 2:Transparent rptr only supports either 100baseX or 100baseT4, not both at same time.delay=92 bit times. FastHub300: Rptr compatible with 802.3u for fast Enet.

LAN Switching :    Uses dest MAC add to forwd pkts. same as multi port Bridge. Multi simultaneous convs, high speed data exges, low latency & high frame fowd rates, full duplex, media rate adaption (10/100). Port Switching: port assigned to a phys net by software. Frame Switching: increases b/w on the net. Allows multi trans in parallel. used in Cisco Cata switch. Cell Switching: ATM. Uses small fixed length cells that are switched on the net. Used in Cisco light stream switches.

Router Switching : Store & Forward: copies entire frame in to buffers and checks CRC. Forwards 64-1518 Bytes only. latency high depends on    Frame Length. used in cat 05000/5505 series. Cut-through (Real Time): copies only dest add (6B) in to buffers. some switches automly uses both modes, Low latency, 10/100mb full duplex    Fragment Free: modified cut thro. Waits for collision window (first 64 B). Better error checking,    no latency.

Spanning-Tree Protocol (STP) IEEE 802.1d- to prevent routing loops. STA (Spanning-Tree Algorithm) is impled by STP to calculate a loop-free net topology. In Cat 5000 based nets Bridge proto data units (BPDU) are sent & recd by all switch in regular intervals & processed to determine spanning-tree    topology.

Virtual LAN’s-have diff ports of a switch be parts of diff    sub nets.    can be by location, function, grp & dept. Simplify moves, adds, changes. less admn costs, better control of broadcasts, tighten net security, distribute load, Relocate server into secured locations. Frame tagging: A user defined ID for each frame,    places a unique user defined ID in the header of each frame used within switch fabric itself. Used in Cat 3000 & 5000 series. Developed    for multi    VLAN, Inter switch Commn, needs processor & admn    load.

IP Address Classes:

(2)192-2-4194302-16382-62          Class A    Net.Node.Node.Node    0      1-127              127 net        16777214 nodes

(3)224-6-2097150-8190-30              Class B    Net.Net.Node.Node      10      128-191      16384 net            65534 nodes

(4)240-14-1048574-4094-14          Class C    Net.Net.Net.Node        110      192-223      2097152 net            254 nodes

(5)248-30-524286-2046-6                    (net/node all 0-this net/node, all 1-all net/node) (127-loopback add )

(6)252-62-262142-1022-2                    (entire IP all 0-cisco default route, all 1-broadcast to all nodes)

(7)254-126-131070-510-na                Max # of Subnets: 2(masked bits)-2, Max # of Hosts / subnet : 2(unmasked bits)-2

(8)255-254-65534-254-na                    Class D-224-239-Multicasts, Class E- 240-255-Future.

##### Directed / only subnet broadcast add- 172.16.3.255,    Flooding / total Network broadcast add- 172.16.255.255

##### Cisco Routers :

2500 : Cons port- async serial port RJ45 60 Pins, Aux port- Modem RJ45,Telnet port - Ethernet 0, (S0,S1-serial port),    External ports - TFTP server, Virtual Term, Net Mngmt Stns. ROM-holds Bootstrap Startup programs, OS software, POST. Flash-EEPROM-holds cisco IOS image and Microcode,    RAM-holds running OS, running config, provides caching & pkt filtering. NVRAM-Stores startup config,              config mem: executes NVRAM commands, copies start to Run,

config net [ip add of    tftp serv]: load config info from TFTP server, Config t: console / telnet (config from my term)

Catalyst 5000 : to config port, type slot/port (e 4/2 for Ethernet, card 4, port 2), Catalyst 7000 / 7500 VIP (versatile int    processor    cards): type slot/port adapter/port; e 2/0/1,    3 rd    Enet vip card3, first port 0, 2 nd    Enet Int.

Auto Install : config router autoly and remotely. New router acts as BootP work stn & gets IP from existing router acts as BootP or RARP server, connects to TFTP server, gets network-config, hostname-config/router-config files.

BootP server: provides (by TFTP) stn.IP add, server IP add and the name of the file to be loaded and booted.

Modes: User EXEC : After log in, you are automly in this mode, basic tests, temp term setup, conn to remote devices (router>), logout to exit, Privileged EXEC: Set OS params, config, debug,(router#) Quit/Exit to exec mode, ROM monitor: Occurs when no cisco IOS image found / boot interrupt (rommon>), Set up: occurs on first switch on, RxBoot : reboot from ext (router<boot>), Global config Mode: router(config)> commands affects whole system, exit/end to pre mode

Interface Config mode: router(config-if)#    comm for config single int.

Editing: Ctrl+A-beging of    line, Ctrl+E-end of line, Ctrl+F(R arrow)-move fwd 1 char, Ctrl+B(L arrow)- back 1 char, Ctrl+P(U arow)-scroll/Rpt prev comm, Ctrl+N(D arow)-recent comm recall, Esc+F-move fwd 1 word, Esc+B-back 1 word.

###### R>terminal editing / terminal no editing : advanced editing features on / off.    R> enable / disable :    privileged mode on / off

###### R>sh history-sh comm buffer, R>term history size- set comm buffer size, Tab: Completes comm entry,

###### Passwords: config t,    Enable secret x,          enable password x,        line vty 0, login, pw x;                    line aux 0, login, pw x;                                  line con 0, login, pw x;          Banner: config t, banner motd #, xxx # ;      Hostname: config t, hostname    [newhostname]

Shutting down : config t,    # int s0, # shutdown (Admntly    down)            # no shut down (Turn on)

DTE/DCE    Cable: router use synch comm, needs clk, clock    rate on DCE.+ specify bandwidth (for IGRP), Config t, int s0, clock    rate    56000, bandwidth    56. def clk rate-T1, R# show controllers    s0- Identifies serial port cable as DTE or DCE cable.

###### CSU/DSU : Chl svc unit/Data svc unit. Handles clocking on serial ports (WAN links), not used by Cisco router.

Token Ring Speed : # int token ring 2/0,    # early token release, # ring-speed 16 (4-16 mb),

Media type : # int e1,    # media type 10baseT, Clock set: R# clock    set 11: 02: 01    04    August 1999

Config Comm: RouterR#      sh    startup-config/config: contents of    NVRAM,    sh    running-config: contents of    RAM,

R# erase startup-config (write erase)- erases NVRAM, starts initial config dialog, R# Setup: starts initial config dialog,

R# write term-sh run config, write mem- copy run    start, config mem- copy start run, config net [ip ad of tftp ser]-lod config frm tftp, write network/R# copy run tftp (backup, ram>tftp serv), copy tftp run (restore), R# reload: lod startup confi to mem,

IOS Comm: R # copy flash tftp (ios backUp), # copy tftp flash (ios restore),  Fallback-gives    alternative IOS loading booting sources: config t, # boot system flash [ios_filename],    boot system TFTP [ios_filename] [ TFTP_ip add],    boot system    ROM

R# erase- erases flash, # delete- (if multi flash), if ext flash card # delete-only marks the IOS, # squeeze-deletes IOS.

R# sh flash-flash mem stat, all IOSs file name, # sh version-sh current IOS file (if multi flash), hw config, sw ver, Register setting, boot images, sh process: active process, cpu utilzn,    sh mem: mem allctn stat,    sh stacks- sh reason for crash/reboot,

R# sh buffer- sh buffer stat,    sh int- sh all int config,      sh proto: sh all layer3 proto config,      sh ip proto- IP proto in detail

Cisco Discovery Protocol (CDP): allows access config info on other routers with single command, uses SNAP broadcasts at DLLayer. CDP starts by default (ver10.3/later) & discovers neighboring routers running CDP thro data link broad. updates: 60 sec, holdtime: 180 sec, # config t, # cdp timer 60, # cdp holdtime 180    (global), R# sh cdp    int: int stat, config info, encaps, cdp timers. R# sh cdp entry */ routerA: * shows info recd from all routers / only router A. R# sh cdp neighbors: shows cdp updates (device ID, local int/port, holdtime, capability, platform5000, neibhor port ID) (capa- R router, T trans bridge, B source route bridge, S switch, H host, I igmp),    R# sh cdp neighbors detail: shows cdp enrty+ sh cdp neighbors

R# sh cdp traffic: reveals amount of pkts sent & recd among neighbors;        config t, # no cdp run (cdp off-all int-router),

# config int, # no cdp enable (cdp off- single int)

Telnet : Virtual Term conns: part of    tcp/ip proto allows    to connect to remote hosts and routers. Cisco supports 5 simultaneous (vty 0 4) telnet sessions. R# telnet [ip add], ip add of any active int on the router. R# exit: to close session.

Ctrl+shift+6 X : return to org router w/o closing telnet. # 1, return: takes back to telnet session

R# sh sessions : sh all telnet sessions/conn no, host, add, byte, idle, conn name.

Hostnames: router can resolve ip add to hostnames. Config t, # ip host    routerC    23    172.16.40.2 (23 telnet port- optl),

R # sh host / s: sh all> hosts, flags (perm/temp, ok/ex), age (hrs), type (proto), ip add, t# no ip domain-lookup: disables router    DNS lookup (enabled by default), to re-enable DNS lookup, t# ip domain-lookup, t# ip name-server 172.16.30.8 (max 6 add)

IP Add config: config t, # int s0, # IP address [ ip-add] [ subnet-mask] - config IP add, Subnet mask for serial int s0.

# sh interfaces-shows all interface types & their associtd IP adds. use CDP command to see IP add of neighbor routers.

Inter Network Testing : R# telnet : shows all layers are working, verify appl layer commn between    the    hosts.

R# Ping 172.16.10.1- uses IP & ICMP proto to verify commn at Inet layer between hosts (! echo replay recd, . time out,

U dest    un reachable, C congested exp pkt, |    ping interupted, ? pkt type unknown, & pkt TTL exceeded.)

##### R# trace ip 172.16.40.2 uses TTL values (!H-recd probe not forward due to access list, P proto un reach, N net unreach,

##### U port unreach,* time out, (trace used in IP, AppleTalk, CLNS, Vines, Old Vines), R# sh int so : shows line & data link

##### proto    status, R# debug all, # undebug all,

##### IP Routing : Source and Destn. Hardware add keeps changing, but IP add remains same as    pkt    travels thro’ routers.

##### Dest IP remains same, frame keeps changing at each hope b’cos source, destn h/w add    keeps changing.

##### Static Routing config t, # ip route [rem net ID] [subnetmask] [def Gway,next hop add / interface][distance]- cisco book

##### # no ip route [remote net ID]- to cancel a route, # sh ip route-sh R table/all routes, # sh ip route static-sh only static routes,

# ip classless    global command to use def routing with static routing to forward pkts destd to obscure subnets to best route. Default Routing : default route = router or gateway of last resort, # ip route [0.0.0.0] [0.0.0.0]    [gateway IP/next hop add] # ip default-network 192.68.17.0 : Default route as per Cisco book,    # no ip routing (disable), # ip routing (re enable)

###### Dynamic Routing : Autonomous System(AS): consists of    routers that share info using same protocol. Backbone + areas. Routing within area/domain-Intra Domain Routing, Ext / Inter domain / inter AS Routers    connect areas to backbone.

Codes- # sh ip route- C-dir conntd, S-static, I-IGRP, R-RIP, M-mobile, B-BGP, D-EIGRP, EX-EIGRP ext,

O-OSPF, IA-OSPF inter area, E1- OSPF ext type1, E2-OSPF ext type2, E-EGP, I-ISIS, L1-ISIS lvl 1, L2-ISIS lvl 2.

Admn distance: Source’s trustworthiness 0-255 : Directly connected    Interface-0, Static route-1, EIGRP sum route-5, Ext BGP-20, Int EIGRP-90, IGRP-100, OSPF-110, ISIS-115, RIP-120, EGP-140, Int BGP-200, Unknown-255.

Interior Routing protos:used within an AS. RIP, IGRP (Dist vect), OSPF (Link state), EIGRP (Bal Hybrid) all layer3.

Distance Vector-RIP-uses 2nd hand info, metric (hops 16 ),    slow conv. @    30 sec, routing loops, counting to infinity.

To Stop Loops: Counting to infinity/exceeding TTL: max.16 loops, Split Horizon: stops info return, Route Poisoning: uses net unreachable entry (metric for down link = infinity), and triggered updates. Hold Downs : stops regular updates from reinstating down link, waits & watches status, work with route poisoning, and use triggered updates.

- Link State- OSPF- uses LSP (hello pkts), topology Dbase, SPF algorithm, metric-65533, routing table. needs more power, memory, b/w, fast conv @ 2-12 hrs(uses DR designated Router), LSP time stamps,

Balanced Hybrid-IS-IS, EIGRP: combines and uses best of both above. Link state triggers. Classfull routing: summarizing routing info by major network nos.(RIP, IGRP), Classless and Prefix routing : allows contiguous blocks of hosts, subnets or nets    to be represented by a single route. Cost= Path Cost: based on hope, bw, etc. to determine the best route. Lower the cost value better the path.    Routing metric: method by which routing algorithm decides one route is better than other. Stored in routing table. Includes bw, comm cost, delay, hop count, load, MTU, path cost, and reliability.

##### RIP: dynamic, dist vect, classful routing proto, no support prefix routing. Provides IP dest add, metric-hop count (16), IP add of next router in the way, marker & flags (recent route change), timer, Hold downs, Split horizon, Poison rev, slow conv, loops. Timers: update-30sec, invalid-180sec, flush-240sec., hold-180sec, Admn Dist.: 120,

RIP pkt format: command1,ver no1, zero2, add family ID2, zero2, add4, zero4, zero4, metric4.

Enab RIP: cancel all stat/def routes, config t, Enab RIP(# router rip),Enab net (# net work    x.x.0.0),    R# wr mem

Testing RIP: # sh ip route rip, # ping [add]. Passive RIP: stops    router from advt to internet. Commands:    config t,

# router    rip,    # network 172.16.0.0,    # passive serial    1 ( serial 1 connects    the local net to the internet),

Neighbor RIP: WAN link (frame relay) doesn’t broadcast RIP update by default. use neighbor command    to solve this.

config t, # router rip, # network 172.16.0.0, # neighbor 172.18.3.10.      RIP Broadcast : sending update to 255.255.255.255 via e0

Monitoring RIP: R # sh ip route rip-sh only RIP routes, # sh    ip    protocol-sh RIP timers, net ID, routing info sources,

# sh ip interface: shows interface config and statistics, R# debug ip rip: turns on RIP debugging-shows RIP updates live,

# undebug    ip    rip: stops debug, R # trace    172.16.50.2: sh    path to dest    add, config t # no router rip: stops running RIP proto.

##### IGRP: Dist vect, cisco prop, scalability, fast convergence, multi paths (def 4, max 6) & unequal load balancing, Periodic & Flash updates, no VLSM, complex metric (1-255) [delay-media speed (10mb=100=1ms), bw (1.2kb-10Gb), reliability (1-255optm), load (optm 0-255), MTU], hold down, poison rev, split horizon,

##### Timers: update-90s, invalid-270s, holddown-270+10s, Flush-630s, Admn Dist:100

IGRP routes : Interior: routes between subnets, System: routes within AS, Exterior: routes out side AS (1-65655).

##### Enable IGRP: Enable IGRP with AS no. (config t,    # router    igrp 10,      # network    172.16.0.0, #    wr mem )

Monitoring IGRP: R# sh ip route igrp : sh like RIP+ delay, bw, reliabilty, load, R# sh ip route igrp-shows IGRP routes,

R # sh ip protocol-sh all timers, broad freq, etc,    R# sh ip int-shows int config, status & global parameters,

R# un / debug ip igrp events / transactions : sh    IGRP traffic live, config t # no route igrp-stops IGRP.

##### OSPF: Link state proto, used within AS, a grp of routers that share a routing proto. Each router retains own dbase.

Huge metric limit 65533, assign cost to a link, load balancing among equal cost links, update-only for a change, pkts carry info for multi routers (tell a friend), can communicate out side AS, supports VLSM, TOS (types of    svce    routing)

EIGRP: Cisco prop, Balanced Hybrid Proto (best path by dist vect, update by link state), fast conv, support VLSM, partial updates, multi proto support (IP/IPX/Apple talk), update-only for change only for neighbors.

Exterior Routing proto : used outside an AS. More complex, stores more info about more    routers.

EGP (Exterior g/way proto): dist vector proto, Dynamic, doesn’t use metrics like interior routing proto, can’t detect or correct routing loops, communicates thro’ a core backbone    routing net that connects AS together, establishes a set of neighbors to share routing info,    send polls to neighbors to check ‘alive’, updates other routers within an As.

Message types: neighbor acquisition message to check ‘alive’/ not, neighbor reachability message to check down/not, Poll messages to collect info about remote hosts’ network, Error message to identify various errors.

BGP (Border g/way proto): An inter AS routing proto used on internet, used between &    within AS, Determines loops, two BGP neighbors must be on same physical net to communicate, doesn’t use a core, all areas can comm    w/o going thro’ a core backbone, incremental updates for optimal routes only, metric with degree of preference.

Message Types: Open-conn established, Notification: error det, Keepalive: keeps the hold timer alive,

Novell IPX proto stack : ( SPX Sequence pkt exch,    IP Internetwork pkt exch)

7/ 6/ 5-SAP, NCP; 4-SPX; 3-IPX, RIP, NLSP; 2+1-Thro Open Data Link ODL Interface Novell supports all media types.

IPX: connless proto, uses sockets, controls assignment of address ( software addressing), governs pkt delivery, makes routing decisions based on info from RIP or NLSP proto, SPX: adds conn oriented comms to the connless IPX. Works by creating virtual ckts or connections between machines with each conn having a conn ID included in the IPX header.

Novell    RIP: dist vect routing proto, uses ticks (1/18 sec) & hop count (max 15) as metrics, split horizon, Aging timer,

RIP Broadcast @ 60 sec,    SAP: svce advt proto, servers use it to advt svcs @ 60 sec, nodes use it to locate svcs,

Netware server: SAP(svcs table)& RIP (routing table) info xge between servers by broadcast @ 60 sec,

NLSP: Netware Link svce proto, advanced link state routing proto, might replace RIP & SAP. NCP: Netware core proto, provides file access, printing, synch, & security. GNS Request : Loc server, Cisco router + Rem    server (book)

IPX Addressing: use 80 bits (10 bytes), first 4 B-Net add; 6 B-Node add & is assigned autoly, mostly MAC add (3B vendor code +3B sl no- so no need ARP). Since hw MAC add is included as part of software add IPX , no need of ARP eqvlnt.

Eg. 00007C80.0000.8609.33E9 is an IPX add in Hex. 20 Hex dig=10 B.    In dual proto nets TCP/IP add is converted to Hex (4B) and used as IPX net add for uniqueness. Find IPX add: ask admn, use CDP, telnet to neighbor router,

Cisco Router: build SAP tables, respond to GNS req in locating svcs like Netware servers, saves WAN link b/w by doing this incase of remote servers, by def    can’t pass SAP broadcast (can forwd if progmd), own SAP broadcast @ 60 sec,

Enable IPX routing: # config t, # ipx routing [node], (node-optional-mac add of int by def, must if    R has only serial ports)

# ipx maximum paths 2 (max 512) forwards IPX pkts to mult paths. cisco does load sharing by def over parallel paths.

# ipx delay [6/1] =6 ticks, 1 hop. Tick metric- Cisco default for LAN int- 1 tick, WAN int- 6 ticks

IPX on each sub/interface for multi frames: # ipx network [number] [encapsulation encap-type] [secondary]:uses def    if [encap+sec-optional] not given, # config t, # ipx routing,    # int e0.1,    # ipx    network    3200    encap    arpa      sec,

Encapsulation / Frame types: Netware supports multi incompatible frames on the same media, All of them works in one media if Cisco router supports all frames. to use multi encap use subinterfaces, seconary nets. 4 types: Enet_802.3: def till Nw 3.11, Enet_802.2:def since Nw 3.12, Enet_II: supports tcp/ip & ipx, Enet_snap: supports Appletalk, ipx, tcp/ip.

Encapsulation :      Ethet, Token Ring, FDDI, (Interface    type, Novell Frame type, Cisco IOS key word)

Ethernet :              IPX Enet_802.3: novell-ether (default),    IPX Enet_802.2: sap,        IPX Enet_II: arpa,          IPX Enet_snap: snap

Token Ring : Token-Ring: sap (default),                                            Token-Ring_snap: snap,

FDDI :                      fddi_snap:      snap (default),                                          fddi_802.2: sap                            IPX Fddi_raw: novell-fddi

Serial :                      HDLC (default)

arpa : IPX    Enet_II,                    hdlc : HDLC on ser links,                novel-ether : IPX Enet_802.3,        novell_fddi : IPX    Fddi_raw

sap : IEEE 802.2 on Enet, FDDI, Token    ring,                                    snap : IEEE 802.2 SNAP on Enet, FDDI, Token    ring

Sub interfaces: are virtual interfaces on a Cisco router, A new way to run secondary    ip, ipx adds on the same interface.

To define sub Int use    # Interface    ethernet    slot / port.    number, (number =e0.0-e0.4292967295),    #config    t

# int    e0.100,    # ipx    network    2300    encap    sap

Secondary address: To add multi frame types and IPX add on the same int. (No cisco support, cisco supports only sub interfaces.    # config t, # int s0, # ipx    network    2200    encap    sap,        # ipx    network    3200    encap    hdlc    sec

Monitoring IPX on Cisco routers:.

R# sh    ipx    route : sh IPX routing table entries. C-conntd prim net, c-conntd sec net, S-static, F-floating static, L-local (int), W-ipx wan, R-rip, E-eigrp, N-nlsp, X-Extl., A-Aggregate, s-sec,

R# sh    ipx    servers: sh all net servers, SAP table with all SAP svcs, P-periodic,I- incremental, H-Hold down, S-static

R# sh    ipx    traffic: sh summary of    No & type of both IPX    RIP & SAP    update pkts recd & sent @ 60 sec by the router.

R# sh    ipx    interface: sh status of    IPX int & IPX paramtrs set on each interface. sh sap,rip info(book-ans)

R# debug    ipx    routing activity / events: shows IPX RIP & SAP update pkts recd & sent, live IPX as its running thro    network

R#    undebug    ipx    routing    act,    R# ping    ipx    5200.0000.0c3f.1d86- test ipx congif, R# debug ipx sap : shows IPX    SAP pkts sent & recd @ 60 sec. SAP    0x1/0x2 gen qry / resp, 0x3/0x4 GNS reqst / resp.

Extended Ping: R# ping, ipx, 5200.0000.oc3f.1d86.              R # sh    proto e0-      also shows  IPX add    of    an int.

Monitor neighbor router: R# sh cdp entry routerX; R# sh cdp neighbor detail; R# telnet routerX / ipadd, R# sh ipx add

Access Lists : A list of conditions that control access, filters un wanted pkts. IP + IPX works similar. applied inbound or outbound traffic. Rules: compared in sequential order, compared until first match, implicit deny if there is no match.

IP Std 1-99, IP Ext 100-199, IPX Std 800-899, IPX Ext 900-999, IPX SAP 1000-1099, DECnet- 3xx, AppleTalk-    6xx

Cisco IOS ver 11.2 or later allows to use Access list name rather than Number.

STD IP access list: acts up on source IP add. # config t, syntax  # access-list [number] [permit / deny] [source add]

R# config t, t# int e0, if# ip access-group 10    out,    only 1 Access list each on    i/c & o/g is possible for each protocol.

Wild cad masking: 0-check, 255-ignore, # config t, # access-list 11 permit [172.16.50.2    0.0.0.0 / 172.16.30.0    0.0.0.255]

Eg:    [172.30.16 (00010000).0] [0.0.15 (00001111).255]- permits sub nets from 16 (00010000) to 31 (00011111)

Extended IP access list: acts up on source add, dest add, IP proto, Port , number=100-199,    # config t,    Syntax

# access-list [number] [permit / deny] [proto] [source add] [detn add] [port],

# access-list    110    permit    tcp      host    172.16.50.2      host    172.16.10.2    eq    8080, # int e0, # ip access    group 110    out

# access- list 110    permit    tcp      any    any    eq    www, host 172.16.50.2 = 172.16.50.2    0.0.0.0, any = 0.0.0.0    255.255.255.255

Port nos.-bgp179, ftp21, ftpdata20, gopher70, lpd515, smtp25, telnet23, www80, tftp69, Dns53.

Monitoring Ext IP access list 3 ways: R# sh    ip    interface,      R# sh    acces-lists,    R# sh    run

#    sh access-lists: sh all running access lists, no of hits/matches for each line (34matches)

o/p:    Ext IP access list 110,    permit    tcp    host    x.x.x.x    host    x.x.x.x    eq    8080    ( 15 matches)

R# sh ip access-list / xxx : shows all / xxx    IP access lists. R# clear access list counters : clears no of matches counters

R# sh ip int    eo    : shows int config,< which ports have IP access lists applied>    R# sh running config : shows full config

Logging : config t,    # access-list 110    deny    ip    any    any    log- can see no of matches on the denied line eg. (4 matches)

By default will log to the console. Log contains:    acc list no, source add, source port, dest add, dest port, no of pkts.

All the log info can be redirected    to a syslog server and stored for security purpose.

Placing Access List :    STD    Access list -    Close to the    Destination,    Extended Access list- Close    to    the    source

Efficient Access list: Most commonly matched lines in the access list should be on top, this lets most pkts to match fast than going thro all the lines in the list causing a delay in traffic.

STD IPX access lists : based on source and dest add, config t, # access-list [number] [permit / deny] [source] [dest]

t# access-list 810 permit 30    10,    If# ipx access-group 810 out - permits traffic from net 30 to net 10

t# access-list 811 permit -1    -1,    -1 = any host, any net ID,

Extended IPX access list:    based on source net/node, dest net/node, IPX    proto (SAP, SPX etc.), IPX socket

# access-list [number] [permit/deny] [IPX protocol] [source] [socket] [dest] [socket] , # config t

# access-list    910    deny -1    50    0    30    0,          # int e0,      If # ipx    access-group    910    out

Logging: logs source add, source socket, dest add, dest socket, proto type

Monitoring:    # sh access-lists (sh list contents),      # sh    ipx    int (sh total int config),      # sh run    (sh total router config)

IPX SAP Filters : control access to IPX devices by controlling SAP, Cisco doesn’t forw SAP broad, save WAN link, Builds own SAP table, advt @ 60 sec. SAP svcs 4 Nw file server, 7 Print server, 24 Rem bridge server (router)

# config t, # access-list [number] [ permit / deny] [source] [svce type, 0 means all svcs],

# access-list    1010    deny    3d    7, # access-list 1001 permit -1, # int    e0, (# ipx net 9e), If # ipx    input-sap-filter 1001 (-1 all nets)

Input    SAP    filter : No. of    svcs entered in to the SAP Table is reduced, contains only a subset of    all svcs for propagation.

Output    SAP    filter : No. of    svcs propagated from the table are reduced, only a portion of the table is propagated.

Placement : SAP filters must be placed close to    the    source to conserve critical bandwidth

MISC : Interface Status: Operational : serial1 is up, line protocol is up, Conn    problem: serial1 is up, line protocol s down,

Int    problem: Serial1 is down, Line protocol is down, Disabled : Serial1 is administratively down, line protocol is down

Conn Oriented- vir ckt, conn establmnt, term, sequenced, delivery ack, error recovery, sliding window flow control,

Conn Less- no VC, no sequence, no delivery guarantee, higher layer respon for error recovery, flow control, reliability.

Flow control: no data lost, simple hand shake, sliding window, ack with source,

AppleTalk config: # config t, # appletalk routing, # int e0, # appletalk cable-range 100-110, # appletalk    zone    bldgA

WAN Services : Switched/Delayed svcs:X.25, ISDN, Fr Relay, SDLC: P-P, P-multi P links, HDLC: Peer devices DDR.

POTS-most commonly used WAN service: DTE, DCE, demarc, CPE    : Demarcation point: at which MOC cable ends (DP), CPE: Customer premise eqpt, Local loop: MOC cable path, CO (Central off): high speed trunk line area,

SDLC (Sync Data Link Control) : efficient, flexible & high speed, p to p & multi p links, un / bounded media, Half / Full Duplex, ckt & pkt switched nets, IBM uses in SNA (System Net Architecture) proto, Bit sync Data Link Layer proto, used in Cisco serial int to conn remote offices,    supports two net node types:

Primary stns: control opn of other stns, poll secondaries, and setup/tear down/manage links.

Secondary stns : are controlled by primary stn, secondary    can only send data to primary only after getting permission.

HDLC (High Level Data Link Control) : Bit oriented, link layer proto, specifies encapsulation for data on SDLC links.

(LAP & LAPB: are modified form of    hdlc).                          R# sh int s0: shows hdlc config

Cisco HDLC: Cisco prop, doesn’t    commte with other HDLC’s, default encapsulation in Cisco over sync serial links

Transfer Modes: used to define media access on WAN. HDLC supports following transfer modes

Normal Response Mode(NRM): Implemented in SDLC, Sec can’t commte with Prim unless Prim asks it to do so.

Asynch Response Mode (ARM): Allows Sec to commte with Prim w/o it’s permissioin.

Asynch Balanced Mode (ABM): Combined nodes (one acts as Prim / Sec) commtes each other w/o perm.

Link Access Procedure Balanced (LAPB): Integrated in to the X.25 proto stack, shares same frame format, frame types, and field functions as HDLC. Uses ABM xfer mode to est ckts with DTE / DCE.

Primaries- devices initiate commn, Secondaries-those responds to prim commn

Dial on Demand Routing (DDR): Allow WAN Link to be used intermitend for selected appl only. disable if, not in use.

Provides the missing software ingredient to create a fully functional back up sysetm; supported in Cisco IOS ver 9 or later; supports IP, IPX, Aplletalk, DECnet, OSI/CLNS; DDR can be used over sync/Async serial links & ISDN; supports only Static routing(not always active to support dynamic).

X.25: std defines conn between a term & a pkt switching net. works on switched/perm virtual ckts, uses addsng defined by X.121(1-15 decimal digits long), first 4 bits used for DNIC (Data Net Identifier Code), rest is NTN (Net Term No.) & can be assigned by admnr/PSN provider. X.25 defines p-p comms between DTE (router) & DCE (modem or csu/dsu). Can be placed in b types of eqpmts. Works at layer3 using LAPB at layer2. reliable & sliding windows.

DTE , Data Ckt Term Eqpt (DCE), Pkt switching Exge (PSE)

X.25 Sessions: used for DTE to DTE    commn. process estblmt > One dte contacts other, sec one accepts/refuses, if accepted full duplex comm begins, either one can terminate conn. Once terminated must repeat same process.

Virtual Ckts: is a logical bi-directional path from one DTE to other, can pass thro X no.of nodes, can be mux/demuxed,

SVC: Switched Virt Ckt is a temp conn made & terminated for intermittent use. max 8 svc/proto/destn. max 9 proto/svc.

PVC: Perm Virt Ckt is always active once setup, DTE’s can use it any time w/o make/termn.

X.25 Proto suite : maps to lowest 3 layers. Pkt layer proto(PLP), LAPB, X.21 bis are typically used in X.25 imples.

PLP (Pkt layer proto): X.25’s net layer proto, manages pkt exges between DTE devices, can also run over LLC2 imples on LAN, and over ISDN int running LAP on D chnl (LAPD). Five    modes of    opns : 1.Call set up mode:    creates SVC between DTE’s, not used with PVC’s; 2.Data Xfer Mode: used for data xfer between DTE’s via virt ckt, segmentation, reassembly, bit padding, and error & flow control, used with svc & pvc 3.Idle Mode: used when a svc estbed, not    with pvc                        4.Call clearing mode: used to terminate commn sessions & svc’s, not    with pvc’s    5. Restarting mode: used to synch commn between DTE & local DCE,

Link Access Procedure Balanced (LAPB): Bit oriented proto, makes sure that frames are error free & properly sequenced, 3    diff frame types    1. Info frame (I-frame): transports upper layer & some control info, helps sequencing, flow control, error detection & recovery jobs. 2. Supervisory frames (S-frame): Bearing control info, handle req for and the suspension of xmission, 3.Unnumbered frames (U-frames): Bearing control info, handle link setup & termn.

X.21bis: used in X.25 physical layer, at 19.2KB x.21bis supports p to p conn and synch, full duplex Xmission over 4 Wires.

X.25 on Cisco: # config t, # int s0, # encap    x25 - x.25 enabled; If# x25 address 12345-config local x.121 add on x25 ser int;

Mapping proto: # x25 map[proto] [add] [ x.121add] [options], # x25 map    ip    10.6.8.1    123456    broadcast

config i/p, o/p pkt size: If# x25    ips    256, # x25    ops    256 (def-128,max-4096B), config VC range: # x25    pvc    xx (1-4095)

config i/p, o/p window sizes If# x25    win 7, # x25 wout 7 (def-2, max-127).

Monitoring X.25: R# sh    int    s0- shows x.25 config & encap

FRAME RELAY: A Layer2 proto, efficient(sharing), faster(no error check), shares b/w with other users, less cost.

CIR-Committed info rate (min guaranteed b/w)-Gives commn int bet    Dte & Dce, uses more pvc than svc. Gives conn oriented Dllayer comm by using virtual ckts like x.25. uses Inv ARP(find remote IP from dlci values)

A FR vir ckt is a log conn between 2 DTE’s across a pkt switched net (PSN) and is identified by DLCI.

DLCI (Data Link Conn ID-given by svce provider): A number that Identifies a logical conn between CPE & FR switch.

LMI Local Mngmt Interface: A signaling std betwn CPE & FR switch. Gives current DLCI values, Global/Local significance of    DLCI values, status of VCs. LMI-3 types, Cisco(default), Ansi & Q.933a,(since IOS 11.2 LMIs auto sense)

Config FR: specify FR as encaps on Serial Int. Cisco (def, cisco-cisco)  IETF (Inet engg task force, cisco-non cisco)

If# encap frame-relay-fr cisco enabled, # encap frame-relay IETF: fr IETF enabled,

config LMI: if# frame-relay lmi-type [ansi/cisco/q933i], (for IOS before 11.2)

config DLCI: at int s0: # frame-relay interface-dlci 16 : (16-1007) ; # sh int    s0–sh dlci, lmi tafic,fr line speed,no dlci no.

Enab broadcast:    # frame-relay    interface-dlci    16    broadcast- allows routing updates only if broad is enabled.

Sub Interfaces: multi VCs on single serial int for diff proto. Config sub int: # int    s0.16 [p-p / multi point]. (0-4292967295),

point-point: one sub int each to each rem router, do not forw broad, rout updates, each pr of routers are in its own subnet.

multi point: one sub int to multi rem router, forw broad, updates it recvs,    all routers in one subnet & IP routing.

Mapping Frame Relay: IP add of both end devices on a VC must be mapped to their DLCIs. Called FR map table

1.use frame relay map, config: # int s0.16, # encap frame-relay ietf, # no inverse-arp, # ip add 172.16.30.1 255.255.255.0, # frame-relay    map    ip    172.16.30.17    30, # fr map ip 172.16.30.19 50 broad, # fr map ip 172.16.30.18 40 cisco    broad, 30/40/50 dlci nos, VCs 1,2 ietf encap, 3 cisco encap, FR map : only way to mix ietf &    cisco encap

2. use Inv ARP: # int s0.16, # encap frame-relay ietf, # ip add x.x.x.x    x.x.x.x    (Inv ARP on def, does dynamic mapping)

Monitoring Frame Relay: many ways to check status of int & pvc’s, # sh frame pvc- sh all    configured dlci    nos +    pvc stat,

R# sh frame-relay [ip/lmi/map/pvc/route/traffic]-shows fr ip stast / lmi stat / fr map table / each pvc, dlci / route / proto stat

R# sh int s0-sh lmi traffic, line speed, dlci, lmi info, no dlci no,    R# debug frame-relay lmi- BrainDump.

PPP: layer 2+3 proto can be used over Asynch / Synch    serial, ISDN media. uses LCP (Link Control Proto) to build, config, maintain & termte    data link conns. provides Authentication PAP, CHAP and Data compression. Thro’ NCP (Network control proto) it encaps multi proto    IP, IPX, AppleTalk, DECnet, OSI/CLNS and transparent bridging.

Config PPP Encap:    # config t,    # int s0,    # encapsulation    ppp, # ppp auth chap,    # sh    int    s0- shows ppp config.

# debug ppp authentication: shows CHAP/PAP auth info Exge sequence as it occurs. Live.

Config    PPP password:    # username xxxx    password    xxxx- user name/ p.w    expected from remote router.

ISDN : a set of protos carries a group of Digital svcs that simultaneously convey Data, Text, Voice, Music, Graphics & Video to end users and it is designed to achieve this over the existing tel line. ISDN works in phys, DLL & Net layers.

Devices connecting to ISDN net are known as Terminals. TE1 Term    Eqpt type 1 understand ISDN stds, TE2 predate ISDN stds, use TE2 thro Term Adapter (TA).    To access ISDN with Cisco router- need Network    Termination 1(NT1) or ISDN modem. If    router has BRI Int.- can directly access ISDN. Can access from serial int thro a Term Adaptor (TA),    Router with BRI int= TE1(Terml end-point1),    R that require TA= TE2 (Terml end point 2).

ISDN Ref points: uses 4    Ref points to define logical interfaces. R : defines the ref point between non ISDN eqpt & TA,

S : defines the ref point between user terminals & an NT2,    T : defines the ref point between NTI & NT2 devices,

U : defines the ref point between NTI devices and line term eqpmt in a carrier net (only in N. America)

ISDN protocols:      E-series specifies ISDN on existing tel network, Basic telephone services.

I-series specifies Concepts, terminology and services,    Q-series specifies switching and signaling.

Basic Rate Interface (BRI) : 2B+1D Chl. 2x64kb B Chl. carries Data, 16kb D chl. carries cntrl & signaling info.(tot 144kb). D chl signaling proto spans phy, dll, Net layers. Config BRI: Need SPID (Service Profile Identifiers), one for each B chl.

Its like Tel no. for each B chl. ISDN Device can access BRI service only after giving SPID no. to the ISDN switch.

# config t,    # isdn    switch-type    basic-dms100, # int bri0, # encap    ppp, # isdn spid1 77545, # isdn spid2 77546

Primary Rate Interface (PRI): 23B+1D64 chl. (1.544mb-T1), 30B+ 1D64 (2 mb-E1)

Config PRI: for each ISDN PRI int, need to specify Data link specific info for the T1 controller, this way PRI commtes with ISDN switch, Framing & line coding info is essential. # config t, # controller T1 1/0,      # int    pri0,    # framing esf,

# linecode    b8zs, # pri-group timeslots 1-24,    Extended Superframe Framing (ESF) is used in T1 ckts, consists 24 frames of 192 data bits each, with bit 193 provides timing & other functions. B8ZS is a binary eight-zero substitution line-coding mechanism    guarantees delivery of    data of 8 cont. zeros. Time slots 1-24 defines ISDN PRI time slots 1 to 24.