# ICRC by Laura Chappel   Questions   brought to you by Chillnz

# The Internetworking Model

1.1 Define the three types of networks that make up today’s global internetworks:

- LANs

- WANs

- Enterprise networks

1.2 What are the seven layers of the OSI model?

7: Application

6: Presentation

5: Session

4: Transport

3: Network

2: Data Link (contains LLC and MAC sublayers)

1: Physical

1.3 How can data be defined when it is at the network layer? Packets and datagrams

1.4 Define the function of the network layer of OSI? Defines network addressing and determines the best path through an internetwork.

1.5 Correct steps of converting data to bits for transmission: data, segments, packets, frames, bits.

# Applications and Upper Layers

2.1 Three examples of each application type:

- Computer application: word processing, presentation graphics, spreadsheet, database, project planning.

- Network application: e-mail, file transfer, remote access, client/server processes, information location, network management.

- Internetwork application: EDI (electronic data interchange), WWW, e-mail gateways, special-interest bulletin boards, financial transaction services, internet navigation utilities, conferencing (video, voice, data).

2.2

- Guarantees receipt of data; ensures data integrity: positive acknowledgment

- Occurs when receipt timers expire or an error is detected: retransmission

- Often referred to as the “handshake process”: connection-establishment

- Enables many applications to use the same data stream: multiplexing

- Uses buffering and congestion avoidance to prevent data overflowing memory or being lost before it’s processed: flow control

- Controls amount of data sent end-to-end by defining how many data segments may be sent before acknowledgement is required: windowing

2.3 Host A has established connection-oriented session with Host B.  Host A transmits data packets 1, 2, 3 to Host B.  Packet 2 is involved in a collision during transmit process.

How should data transfer recover and resume? Host B will send back an acknowlegment indicating that packet 2 must be retransmitted.  Host A will retransmit packet 2.  Host B will then send an acknowlegment requesting packet 4.  From this point communication flow should resume normally.

# Physical and Data Link Layers

3.1 What are the Cisco router interface abbreviations for Ethernet, Token Ring and FDDI?  E, To, Fo

3.2 Which frame types use an Ethernet type field to define the protocol in use?  Ethernet II and SNAP frame types

3.3 Two sublayers defined within the data link layer? LLC (logical link control) and MAC (media access control)

3.4

- SDLC = Uses primary and secondary roles for IBM data links

- EIA/TIA-232 =  Voice-grade access, formerly a recommended standard

- 802.3 =  From IEEE efforts; uses field for length rather than type

- Frame Relay =  Uses simplified HDLC for higher-speed communication

- Ethernet II = From original Xerox work; uses field for protocol type

- FDDI =  Uses five-bit symbols rather than octets in it’s framing

- Token Ring = Equivalent of IEEE 802.5

# Network Layer and Path Determination

4.1 Link-state routers build a common view of internetwork topology? T

4.2 Link-state routing uses periodic updates, resulting in relatively slow convergence? F

4.3 Distance vector routing develops a view from neighbor routers’ perspectives? T

4.4 Distance vector routing passes an updated routing table from neighbor to neighbor? T

4.5 In link-state routing, events trigger updates for relatively fast convergence? T

4.6 Distance vector routers process updates in parallel with other routers? F

4.7 TCP/IP routers forward packets based on the contents of the packet’s IP header.

4.8 On a 10BaseT network, the Ethernet/802.3 frame is placed directly in front of the network layer header.

4.9, 4.10, 4.11 Which header could contain the following data?

- Destination 10.4.0.6  Network-layer header

- Source 00.00.OB.A4.26.39  Data-link header

- Destination port 20  Transport-layer header

# Basic Router Operations

5.1 Correct command to access each router element:

- RAM: show version, show running-config

- NVRAM: show startup-config

- Flash: show flash

- Console privilege EXEC mode: enable

- Interfaces: show interfaces

5.2 How can you use the context-sensitive help to list privileged EXEC commands? Type ? at the privileged EXEC mode prompt

5.3 What command is used to enter privileged EXEC mode? enable

5.4 What happens when you enter the show history command? System displays last 10 recorded commands

5.5 What happens when you type exit at the privileged EXEC mode prompt?  Router quits privileged EXEC mode and goes back to user mode

# Configuring a Router

6.1  Configuration files can come from the console, NVRAM or a network server? T

6.2  The router loads the image file based on the values set in the Configuration Register? T

6.3  Multiple source options provide flexibility and fallback alternatives? T

  - If the router cannot find a configuration file, the router enters shutdown mode? F

  - The router has a single configuration mode to handle all configuration areas? F

  - Cisco routers provide multiple levels of password protection? T

  - Cisco routers support configuration parameters that aid in router identification? T

  - Cisco IOS Release 10.3 or later uses the copy configure command to save configuration files? F

  - What command is used to enter global configuration mode and specify that configuration commands will originate from the terminal? configure terminal

6.10 What command defines a login banner?  banner login

6.11 What command is used to create a message-of-the-day banner?  banner motd

6.12 What must you enter after the message-of-the-day banner command to indicate the end for the banner message?   delimiting character

6.13 What command string puts you in interface configuration mode for interface Serial1?  interface serial 1

6.14 If you set both the enable and secret passwords, which one is used to enter privileged EXEC mode?  secret password overrides enable password

6.15 What command do you use to verify that the interface Serial 1 is cabled as a DCE interface? show controllers serial

6.16 What command do you use to enable an interface? no shutdown

6.17 In what mode must the router be before you can issue the no shutdown command?  You must be in the interface configuration mode for the specific interface you want to enable

6.18 What command do you issue to obtain the current Configuration Register setting?  show version

6.19 What commands boot the system file from:

- Flash: boot system Flash [IOS_filename]

- ROM: boot system rom

- TFTP server: boot system TFTP [file name][TFTP server address]

6.20 What mode must the router be in before you can issue a boot system command? The router must be in global configuration mode

# Discovering and Accessing Other Cisco Routers

7.1  CDP (Cisco Discovery Protocol) is a protocol and media independent tool?  T

7.2  CDP displays information about directly connected routers, hubs, and management consoles? F

7.3  CDP sends and receives updates at regular intervals? T

7.4  CDP can be enabled by router or by interface? T

7.5  CDP is off by default?  F

7.6  CDP uses broadcast for discovery? F

7.7  CDP is used only for discovery on LANs? F

7.8  What is the purpose of the CDP timer setting?  Specifies the length of time between CDP updates

7.9  Why would you consider disabling CDP?  Network with a single Cisco router or very stable network that cannot support the extra traffic

7.10 What is the purpose of CDP holdtime parameter?  Indicates how long the receiving device should hold a CDP packet sent from the local router

7.11 How can you identify CDP packets on your network?  Multicast address 0100.0ccc.cccc and the value 2000 in the SNAP header

# TCP/IP Overview

8.1 Four TCP/IP conceptual layers?

1 Application

2 Transport

3 Internet

4 Network

8.2 OSI layer does IP map to? Layer 3

8.3 OSI layer does UDP map to? Layer 4

8.4 Describe protocol in use:

- Maps a known IP address to a MAC sublayer address: ARP

- Includes layer 4 (Transport) protocol ID in header: IP

- Used to send “Destination Unreachable” messages: ICMP

- Breaks messages into datagrams: TCP

- Uses sequence numbers: TCP

- Relies on application-layer reliability: UDP

- Provides “best-effort” delivery: UDP or IP

- Reassembles datagrams into messages: TCP

- Handshakes with receiving device: TCP

- Used to send error and control messages: ICMP

- Provides connectionless transmission: UDP or IP

- Send acknowledgments: TCP

- Uses no windowing: UDP

# IP Addressing

9.1, 9.2, 9.3 Determine address class and subnet of the following:

172.16.2.10 255.255.255.0 B 172.16.2.0

10.6.24.20 255.255.0.0 A 10.6.0.0

10.30.36.12 255.255.255.0 A 10.30.36.0

9.4, 9.5, 9.6 Calculate address class, subnet number and broadcast address of the following:

201.222.10.60/29 255.255.255.248 C  201.222.10.56  201.222.10.63

15.16.193.6/21 255.255.248.0 A  15.16.192.0  15.16.199.255 (check this one!)

128.16.32.13/30 255.255.225.252 B  128.16.32.12  128.16.32.15

# IP Routing Configuration

10.1  To verify that IP routing is enabled, you can enter the show protocols command? F

10.2  You issue the router rip command to enable the RIP routing protocol? T

10.3  You issue the network network-number subnet mask command to associate a network with a routing process? F

10.4  You issue the show ip protocol command to verify that the RIP routing protocol is enabled? F

10.5  You issue the show ip rip route command to display the current status of the RIP routing table? F

10.6  In the RIP routing table display, how do you determine which networks are discovered by the RIP routing protocol? Networks discovered by RIP are preceded by code letter R

10.7  What command displays the RIP routing updates sent from and received by your router? debug ip rip

10.8 What command disables the display of the RIP routing updates sent from and received by your router? no debug ip rip

10.9 What command do you issue to enable the IGRP routing protocol? router igrp autonomous-system

10.10 Can you add multiple autonomous groups in a single enable IGRP command? NO.  A new router igrp command must be issued for each autonomous system

10.11 What command do you issue to verify that the IGRP routing protocol is enabled? show ip protocols

10.12 What command do you issue to display the current state of the IGRP routing table? show ip route

10.13 What command displays the IGRP routing updates events sent from the router? debug ip igrp events

10.14 What type of routing metric does IGRP use, and what are the five components of that metric?

1.Bandwidth

2.Delay

3.Reliability

4.Loading

5. MTU (Maximum transmission unit)

# Configuring Novell IPX

11.1 Determine the required IPX address and encapsulation type for a given router port. Sorry, find out for yourself!

11.2 What command do you issue to enable IPX routing on a router? ipx routing

11.3 What command mode must the router be in before you can issue the ipx routing command? Router must be in global configuration mode

11.4 What command do you issue to assign IPX network numbers on a router? ipx network number

11.5 What command do you issue to verify entries in the routing table? show ipx route

# Configuring AppleTalk

12.1 AppleTalk addressing is composed of network.node? T

12.2 Node numbers are dynamically acquired? T

12.3 Multiple network numbers can exist on one wire? T

12.4 Servers use broadcasts to learn about available clients? F

12.5 What command do you issue to enable AppleTalk routing on a router? appleTalk routing

12.6 What command do you issue to assign ranges to each interface on a router? appleTalk cable-range cable-range

12.7 What command do you issue to assign zones to each link on a router? appleTalk zones zone name

12.8 What command do you issue to verify the address assignment on a router? show appleTalk interface

12.9 What command do you issue to verify entries in the routing table? show appleTalk route

# Basic Traffic Management with Access Lists

13.1 What command do you issue to verify that no prior access lists exist on a router? show ip interface

13.2 What command activates the IP extended access list on a router interface? ip access-group access-list-number in

13.3 What command mode must the router be in before you can issue the command in the previous question? Router must be in interface configuration mode

13.4 What command do you issue to verify that a new IP access list is active on an interface? show ip interface

13.5 What command do you issue to display the contents of an access list? show access-lists

13.6 What range of list numbers must you use to define a standard IPX access list? 800 to 899

13.7 What command do you issue to verify that a new IPX access list is now active on the interface? show ipx interface interface

13.8 What command do you issue to display the contents of an IPX access list? show ipx access-lists

13.9 What range of list numbers must you use to define an AppleTalk access list? 600 to 699

13.10 What command do you issue to display the contents of an AppleTalk access list? show appletalk access-lists

### Introduction to WAN Connections

14.1 A WAN subscriber must know how to interface customer premises equipment to the provider service? T

14.2 PPP sets data-link encapsulation capable of transmitting packets from multiple protocols? T

14.3

- Uses a separate channel for control messages between transfer points to a called destination: Call setup service

- Statistically allocates bandwidth on a single channel to multiple circuits: TDM (Time-division multiplexing)

- Also known as signaling: Call setup service

- Uses fixed time slots to eliminate congestion: TDM (Time-division multiplexing)

- Packets of information share a nondedicated channel: X.25 or Frame Relay

- Sets up and clears calls between users: Call setup service

- Route is a dedicated path between sending location and receiving location: TDM (Time-division multiplexing)

- Uses virtual circuits to avoid call setup delays: X.25 or Frame Relay

14.4 Three forms of WAN access available on Cisco routers?

1. Switched or relay services

2. Front end to an IBM enterprise data center computer

3. Connection between peer devices

14.5 What command do you issue to verify that your interface is configured for PPP encapsulation? show interfaces interface-number

14.6 What command do you issue to display the CHAP exchange sequence? debug ppp authentication (This command is issued prior to initiating the line.)

# Configuring X.25

15.1 X.25 defines the lower three layers of the OSI model? T

15.2 LAPB (Link Access Procedure, Balanced) is the network protocol? F

15.3 Tunneling of other protocols inside X.25 is supported? T

15.4 To configure an X.25 interface you must:

- Define the interface encapsulation? T

- Set critical parameter values for attaching to the PDN? T

- Configure the interface X.121 address? T

- Define any protocol to X.25 mapping? T

- Define static IP addresses on both sides of the link? F

- Configure one router as a master to perform setup steps? F

15.5 What command do you issue to specify X.25 encapsulation on your DTE interface? encapsulation x25

15.6 What command do you issue to specify X.25 encapsulation on your DCE interface? encapsulation x25 dce

# Configuring Frame Relay

16.1 Subinterfaces enable you to set up point-to-point Frame Relay networks? F

16.2 Frame Relay can be used in a variety of topologies, depending on your network  requirements? T

16.3 Frame relay routers and hubs can assist in managing traffic congestion? F