Cisco Certified Network Associate Study Guide

- Interoperability involves making sure that the data can be understood by equipment manufactured by different vendors that use different operating systems, hardware or languages.

- The data rate range for X.25 is from 9.6 Kbps to 256 Kbps.

- IBM's Token Ring specification is nearly identical and compatible with IEEE's 802.5 specification.

- OSI stands for Open Systems Interconnection

- The three switching modes included in Catalyst 1900/2820 switches are: fastforward, fragmentfree, and store and forward.

- A backoff algorithm is used to prevent two nodes from retransmitting simultaneously and creating an endless number of collisions by delaying the retransmission for a variable length of time on each node.

- A Network Control Protocol frame in a Point to Point Protocol is used for selecting and configuring the network layer protocol. A Link Control Protocol (LCP) is used to establish and configure a connection.
FDDI specifies protocols at both the MAC sublayer and the physical layer of the OSI reference model.

- In a Token Ring network architecture, what does it mean when a node possesses a token? When a node receives the token, if the node has data to transmit, it flips a bit in the token and transmits the data. If the node has no data to transmit, it sends the token to the next node.

- What two types of frames are found on a Token Ring network? There is a token frame and a data frame used on a Token Ring network.

- A Frame Check Sequence (FCS) in the IEEE 802.3 frame includes a CRC value.

- “plntspa” – plant spa is the reverse order of the layers of the OSI model.

- Which statements are true of a preamble in the IEEE 802.3 frame?   The preamble is an indication of the start of a new frame. It contains alternating zeroes and ones.

- The session layer is usually implemented in the software. The physical and data link layers are usually implemented in hardware and software. The remaining layers are usually implemented in the software.

- Data Link Connection Identifier in a Frame Relay network identifies a connection between two DTEs.

- Hardware addresses are defined at the MAC portion of the data link layer.

- RS-232 is an extremely common serial port interface.

- Beaconing is a Token Ring mechanism for notifying other stations that a serious error has occurred on the network.

- The ITU X.25 Standard applies to the physical, data link, and network layers' protocols.

- The maximum data transmission rate for HSSI is 52 Mbps.

- The two header formats in ATM terminology are: User Network Interface (UNI) and Network Node Interface (NNI).

- Out-of-band management is network fault tolerant, because it can manage a switch or hub even when network services are interrupted.

- The data rate range for X.25 is from 9.6 Kbps to 256 Kbps.

- In a Frame Relay network, which statement is true of Forward Explicit Congestion (FECN) when the network is congested? Both FECN and Backward Explicit Congestion (BECN) contain a bit that is set to "1" and is sent by DTE to upper protocol layers to notify of traffic congestion between source and destination.

- In a Cisco router, configuration register information can be used to select a boot source and default file name, enable booting from a TFTP server, and load operating software from ROM.

- The command SHOW CDP NEIGHBOR DETAIL will show the IOS version.  CDP does not need an IP address to run, it can use MAC.

- The configuration register has 16 bits.

- What is the sequence of events that occurs when you power up your router? The router first tests its hardware, then attempts to locate and load an IOS image, and finally locates and applies its specific configuration information.

- The SHOW STARTUP-CONFIG will show the configuration stored in NVRAM. The SHOW RUNNING-CONFIG will show the configuration currently running.

- A good connection is indicated by which of the following in an output display of the PING command? A series of exclamation points (!!!!!) indicates a good connection. A series of dots (…..) indicates that the pings timed out.

- IOS image is usually stored in Flash and booted from Flash. RAM stores routing tables and the running configuration. NVRAM is for the startup configuration.

- The boot field set to 0x1 will cause the router to boot from ROM.

- The command history buffer will allow you to paste in your last 10 commands. This can be changed with the command HISTORY SIZE X.

- To suspend a Telnet session, use the keystrokes CTRL-SHIFT-6 x.

- The response to the command ROUTER# CONFIGURE ? will be a list of possible options from where the router can be configured.

- The boot field is the lowest four bits of the configuration register.

- If a mistake is made in specifying the file name in the COPY TFTP FLASH command, the router will still function because it still has working image in RAM.

- The router will try to resolve an incorrect command to an IP address, thinking it is a hostname, so it can establish a Telnet session with it.

- You would use either LOGOUT or EXIT to log out of the router and end your session.

- The configuration register setting is at the last line of the SHOW VERSION command. This command also shows router up time and IOS version.

- CTRL-Z will get the user completely out of the configuration mode. EXIT takes the user back one step. LOGOUT terminates your session.

- TERMINAL MONITOR sends Debug output to the VTY session.

- The command executive is the part of the IOS software that provides the user interface and interprets the commands you type.  The configuration register is a memory location in NVRAM that controls the router's operation as it boots up.

- You must have an IP address assigned to an Interface in order for CDP to operate? False. CDP will multicast to a functional MAC address. There is no need to have any network layer address assigned to the interface to get the CDP messages to go out.

- Supernetting removes some of the rightmost bits of the default subnet mask, which summarizes several contiguous Class C networks into a single network entity.

- “ROUTER# PING” Extended ping requires Privileged mode, and also requires that no command-line options be entered (because it prompts for each option).

- To configure a name-to-address mapping in the router mapping table, you would issue which of the following commands? ROUTER(CONFIG-IF)# IP HOST  or  ROUTER(CONFIG)#IP HOST

- The significance of the address 3.255.255.255 is that it is a directed broadcast to network 3.0.0.0. A host number cannot have all ones. A local broadcast would have all ones (for example, 255.255.255.255). While this is not a legal host address, it is perfectly valid as a broadcast address.

- Class D subnet masks are used for multicasting purposes.

- What IOS command turns off name-to-address resolution? ROUTER(CONFIG)# NO DOMAIN-LOOKUP

- Which of the following statements is true regarding IP host addresses? The subnet address part of an IP address can not be set to "all binary ones" or to "all binary zeros".

- The value 24 after / in the IP address 135.120.25.20/24 is called a prefix.

- Subnet bits are taken from the highest-order contiguous bits of the host address in order to add subnets and/or reduce the number of hosts.

- A Class D network is used for multicasting, not for normal IP network/host addressing, so it doesn't have a default subnet mask.

- Up to six name server addresses can be specified using the ROUTER(CONFIG)# IP NAME-SERVER command.

- Given a subnet address of 5.32.0.0 and a subnet mask of 255.224.0.0, what is the highest allowed host address on this subnet? 5.63.255.254

- If we saw the following subnet addresses, what would be the subnet mask associated with these subnets? 140.120.4.0 | 140.120.8.0 | 140.120.12.0 | 140.120.16.0  - 255.255.252.0

- The IOS command you would use to define a global format to view the subnet mask during the "current session" is ROUTER # TERM IP NETMASK-FORMAT.

- Subnet bits are added to the default subnet mask  to segment the network into subnets.

- A user on a Washington DC network receives the following response after issuing a router command: Tracing the route to Honolulu 1Tokyo(127.893.81.2) 800 ms 6 ms 4 ms 2 Lisbon(141.925.64.7) 600 ms 8 ms 6 ms Honolulu(151.666.59.4) 400 ms 10 ms 8 ms Washington dc# This response was most likely obtained by issuing the command:  WASHIGNGTONDC# TRACE HONOLULU

- We do not assign IP addresses to terminal lines. The terminal lines include the console, auxiliary port, and virtual terminal lines 0 - 4, which are used to configure the router. IP addresses are assigned to the interfaces on the router.
Telnet is the only command that would test all layers of a protocol up to the application layer.

- The leading bit pattern 1111 is associated with Class E addresses, which range from 240-254 in the first octet.

- The Class D addresses have bit pattern which begins with 1110. This allows values from 224 - 239 to be assigned for the purpose of multicasting.

- The purposes of subnetting are to segment and organize a single network at the network layer.

- VLSM allows us to use different subnet masks in different parts of the network and to divide a subnet into secondary subnets.

- A socket is an IP address plus a port

- TLI (transport layer interface) makes the transport layer independent from upper-layer services and represents a System V (API) Application Programming Interface.

- NetBIOS functions at the OSI model session layer.

- Windowing is a form of flow control at the transport layer.

- IP (Internet Protocol) is the network layer protocol that moves data from one node to another.

- The urgent pointer is not a part of the UDP header. The UDP header has only four parameters: source port, destination port, length, and UDP checksum.

- With ARP, an administrator does not have to physically manage the MAC address of each NIC. ARP separates routing function from the physical and data link layers.

- WinSock is a Microsoft Windows version of sockets.

- The transport layer is the single most important layer in assuring reliable data transfer regardless of the underlying networks in between the TCP/IP protocol suite.

- RARP is used by a system to resolve its IP address from the known MAC address.

- A port is where upper-layer processes access the transport layer.

- The spanning-tree algorithm allows for the prevention of duplication of forwarding of packets and forwards packets in a controlled manner.

- ICMP Redirect is sent by a gateway to the host. It instructs the host to use a different route when the router detects that its route is not as optimal as that of another router.

- The TCP/IP protocol suite does not have a formal session layer.

- RPCs provide transparency to make remote calls look local, and portability of applications between heterogeneous systems. RPC is a connectionless session.

- ARP is a broadcast protocol, and ARP caching is used because broadcasts are expensive.
Distributed Computing Environment (DCE) is an example of and RPC implementation.

- Which statement is true of WinSock? It provides the means for sharing an Internet connection between multiple IP protocol suite utilities

- NetBIOS can run over IPX, IP, and NetBEUI but not AppleTalk.

- Sockets allow multiple applications to use the same TCP/IP connection.

- Inverse Address Resolution Protocol (InARP) is generally used by nonbroadcast networks such as Frame Relay.

- The TCP/IP protocol suite has four layers.

- ICMP, IP, ARP, and RARP all map to OSI layer 3.

- Fragmentation is a process that occurs on a router somewhere between the source and destination. This process segments the datagram into a convenient size to fit into a single frame for transport over the network.

- If an intermediate node such as a router becomes congested, there is no mechanism defined that can notify TCP.

- The sequence number in a TCP header is used for acknowledgments, for reordering of the octets received, and for rejecting duplicate octets.

- What is SMTP's transport layer protocol port?  TCP/23

- UDP and TCP represent mechanisms used by which layer of the TCP/IP?   Transport Layer.

- What layers do not exist in the TCP/IP model that are in the OSI model? Presentation, session, and data link.

- IGRP is a distance vector and Interior Gateway Protocol with a 224 maximum network diameter (maximum 224 hops).

- The two mechanisms that make up a routing protocol are selecting the best route and transmitting the data

- Aging in a distance vector routing algorithm is set for all routing information.

- How can the administrator change how IGRP selects routes? The administrator can adjust the metric weights used for determining the route selection

- Routing updates are the method used to maintain the routing table.

- A route is removed from the routing table after seven update periods. In IGRP, updates are broadcast every 90 seconds. After three update periods where a route is missed, the route is considered unreachable. After seven periods, the route is removed.

- Low overhead of a routing algorithm is normally associated with CPU usage, and bandwidth. Accuracy is associated with optimal route.
A default route is normally a type of static route that can be known as the gateway of last resort, where all nonroutable packets are forwarded to for further handling.

- The maximum number of hops used in EIGRP is 224.

- RIP authentication command can be used only with RIP version 2.

- In a hybrid network, the nonroutable packets are routed to a static router designated a router of last resort.

- For larger or complex networks, if the holddown period is made sufficiently large, then problems associated with the network stability could be minimized, though not eliminated in all cases.

- In a Frame Relay network, which statement is true of Forward Explicit Congestion (FECN) when the network is congested?

- Both FECN and Backward Explicit Congestion (BECN) contain a bit that is set to "1" and is sent by DTE to upper protocol layers to notify of traffic congestion between source and destination.

- OSI stands for Open Systems Interconnection

- The presentation layer of the OSI reference model is concerned with data formats, syntax and encryption.

- The maximum data transmission rate for HSSI is 52 Mbps.

- The ITU X.25 Standard applies to the physical, data link, and network layers' protocols.

- DIX (Digital, Intel, and Xerox) co-created Ethernet.

- To turn off name-to-address resolution, the NO DOMAIN-LOOKUP command is issued from the global command prompt.

- The invalid subnet ID is 200.0.16.0. This represents an ID where all the subnet bits are zero.

- The significance of the address 3.255.255.255 is that it is a directed broadcast to network 3.0.0.0. A host number cannot have all ones. A local broadcast would have all ones (for example, 255.255.255.255).

- To configure a name-to-address mapping in the router mapping table, you would issue which of the following commands? ROUTER(CONFIG-IF)# IP HOST or  ROUTER(CONFIG)#IP HOST

- A Class D network is used for multicasting, not for normal IP network/host addressing, so it doesn't have a default subnet mask.

Chapter 1

Layer 7 – Application

Identify and establish the availability of the intended communication partner, and determine if sufficient resources exist for the communication.

Layer 6 – Presentation

Data compression, decompression, encryption, and decryption are presentation layer.  Presentation layer standards include MPEG, MIDI, PICT,TIFF, JPEG, ASCII, and EBCDIC.

Layer 5 – Session

Dialog control between devices or nodes.  Organizes the communication through simplex, half and full duplex modes.  Deals with connection establishment, data transfer, and connection release.  Protocols include NFS, SQL, RPC, Appletalk Session Protocol (ASP) and X Windows.

Layer 4 – Transport

Handles multiplexing upper-layer applications, session establishment and tear down of virtual circuits.  Hides details of network dependent info from the higher layers by providing transparent data transfer.  The “window” works at this level to control how much information is transferred before an acknowledgement.

Layer 3 – Network

Sends data from the source network to the destination network.  The router will use packet switching to move a packet from one interface of port, to another through the network cloud.

Layer 2 - Data Link – MAC & LLC

Translates messages from the upper layers into data frames and adds customized headers containing the hardware destination and source address.

Logical Link Control Sublayer – Acts as a managing buffer between the upper layers and the lower layers.  Uses Source Service Access Points (SSAPs) and Destination Service Access Points (DSAPs) to help the lower layers talk to the Network layer.  Responsible for timing, and flow control.

Media Access Control Sublayer – Builds frames from the 1’s and 0’s that the Physical layer picks up from the wire as a digital signal, and runs Cyclic Redundancy Checksum (CRC) to assure that nothing was damaged in transit.

Media Access Types – Contention, Token Passing, and Polling.

Layer 1 – Physical

Sends and receives bits.  Communicates directly with communication media.

Layer 2 WAN Protocols (Data Link Layer) – How frames are carried between systems on a single data link.

High Level Data Link Control (HDLC) – default protocol for all Cisco serial links.  Won’t talk to another vendors HDLC.

Synchronous Data Link Control (SDLC) – designed to work with mainframes and remote offices, uses polling media-access.  The front-end primary polls the secondaries to determine if they need to communicate.

Link Access Procedure, Balanced (LAPB) – Detects out of sequence frames and was designed for X.25.

X.25 – Point-to-point communication between DTE and DCE and supports Switched and Permanent Virtual Circuits.  Cisco routers (DTEs) connect to modems or CSU/DSUs (DCEs).

Serial Line IP – SLIP is an industry standard for low speed serial interfaces.

PPP – Adds login, password, and error correction capabilities and is used by IP, IPX, and Appletalk.

Frame Relay – upgrade from X.25 and replaces need for LAPB.  High quality digital from telco.

Repeaters work at the Physical Layer and boost the signal to alleviate attenuation.

Bridges work at the MAC sublayer of Datalink and forwards data to the network segment with the correct MAC address.  Runs at Full Duplex, and can cause Broadcast Storms due to forwards to all attached networks when a broadcast.  Bridges cannot read network numbers.

Routers filter by both hardware and network address.  Only forward packets to the network segment it is destined for.

Data Encapsulation is process where info is wrapped in the data section of another protocol in the chain of Layers.  Each layer encapsulates the layer immediately above it as data flows down the protocol stack.

802.2 = SAP

802.3 = Novell-Ether (default)

Ethernet_II = ARPA

Snap = Token_Snap, FDDI_Snap, etc.

Destination MAC address is FFFFFFFFFFFFFFFF for a broadcast.

ATM uses 53-byte fixed size cells instead of frames.  Can allocate bandwidth on demand.

SVC – Switched virtual circuit that is dynamically created and torn down after transmission.

Which layer defines bit synchronization?

Layer 1, the Physical Layer. Sets up the specifications for activation and maintaining the physical link between layers

The two sublayers of the IEEE Data Link layer are which of the following?

At Layer 2, Logical Link Control and Media Access Control

Which layer defines the physical topology?

Layer 1 Physical controls topology.

Which of the following are Session layer standards?

Session Layer includes NFS and SQL.

What is the Network layer of the OSI reference model responsible for?

Routing of packets.  Bridging occurs at the Data Link Layer and regeneration of the digital signal happens at the Physical Layer.  Gateway services act on all layers.

Which three pairs of the following are Presentation layer standards?

MPEG, MIDI, PICT,TIFF, JPEG,ASCII, and EBCDIC.

Which layer is responsible for putting 1s and 0s into a logical group?

Layer 2 – Data Link.  Is responsible for transmission of data over a particular physical link.

Which layer is responsible for framing?

Layer 2 – Data Link.  Responsible for transmission of data over a particular physical link.

Which layer is responsible for addressing devices and routing through an internetwork?

Layer 3 – Network.  Provides connectivity and path selection between two end systems.  The routing layer.

Which layer is responsible for flow control, acknowledgment, and windowing?

Layer 4 – Transport.  Specifies reliable communication between end nodes.  Establishes maintenance and termination of virtual circuits, transport fault detection, recovery and flow control.

Which layer hides details of any network-dependent information from the higher layers by providing transparent data transfer?

Layer 4 – Transport.  Specifies reliable communication between end nodes.  Establishes maintenance and termination of virtual circuits, transport fault detection, recovery and flow control.

Which layer is responsible for coordinating communication between systems?

Layer 5 – Session.  Establishes and terminates sessions between applications and manages the data exchange.

Which layer is responsible for negotiating data transfer syntax?

Layer 6 - Presentation.  This layer insures that the application layer of another system will understand what is being sent.

- Which layer is responsible for synchronizing sending and receiving applications?

- Which layer is responsible for identifying and establishing the availability of the intended communication partner?

- Which layer is responsible for determining if sufficient resources for the intended communication exists?

Layer 7 – Application.  E-mail, file transfer, and terminal emulation outside of the OSI model.  Identifies and establishes the availability of intended communication partners and synchronizes the applications.

CPE is an acronym for which of the following?

Customer Premises Equipment. Terminating and telephone equipment on a local site.

Which three of the following are true statements about connection-oriented sessions?

Connection-oriented sessions take place at Transport Layer. TCP is responsible for segment delivery.  Unacknowledged segments are resent.  All received data is acknowledged by the sender.  Segments are sequenced and put back into order upon arrival.

CSU/DSU is an acronym for which of the following?

Channel Service Unit is a device that connects end-user equipment to the local digital telephone loop.  Data Service Unit is used to adapt the physical interface on a DTE device to a circuit like T1.  DSU does the signal timing as well.

CO is an acronym for which of the following?

Central office.  The local telephone company office where local loops are connected to circuits.

Choose three reasons why the networking industry uses a layered model.

It allows changes in one layer to occur without interrupting the operations of another layer.  Allows for systematic troubleshooting.

Chapter 2

Routers do not handle as much traffic as switches because routers take longer to process frames and packets because there is more to look at.

Three kinds of switching

- Port switching allows a port to be assigned to a specific network segment.

- Frame Switching is used to increase bandwidth by allowing transmissions to occur in parallel.

- Cell Switching is used in ATM environments were cells are switched.

Three switching modes

- Store and forward – LAN switch copies the entire frame into buffers, and runs a CRC.  High latency.

- Cut-Through – LAN switch only copies the destination address and immediately forwards out the correct interface.  All errors are filtered.  Lowest latency.

- Fragment Free – Modification of cut-through where the switch waits for the collision window to pass before forwarding.  Checks only for collisions.

Frame identification (tagging) uniquely assigns a user-defined ID to each frame’s header field as it passes through the switch fabric.  After passing the fabric, it removes the header for transmittal to the target host.  It then removes the VLAN ID since nothing outside the switch would understand it. (Inter-Switch Link – ISL is the only exception and keeps the VLAN ID as it passes over the ISL link.)  All Layer 2 functions.

The CSMA/CD Ethernet IEEE committee is defined as which of the following?

IEEE 802.3 uses CSMA/CD to access a network at a variety of speeds over different media.  Other physical variations include

10BaseT – Cat 3 – 5 cable with one pair for sending, other pair for receiving @ 100 meters

10Base2 –Thin coaxial cable @ 185 meters

10Base5 – Thick coaxial cable @ 500 meters

The maximum distance of a 10Base5 network is which of the following?

500 meters per segment.

Which of the following is a characteristic of a switch, but not of a repeater?

Switches are network devices that filter, and forwards frames based on MAC address.  Will send broadcast to all connected ports though.

How does the cut-through switching technique work?

Packet switching where data is exiting the switch at the same time it is still entering the inbound port.

How do switches use store and forward?

This is a packet-switching technique in which frames are completely processed before being forwarded out the port.  The process includes calculating the CRC and checking the destination address.  Frames are stored until network resources are available.

Choose all of the following that are needed to support full-duplex Ethernet.

# Full duplex NICs

Loopback and collision detection disabled.

What two types of technology does 100BaseT use?

CSMA/CD and 802.3u.  100-Mbps baseband Fast Ethernet using UTP.  Sends pulse signals over the network segment when no traffic is present.

Choose all of the following that are advantages to segmenting with routers.

Manageability, Flow Control, Explicit packet lifetime control, and Multiple active paths.

Some advantages to segmenting with Bridges are?

Manageability, reliability, and scalability.  Bridges do not do datagram packet filtering.

Which two of the following describe frame tagging?

Unique ID placed in header of each frame as it travels the switch fabric.

A user-assigned ID defined to each frame.

Used in VLANs to uniquely identify each frame.

Which of the following describes a full-duplex transmission?

Uses a point to point connection from the transmitter of the transmitting station to the receiver of the receive station.  Simultaneous data transmission between a sending and receiving station.

If a frame is received at a switch and only the destination hardware address is read before the frame is forwarded, what type of switching method are you using?

Cut-Through Switching.  The device reads, processes, and forwards packets as soon as the destination address is looked up and the outgoing port determined.  On-the-fly.

Which is true regarding store-and-forward switching method?

Latency depends on frame length.  Once received, the process calculates the CRC, checks the destination address, and temporarily stores the message.  This has a higher latency period due to lag time in the router.

Which of the following switching types is the default for Cisco 5505s?

Store and forward.

What is the IEEE specification for Spanning Tree?

802.1d.  STP is a bridge protocol that uses spanning-tree on a learning bridge to dynamically work around loops in a network topology by creating a spanning tree.  Bridges exchange BPDU messages with other bridges to detect loops, and then shutdown interfaces that are looping.

What does the Spanning-Tree Algorithm (STA) do?

STA is implemented by STP to prevent loops.  Used by Spanning-Tree protocol to create a spanning tree.

Which can be true regarding VLANs?

They are created by location, function, group, and department.  Virtual LANs are a group of devices on one or more LANs that are configured so they communicate as if they were attached to the same wire when they are actually located on different segments. VLANs are based on logical grouping instead of physical and are very flexible.

Of the three switching types, which one has the lowest latency?

Cut-Through.

Of the three switching types, which one has the highest latency?

Store and Forward.

Chapter 3

DOD Model

Process/Application Layer (OSI Application, Presentation, Session) – defines protocols for node-to-node application communication and controls user interface specs.

Host-to-Host (OSI Transport Layer) – defines protocols for setup of transmission service for applications.  Creates reliable end-to-end communication and ensures the error-free delivery of data.  Covers packet sequencing and data integrity.

Internet (OSI Network Layer) – gives hosts IP addresses, handles routing of packets, and communication flow between hosts.

Network Access (OSI Data Link and Physical Layers) – oversees hardware addressing and defines protocols for the physical transmission

UDP does not create a virtual circuit, and does not contact the destination before delivering info.  But it also does not trust the lower layers and runs its own CRC.

When IP sends a datagram, it has been informed from the upper layer of the IP address. However, IP must also inform a Network Access protocol like Ethernet or Token Ring of the destination hardware address.  This is where ARP finds the hardware address for Ethernet.

DSAP – Destination Service Access Point - The SAP of the network node designated in the destination field of a packet.

SSAP – Source Service Access Point – The SAP of the network node designated in the source field of a packet.

Which frame type use DSAPs and SSAPs to identify the upper-layer protocol?

802.2   802.2 frames are really 802.3 frames with Logical Link Control header information.

What is the port number for TCP? UDP?

TCP – 6 and UDP – 17.  Network layer uses port 6h for TCP and port 17h for UDP.

Which of the following protocols are used to get an IP address from a known MAC address?

RARP – Reverse Address Resolution Protocol.

What does the acronym ARP stand for?

Which protocol gets a hardware address from a known IP address?

Address Resolution Protocol.  Used to map IP addresses to MACs.

Which protocol works at the Internet layer and is responsible for making routing decisions?

IP.  It looks at the destination network address in the packet, forwards it based on the routing tables, and determines the best route to the destination.

Which port numbers are used by TCP and UDP to set up sessions with other hosts?

1023 and above.  A port number is also a well known address and works for connectivity from lower layers.

User Datagram Protocol works at which layer of the DOD model?

Host-to-Host.

Which protocol sends redirects back to an originating router?

If a router in your internetwork experienced congestion on serial port 0, which protocol will let the neighbor routers know?

Internet Control Message Protocol - ICMP.  Network layer Internet protocol that reports on errors and provides info for IP packet processing.

Which protocol will send a message to routers if a network outage or congestion occurs?

ICMP – alerts routers if a network outage or congestion occurs in order to make better routing decisions.

Which protocol is used to manage and monitor the network?

SNMP provides means to monitor and control network devices.

Which frame has a Type field to identify the upper-layer protocol?

Ethernet_II has a type field to identify upper-layer protocols.  802.3 has a length field instead, but cannot see upper layers.

Ping uses which Internet layer protocol (besides IP)?

Ping – Packet Internet Groper.  ICMP echo message and its reply.

Which of the following protocols is used to give an IP address to a diskless machine?

IP.  Network layer offers a connectionless internetwork service.  IP provides for addressing, type-of-service, fragmentation, and security.

Which two of the following protocols are used at the Transport layer?

# TCP and UDP.

# Transmission Control Protocol – TCP is connection oriented, reliable and uses sequencing and acknowledgements to deliver properly.

User Datagram Protocol – UDP is unreliable and does not use sequencing or acks.

Which of the following is a connectionless protocol at the Transport layer?

User Datagram Protocol.  Connectionless transport layer.  Cannot handle error correction or retransmission.

Which protocol is used for booting diskless workstations?

Bootstrap Protocol.  Used by a network node to determine the IP address of its Ethernet interfaces in order to boot to network.

Chapter 4

Entire set of IP address set to all 0s is Cisco’s default route.

Sometimes difficult to create small subnets with just a few host ID’s per subnet.  If you are running EIGRP or OSPF it is possible to use VLSM – Variable Length Subnet Masks.  This will allow you to run .252 on your WAN interface, and run .255 on the router interfaces to your LAN.  This works because these routing protocols transmit their routes via packets to the other routers.

You come up with an IP addressing scheme.  You must have a network ID for each subnet, and each WAN (router interface) connection.

When defining the subnet mask, you must consider the number of subnets that the network will need, and the number of host ID’s they will need.  They will need one host ID for each router interface and network adapter install on each host.

Calculating Subnets/Hosts – Network ID then Subnet ID then Host ID portion of the address.  After the SM is calculated, take remaining bits on right (x) to calculate the number of hosts with 2^X – 2 = Hosts.  Then the left side, minus the network portion of the ID is 2^Y – 2 = Subnets Available.

Calculating Range for Hosts – Determine what subnet mask would be using last octet number of bits to subnet mask and add them up to get mask value.  Then subtract that value (IE 256 – 240 = 16) and 16 would be your first available subnet with a range of 16.  Count up to IP Address and range will have a broadcast address in it.  Do not forget to get rid of the loopback address at 127.

If you have a class B network with a 10-bit subnet mask, how many subnets and how many hosts do you have?

1022 subnets and 62 hosts

If you have a 22-bit subnet mask, how many subnets and how many hosts do you have?

494,302 subnets, 2 hosts

You have an IP address of 172.16.13.5 with a 255.255.255.128 subnet mask. What is your class of address, subnet address, and broadcast address?

Class B, Subnet 13, Broadcast 255.255.255.127.  Range 172.16.13.1 to 172.16.13.126

You have a subnet mask of 255.255.255.248. How many subnets and hosts do you have?

8190 subnets with 6 hosts

If you have a class C network with a 6-bit subnet mask, how many subnets and how many hosts do you have?

62 subnets and 2 hosts

You have an IP address of 172.16.3.57 with an 11-bit subnet mask. What are your valid hosts?

172.16.3.33 to 172.16.3.62

You have a Class C network address of 192.168.88.0 and you need the largest possible number of subnets, with up to 12 hosts per subnet. Which subnet mask should you assign?

255.255.255.240

You have a Class C network address divided into three subnets. You will need to add two subnets in the next two years. Each subnet will have 25 hosts. Which subnet mask should you assign?

255.255.255.224

You have a network ID of 192.168.1.0 and you need to divide it into nine subnets. You need to provide for the largest possible number of host IDs per subnet. Which subnet mask should you assign?

255.255.255.240

You have a Class B network address divided into 30 subnets. You will add 25 new subnets within the next two years. You need 600 host IDs for each subnet. Which subnet mask should you assign?

255.255.252.0

You have a Class C network address of 192.168.19.0 with four subnets. You need the largest possible number of host IDs per subnet. Which subnet mask should you assign?

255.255.255.224

You have a Class A network address with 60 subnets. You need to add 40 new subnets in the next two years, but still allow for the largest possible number of host IDs per subnet. Which subnet mask should you assign?

255.254.0.0

You have a network ID of 192.168.55.0 and you need to divide it into multiple subnets. You need 25 host IDs for each subnet, with the largest amount of subnets available Which subnet mask should you assign?

255.255.255.224

You have a network ID of 172.16.0.0 with eight subnets. You need to allow for the largest possible number of host IDs per subnet. Which subnet mask should you assign?

255.255.240.0

You have a network ID of 172.16.0.0 and you need to divide it into multiple subnets. You need 600 host IDs for each subnet. Which subnet mask should you assign that will allow for growth?

255.255.252.0

You have an IP address of 172.16.4.58 with a 12-bit subnet mask. What are your valid hosts?

172.16.4.49 to 172.16.4.62

If you have a 19-bit subnet mask, how many subnets and how many hosts do you have?

524,286 subnets, 30 hosts

Chapter 5

ROM used by the router to store the bootstrap startup, the OS and the POST.

FLASH is an erasable ROM that holds the OS image and the microcode.  Retained even if the router is turned off and is the default load.

RAM provides caching and packet buffering.  Cleared when the router is turned off.

NVRAM stores the routers startup configuration file.  Retains info if router is turned off.

Interfaces are located on either the motherboard or as separate modules for upgrades.  If no NVRAM, the router enters the “question driven config dialog.”

Executive Command Interpreter

- User mode is for ordinary tasks like checking status and viewing system info.

- Privileged Mode has both user mode commands and allows access to test, debug, and access global configurations.  Enter using “en” or “enable”

CTRL+A beginning of command line.

CTRL+E end of command line.

Tab completes the entry for you.

2 Basic Configurations for each router:

- Startup – held in NVRAM and accessed when the router is started and places config into DRAM.  Type “sh startup-config” or “sh star”

- Running – type in “config t” and you can make changes to the running configuration.  When you have it set the way you want, type “copy running-config startup-config” and this new edit of the config will be the new startup config.

Erase startup-config will kill everything in NVRAM and return to the initial configuration dialog.

Virtual Terminal Password  “line vty 0 4, login, password <password>, ^Z”

Aux Port Password “line aux 0, login, password <password>, ^Z”

Console Password “line con 0, login, password <password>, ^Z”

Banner “banner motd #, <text message> #, end”

“Hostname router A, ^Z”

Change the administrative state of the router use shutdown, and no shut.

“write erase” or “erase startup-config” then can run reload.

By default Cisco router are DTE devices, yet without a CSU/DSU to control the clocking, we can set another router to run as a DCE device and set clock rate.

What is the syntax to add a banner to a Cisco router configuration?

Banner motd #.  Type in message and end with #.

What command do you use to change your enable password?

In config mode, “enable password <password>”

How do you change your enable secret password?

In config mode, “enable secret <password>”

Which of the following will change your Telnet password?

In config mode, line vty 0 4 <R>, login <R>, password <password>”

What command can you use to copy the configuration from NVRAM into running RAM?

“copy star run” to put the config into RAM.  Move new config to startup with “copy running-config startup-config”.

What is the syntax for changing the name of a Cisco router?

Hostname <routername>

To exit from privileged mode back to user mode, what do you type at the privileged mode prompt (#)?

Type disable to exit privileged mode.

What is the AUX port used for?

Modem connections for a console or a dialup connection for temporary Dial on Demand Routing (DDR).

When attaching a console cable to your router, how do you log in to user mode?

Press return, type password if prompted.

If the advanced editing feature has been disabled, how do you then enable the advance editing features?

Type “terminal editing”.  However advanced editing is enabled by default.  Turn off using “terminal no editing”.

CTRL+A will provide what function?

Takes the cursor to the beginning of the line for editing.

What key do you use to view the last command that was entered into a Cisco router?

CTRL-P or the up arrow will allow you to scan recent commands entered.

What key do you press to have the Cisco IOS finish typing a command for you?

TAB will complete a command word.

What does the erase startup-config command do?

Erases the startup-config.  Same as wr erase.  This will erase the config in the router’s NVRAM and land back into the initial config dialog.

What is the command to set the clock rate on your DCE interfaces?

clock rate 5600 command will not work unless there is a space between.

If you have two Cisco routers connected with DTE/DCE cables, to which router would you

add the command clock rate in order to facilitate a CSU/DSU?

The serial interface running as a DCE.  Cisco routers run as DTE devices by default. CSU/DSUs connect to the serial interfaces and give clock information.  2 routers together, you need to clock rate the router that has the DCE interface.  You can see which router has the DCE interface by typing sh s x.

What is the syntax to configure a port on a 7000 series router with a VIP card?

“type slot/port adapter/port”  The 7000 VIP creates virtual ports for WAN.  You must identify the slot/port and the adapter/port.

The terminal no editing command will provide what function?

Stop the advanced editing features.

What is the syntax you would use to configure a port on a Catalyst 5000 switch?

“type slot/port”.  Switch is different than a router.  You need to configure the card and the port.

## Chapter 6

Static routing where the network is reachable by only one path is a stub network.

To set a default path out of the network type “ip route 0.0.0.0 172.16.20.2” This tells the router that if it doesn’t know what to do with a datagram, to send it to the .2 machine.

IP Classless – set on a machine when you have an internal network number or subnet that the router cannot find, send it to x interface and that machine will know how to handle.  This will prevent the router from dropping packets and sending to an interface of last resort.

What is an administrative distance of 0?

0 is the default administrative distance for directly connected routes.  The router trusts a 0 distance the MOST.

What is the administrative distance used for in static routes?

To rate the source’s trustworthiness.  Same as dynamic, it assigns the weighted averages of the links, 255 is the last resort, least trusted.

Static routes are used for which of the following?

Defining a path to an IP destination network.  Building routing tables to remote networks.

What is the command that you should use when using static and default routes with your Cisco routers?

The ip classless command tells the router to expect subnetted internetworks on its interfaces.  Default is classful mode, which means they look for an entire address class on each interface and do not consider the subnets when making routing decisions.

What is the command syntax to set a gateway of last resort in your Cisco router?

“ip route 0.0.0.0 0.0.0.0 Next hop address” sets a gateway of last resort, or default.

Which Cisco IOS command can you use to see the routing table?

“sh ip route” shows the IP protocol routing table maintained in the router. Sh ipx route will show IPX.

What are three ways that routers learn paths to destinations?

Static, default or dynamic routing.

When should you use static routing instead of dynamic routing?

When you have very few routers and want to save bandwidth.  Dynamic routing takes up a great deal of bandwidth.  On a slow WAN link static routes may be a better solution.

What are three ways to build routing tables?

Router learns routes by default, statically, or dynamically.

### What is the command syntax for creating an IP static route in a Cisco Router?

IP route destination_network   subnet_mask   default_gateway

How do you create a default route?

By using all zeros to specify the remote network and the subnet mask.  The router will use this route as the gateway of last resort.

When looking at a routing table, what does the “S” mean?

# Statically connected

What is true about IP routing?

A device will send a frame with the hardware destination or the default gateway.  The router will strip the frame and put the datagram in a new frame with the new remote destination address.

What static route parameter will tell a router the name of the interface to use to get to a destination network?

INTERFACE  “ip route <destination network> <SM> <DG> INTERFACE <Administrative Distance>”

The interface parameter is rarely used, but can be used to tell a router what interface to use for a route to a remote network.

When creating a static route, what is the gateway parameter used for?

Defining the next hop.

What does a router do with a received packet that is destined for an unknown network?

It will drop the packet and send an ICMP reply to the sending host.

What is true when creating static routes?

Gateway is required, the administrative distance is optional.

“ip route <destination network> <SM> <DG> INTERFACE <Administrative Distance>”

Interface not required.

When looking at a routing table, what does the “C” mean?

# Directly Connected

### Chapter 7

Novell Link-State Protocol (NLSP)

Interior Gateway Routing Protocol – (IGRP) and Extended IGRP – Both Cisco proprietary

These routing protocols only pass router information between routers.

Autonomous System (AS) – A routing domain or autonomous system, is a portion of an internetwork under common administrative authority and consist of router that share information using the same routing protocol.  Routing within a domain or area is called intra-domain routing.  If the AS is on the Internet, InterNIC will assign you a unique 16-bit number that you use to configure your system.

Convergence Time is the period it takes for all the routers to update their tables after a configuration change.

Distance-Vector updates every 30 – 90 seconds.

Link State Packets (LSPs) - used to inform other routers of distant links and keep them updated on status.  They use time-stamps to determine what order changes occur in.

When a link-state router comes online, it floods the network with LSPs and reduces bandwidth.

Designated Router (DR) –.  Used as the target to consult for all changes as a direct request and lowers the number of LSPs on the wire.

Link-State protocols do create a lot of traffic on initial startup, but only update every 2 hours afterward, unless the internetwork changes.

Balanced Hybrid protocols combine the best of both.

##### RIP uses time for several functions

# Routing Update Timer - sets 30 seconds for updating routing tables.

Route Invalid Timer - determines the length of time (90secs) before a router sets a route to invalid.

Route Flush Timer - set time between a route becoming invalid and removal from the routing table.

IGRP developed by Cisco to overcome problems with RIP in larger AS.  Has a hop limit of 255.  Reliability set to fractions of 255 (255 optimal).  Uses multi-path routing for redundancy.

##### IGRP Timers

# Update Timers - set 90 second updates.

Invalid Timers – Time to wait before setting a route to invalid

Hold-Down Timers – 190 seconds default to hold down updates on a route.

Flush Timers - indicate how much time should pass before a route will be flushed from the table.

## IGRP 3 types of routes

# Interior - are routes between subnets

System - are routes to other networks inside the common AS

Exterior - are routes outside the AS

Typically the router will trust a route from IGRP before RIP due to admin distance of 100 for IGRP and 120 for RIP

##### Protocols within an AS

OSPF – Link state that can balance loads across links and reduces broadcast traffic.  Can run Variable Length Subnet Masks (VLSM) - administrators can set costs associated to particular links.

EIGRP – Combines D-V and link state so is considered a balanced hybrid.  Fast convergence, VLSM, and multi-protocol support.  After initial config, it sends only updates across wire.

Exterior Routing Protocols between Autonomous Systems.  Are more complex due to the passing of more information on larger number of routers.

EGP – Exterior Gateway Protocol does not use hops, and cannot detect or correct routing loops.  Can only be used for connection between a central core AS that has connections to two other AS’s. Talks to neighbors to determine status.

BGP – Border Gateway Protocol can be used between and within AS’s and can find routing loops.  It doesn’t need a core like EGP, because all areas can intercommunicate.  Incremental routing changes are light on BW.  Will advertise only what it sees as the best route.

Passive RIP when you do not want to advertise your route to the Internet out a particular interface.

Frame Relay will not broadcast RIP updates by default.

IGRP configuration must use the same AS number with all the routers that it will communicate with.

What is the routing algorithm used by RIP?

Distance Vector – based on Hop Count.

What is the routing metric used by RIP?

# Hop count

What is the routing algorithm used by IGRP?

D-V and hop counts.

Which command can you type at the router prompt to verify the broadcast frequency for IGRP?

“sh ip protocol” will show broadcast frequency of routing protocols.

Which utility should you use to identify the path that a packet takes as it passes through the routers?

Tracert

What is the routing metric used by IGRP?

Bandwidth, reliability, MTU, delay, and load.  IGRP can also use hop counts to determine the best route to a remote network.

What does a metric of 16 hops represent in a RIP routing network?

16 hops – unreachable.  15 hops is the last number you can have before becoming an unreachable network.

Hold-downs are used for what?

To prevent regular update messages from reinstating a route that has gone down.  Router is placed into state where it will neither advertise the route nor accept advertisements for a specific amount of time.  This will flush bad info about a route from all the routers on the network.  Happens often when a route fails.

What is split horizon?

When a router differentiates which interface a packet arrived on.  Routing technique that prevents the router from sending info through the same interface from which it was originally received.  Another tool against loops.

What is poison reverse?

When a router sets the metric for a down link to infinity.  Routing updates specifically indicate that a network is unreachable.  Sent to defeat large routing loops.

What are the three types of routes that IGRP advertises?

Interior, system and exterior routes.

What is the metric limit for link-state protocols?

65,533

Which of the following are distance-vector protocols?

RIP and IGRP are both D-V using hop counts.

Which of the following routing protocols use Autonomous Systems?

IGRP, EIGRP, OSPF, NLSP can all create autonomous systems.  AS’s are a collection of networks under common administration.  Subdivided into areas, and must have a unique 16-bit number provided by the IANA if used on the Internet.

What is true about link-state networks?

They maintain a more complex table than distance-vector based networks.  Typically more complex than D-V.

What commands are available for supporting RIP networks?

“sh ip route, sh ip protocol, sh ip int, and debug ip rip” are all ways of supporting a RIP network.

What is convergence time?

The speed and ability of a group of internetwork devices running a specific protocol to agree on the topology after a change takes place.

What is true about distance-vector-based networks?

They send their entire routing table every 30 seconds.  RIP networks send their entire routing tables across the wire every 30 seconds and is very chatty.

What is the default administrative distance of RIP?

120

What is the default administrative distance for IGRP?

100

### Chapter 8

Cisco has TFTP server software that can be run on a 9x box.

Three places a router can look for a valid Cisco IOS - Flash, TFTP Server or ROM.

To copy the current config to a TFTP server type “copy running-config tftp”.  To load a copy type “copy tftp run”.

AutoInstall – set the router to act as a BootP workstation and connect to an existing router that is acting as a BootP server. The new router will be assigned an IP address, and then the router will be sent to a DNS server to resolve the IP address to a hostname.  Then the new router will be sent to the TFTP server for its configuration.

Show memory is used to see how the system allocates memory for different purposes.

Show stacks monitors the stack use and if the reboot was the result of a system crash, and displays the last system reboot.

Show Buffers reveals the size of the buffers (S, M, Big, VeryBig, L, and Huge)

Show Flash describes the flash memory and the size of files and how much memory is free.

Cisco Discovery Protocol (CDP) – allows you access to configuration information on other routers with a single command.  Running Subnetwork Access Protocol (SNAP) at the Data Link layer, two devices running different Network Layer protocols can still communicate.  CDP runs by default on 10.3 versions and earlier.  Once a router is found, it can display information about the upper-layer protocols.  Find by typing “sh cdp int” will show interfaces configured to run CDP.

Telnet is a virtual terminal protocol.

“sh hosts” will show all the names of routers that your router knows about, assuming DNS is running on router or on a server.  Router as a DNS server, use “ip domain-lookup, and “ip name-server ip_address”

You just received an output that states the CDP hold time, hardware, port ID, and local interface. What was the command you typed in?

“Show cdp neighbor” will show the hardware platform (Cisco 2500) the local interface the routers are connected through, the hold time, the port id of the remote router and the device id and its capability.

What’s the default CDP hold time in seconds?

180 - 3 times the default broadcast frequency, which is 60 seconds

What's the default CDP update broadcast rate for routers in seconds?

60 seconds.  Changed with the cdp timer command.

What type of frame does CDP use to gather information about its directly connected neighbors?

CDP uses SNAP by default.

Which command do you type to view the hostnames configured in your router (choose two)?

Show hosts or sh host will show the ip host table configured on the router.

How can you view the CDP information received from all routers?

Show cdp entry <router> or sh cdp entry * will show you entries of CDP information received from the neighbor routers.

If you want to type in the hostname Bob instead of the IP address 172.16.10.1 to access the remote router named Bob, what should you do?

“config t, ip host bob 172.16.10.1”.

If you type “copy tftp flash”, which event did you cause?

Copied a file from TFTP server to router flash.  The router will look to a TFTP host for a valid Cisco IOS to copy into EEPROM, or Flash.

What command will allow you to load a Cisco router configuration that is stored on a TFTP server into working ram?

“config net <ip address of TFTP>” this will place a backup config into working RAM.

If you want to load a new Cisco IOS into your router's memory, which command should you use?

“copy tftp flash” tells the router to look to a TFTP server.

What does it mean if you’re running a trace and receive a “P” as a response?

Protocol unreachable

If you want to configure the router configuration stored in NVRAM, which command should you use?

“config mem” copies the startup-config into running-config.

Which command will load the Cisco router configuration into RAM (choose three)?

To reload the router, type “Reload” or “copy startup-config running-config”, or ”copy tftp running-config”.

What command will copy your router configuration to a TFTP server?

“copy star tftp” or “copy runn tftp”

What command should you use to have your router load the valid Cisco IOS from a TFTP server?

“boot system tftp <IOS file name> <ip address of tftp server>”

Which protocols can you use when testing with Trace ?

AppleTalk, Connectionless Network Service (CLNS), IP Vines, and old Vines.

Which command will you use if you want to disable DNS lookup?

“no ip domain-lookup” will disable DNS.

Which command do you use to configure your router to do a domain-lookup?

# IP name-server 10.10.10.10

How can you telnet into multiple routers but keep the sessions open all at the same time?

“CTRL/SHIFT/X” allows you to leave multiple sessions open.  Type “sh sessions” to see all that are open.

After telneting into multiple routers simultaneously, what command can you type to see these connections?

“sh sessions” will show all that are open.

### Chapter 9

IPX functions at layers 3 and 4 of the OSI model, is connectionless, and uses sockets instead of ports like TCP/IP.

SPX adds connection-oriented communications and creates virtual circuits with connection IDs between machines.

SAP – (Service Advertising Protocol) advertises services running.

NLSP (NetWare Link Services Protocol) is a link state routing protocol designed by Novell to replace both RIP and SAP.

NCP (NetWare Core Protocol) gives clients access to server resources, printing, file sharing and security.

Clients issues a GetNearestServer (GNS) request and the server looks at its SAP table to locate the server that matches the request.  Cisco routers can respond with their SAP tables and act like servers.

SAP broadcasts service information every 60 seconds, just like RIP broadcasts at 60 seconds for routing information.

Hardware addresses are part of the unique identifier string of eight hex digits for the network portion of the ID, and the last 12 hex digits represent the node, and are usually the MAC address of the machine.

Frames live at layer 2, and encapsulation is the process of taking packets from upper-layer protocols and building the frames to transmit across the network.

Unless your routers are running both frame types, your nodes will not talk due to lack of translation. Set on interfaces with the following Cisco Encapsulation Keywords on right.

E 802.2 is after 3.12 						SAP

Before 3.12 is always 802.3.					novell-ether (default)

Ethernet II runs both TCP/IP and IPX			ARPA

Ethernet SNAP is AppleTalk, IPX and TCP/IP		SNAP

To configure a router on a IPX network you will need to know the frame type and the network address information for each segment.

Enable IPX on a router by typing “ipx routing”, for a particular interface, type “ipx network number”

Run “sh ipx servers” to see if all your servers are seen in the SAP table.  If not, check your encapsulation type and network addresses.

Need to add the command “ipx maximum-paths 2” will allow the router to accept the possibility that there is more than one path to a destination.  Cisco IOS will do per-packet load sharing with parallel lines using a round-robin logic.

Which of the following provides connection-oriented transport to upper-layer protocols?

SPX, is connection, IPX is connectionless.

Which of the following can respond to a client GNS request ?

Local Netware server or a Cisco router.  GetNearestServer.  A request packet sent by the client (IPX network) is sent to locate the nearest active server of a particular type.

How often do servers exchange SAP information unless set otherwise?

Every 60 seconds.  Service Advertisement Protocol is an IPX protocol that tells clients via routers and servers of available network resources every 60 seconds.

How can you configure a secondary subinterface on your Ethernet interface?

Config t, int e0.24000

Given the IPX address 00007C81.00A0.2494.E939, which of the following is the associated IPX network and node address?

Net 00007C81 node 00a0.2494.e939.  IPX addresses use the network number and the MAC address to create a logical address for network devices.  First 2 bytes are network, and the last 3 are hardware.

What is the default Ethernet encapsulation on NetWare 3.11?

802.3

Which of the following are valid methods of including multiple encapsulations on a single interface?

Secondary, and subinterfaces can add multiple encapsulations and logical address to one interface.

Which command would you use to see if you were receiving SAP and RIP information on an interface?

“sh ipx int” can show both SAP and RIP information received and transmitted from your router.

Which command would you use to see if the router is hearing your server SAPs?

“sh ipx servers” on the router is the same as display servers on NetWare.

Which commands will allow you to display the IPX address of an interface ?

“sh ipx int e0” will show you the complete IPX address of that interface.

You want to forward IPX packets over multiple paths. What command do you use?

“ipx maximum paths” command allows a round-robin traffic flow over 2+ links to the same destination.

Which of the following are valid Cisco encapsulation names?

Arpa = 			IPX Ethernet

Hdlc = 			hdlc on serial links

Novell-ether = 		IPX Ethernet 802.3

Novell-fddi = 		IPX Fddi_Raw

Sap = 			IEEE 802.2 on Ethernet, FDDI and Token Ring

Snap = 		IEEE 802.2 SNAP on Ethernet, FDDI, and Token Ring.

Which commands must be used to enable IPX networking?

“ipx routing, int e0, IPX network 77790, encapsulation arpa”.  IPX routing is not enabled by default like IP is.

What is the default encapsulation and frame type on an Ethernet interface when enabling Novell?

Cisco still uses a default of 802.3 when configuring IPX networks.

If you are running Token Ring with Novell IPX routing, which encapsulation should you use?

SNAP is the default frame type for Token Rings running IPX.

If you want to run the 802.2 frame type on your Ethernet interface, which encapsulation type should you choose?

Cisco uses a keyword of “SAP” for the 802.2 frame type.

If you want to enable Ethernet_II frame type on your Ethernet interface, which encapsulation should you use?

Cisco keyword of “ARPA” is used for enabling the Ethernet_II frame type.

Which of the following is the correct syntax for configuring a subinterface?

# Config t, int e0.100, ipx network 100 encap arpa

Which command will show the network servers advertising on your network?

“sh ipx servers”

Which command displays the status of the IPX interfaces configured in the router and the parameters configured on each interface?

“sh ipx interface” can show RIP and SAP updates, IPX interface addresses and more.

### Chapter 10

Access lists are compared in the order of the lines, and only until a match was made.  There is also an implicit deny at the end that the packet will be thrown out if there is no match.  Usually want to place commonly matched lines at the top of the list.  The list is created and then applied to a specific interface.

“access-list <number> <permit or deny> <source address>”

1-99 		IP Standard

100-199 	IP Extended

  - Protocol type-code

  - DECNet

600-699	Appletalk

  - 48-bit MAC address

  - IPX Standard

  - IPX Extended

  - IPX SAP

  - Extended 48-bit MAC

1200-1299	IPX Summary Address

“access-group <number> <out or in>”  Use group to apply the access-list to an interface.  Only one access list is allowed in, and one outbound from the interface.  Wildcard 0.0.0.255 will give access/deny access to all nodes in the range.  Set to 0.0.0.0 will allow only that host.

In standard IP access lists, we can only compare with source address information.  In extended, we can limit via source address, destination address, protocol, and port information.

Clear access-list will clear the counters for the access list and start new.

Show ip access-list will show only IP based access lists.

Show IP interface e0 will show what access-list is applied to the interface.

In IPX no wildcard masking is required, just add the network number.  The number of the access-list will define which protocol and detail is in the list.

–1 refers to any IPX network address.  It is like using the any keyword.

Extended IPX can filter based on

# Source network/node

Destination network/node

IPX protocol (SAP, IPX etc.)

IPX Socket

0 set for the service type will allow all services.

IP standard access lists use which of the following as a basis for permitting or denying packets?

Source address

To specify all hosts in the class B IP network 172.16.0.0, which wildcard access list mask would you use?

0.0.255.255  The access list is the opposite of the IP.  If you want all hosts on the subnet for Class B, you would enter 0.0.255.255.  This accepts any address in the octet.

IP extended access lists use which of the following as a basis for permitting or denying packets?

Access list can look at the source and destination access lists when making filtering decisions, but can also filter by port and protocol.

Which of the following are valid ways to refer only to host 172.16.30.55 in an IP access list?

172.16.30.55 0.0.0.0 or host 172.16.30.55

Which of the following access lists will allow only WWW traffic into network 196.15.7.0?

Access-list 100 permit tcp any 196.15.7.0 0.0.0.255 eq www

Which of the following will show which ports have IP access lists applied?

# Show ip interface and show running config

Which of the following are logged when IP access list logging is enabled?

Source address, source port, destination address, destination port, protocol, and access list number.

Which of the following is a valid IPX standard access list?

# Access-list 800 permit 30 50

Which of the following can be logged by IPX extended access lists?

Source address, source socket, destination address, destination socket, access list number, protocol.

Which of the following will apply IPX SAP access list 1050 for incoming traffic, assuming you’re already at interface configuration?

# Ipx input-sap-filter 1050

Which of the following commands will show an extended access list 187?

# Sh ip access-list and sh access-list 187

What is the IP extended access list range?

100-199

Which of the following commands is valid for creating an extended IP access list?

# Access-list 101 permit tcp host 172.16.30.0 any eq 21 log

What is the extended IPX access list range?

900-999

What does the –1 mean in an extended IPX access list?

# Any host or any network

What are three ways to monitor IP access lists?

# Sh ip interface, sh run, and sh access-lists

Which of the following can be used to monitor IPX access lists?

# Sh access-lists, sh ipx int, and sh run

### Chapter 11

Demarc is the stop point for the service provider’s wiring.

CPE – Customer Premise Equipment is all the wiring and equipment on the customer side of the demarc.

Local Loop – the wiring running from the demarc to the central office (CO)

CO – Central Office where the local loop gains access to the service provider’s high speed trunk lines.  Also known as the POP – point of presence.

SDLC (Synchronous Data Link Control) – bit synchronous protocol run on serial interfaces for connecting remote offices using SNA.  Supports point to point and multi-point links, bounded and unbounded media, half and full duplex, and circuit and packet switched networks.

HDLC (High Level Data Link Control) – default encapsulation used by Cisco routers over synchronous serial links.  Proprietary and will not work with other vendor’s HDLC.  Supports several transfer modes that define the media access on the WAN.

NRM – Normal Response Mode is SDLC secondary cannot communicate until asked by primary.

ARM – Asynchronous Response Mode is where secondaries can send at will.

ABM – Asynchronous Balanced Mode is where nodes can act as either secondary or primary

LAPB – Link Access Procedure Balanced is limited to the ABM transfer mode and is integrated into the X.25 protocol stack.

Dial on demand routing (DDR) – is used when a data line is expensive and/or you want to limit traffic based on what kind of traffic it is.  Usually defined by static routes, and if admin weighted at 200, the static route will not be used unless all other routes are absent or unreachable.  Router determines whether a packet is interesting by access-list controls.  Usually ICMP packets etc do not meet requirements.

X.25 was originally built to run over noisy lines and is overpowered for today.  Uses X.121 addressing in which addresses are between 1 and 14 decimal digits long and defines point-to-point communications between DTE’s (Data Terminal Equipment - routers) and DCE’s (Data Circuit-Terminating Equipment – modem or CSU/DSU)

Packet Switching Exchange (PSE) – are switches that constitute the majority of a carrier’s network and handle the transfer of data between DTE devices via the X.25 packet-switched network.  Usually DTE sets up a session with another DTE and they negotiate and begin full duplex transmission terminated by either device.  Called a virtual circuit, it represents a logical, bi-directional path from one DTE to another.  There are SVC (Switched or temporary) and PVC (Permanent) types of circuits.

X.25 Protocol Suite

Packet Layer Protocol (PLP) – Network layer, manages packets exchange across virtual circuits.  5 Modes of operation –

Call Setup is used to institute only SVC between DTE’s.

Data Transfer Mode is used for both SVC and PVC’s and sets data transfer betwee 2 DTE devices with error and flow control.

Idle Mode – Used when the circuit is established but no data.

Call Clearing – used to terminate sessions only with SVC’s.

Restarting is used to synchronize the transmission between DCE and DTE.  Packet framing occurs here.

Link Access Procedure Balanced (LAPB) Data Link Layer? – makes sure that frames are error free.

Information Frames (I-Frames) – send both sequencing, flow control, error detection and recovery for the upper layers.

Supervisory Frames (S-Frames) – Control information and report status of receipt of I-Frames.

Unnumbered Frames (U-Frames) – run setup and disconnection of calls.

X.21bis – physical layer that specifies the electrical and mechanical processes for the use of physical media.

Frame Relay is popular for its use of shared bandwidth.  Many customers can share the same pipe, and utilization is much more efficient and cheaper for everyone.  Used at the Physical and Data Link Layer, it was originally designed for ISDN.  The carrier is used to provide the DCE device, the rest are DTE’s, terminals PC’s etc are all CPE.  Frame Relay virtual circuit is a logical connection between two DTE’s across a packet switched network (PSN) and identified by DLCI’s.  Most are PVC’s.

Committed Information Rate (CIR) – allows for a guaranteed amount of traffic throughput, but will allow for bursts up to T1 speeds.

Only 2 encapsulation types are available for Frame Relay – Cisco (default) and IETF for connecting Cisco to non-Cisco gear.

Data Link Connection Identifier (DLCI) - Typically circuits are referred to DLCI’s that have global meaning for the Telco, but local meaning for the end points giving us 2 different DLCI’s for each physical circuit.

Local Management Interface (LMI) – management information that tells current DLCI values, global or local significance, and the status of virtual circuits.  Three types of LMI are cisco, ansi, and q933a.  Typically want to match DLCI numbers to interfaces for easier management.

Point-to-point Protocol (PPP) – Data Link Layer protocol used over asynchronous (dial-up) or synchronous (ISDN) media.  Uses Link Control Protocol (LCP) to build and maintain data link connections using PAP or CHAP and compression over the wire.

Integrated Services Digital Network (ISDN) - Used for data, voice, and video, is a circuit switched network.  Can be Basic Rate Interface (BRI) with two 64kb ‘B’ channels or Primary Rate Interface (PRI) with twenty three 64kb channels, and one Management ‘D’ channel.

Network Termination device (NT) - NT Type 1 (NT1 - 2 wire local loop), converts the 2-wire U into the 4-wire S/T.  North American NT1 is CPE, rest of world NT1 is provided by carrier.

Terminal Equipment (BRI Only) has two types:

TE Type 1 (TE1 - ISDN device)

TE Type 2 (TE2 - non-ISDN device) TE-2 requires a Terminal Adapter (TA)

SPID = Service Profile Identifier - SPIDs two 10 digit phone # + extra digits and binds terminal to profile for a BRI and is assigned uniquely by Telco.  Only used in North America

#### For BRI -

2 Wire U Interface

4 Wire S/T Interface  (3 & 6 + 4 & 5)

Always the center wiring pairs

RJ 11       pairs 2 & 3 (4 conductor/wire)

RJ 12       pairs 3 & 4 (6 conductor/wire)

RJ 45/48  pairs 4 & 5 (8 conductor/wire)

Order does not matter (straight thru)

U Interface - Phone Switch Circuit Side.  Router with Built-In NT1.

S or S/T Interface - User Interface CPE

ISDN’s 3 Layers in OSI model

Layer 1 - Physical Layer (circuit)

Layer 2 - SPID authentication (Q.921)

Layer 3 - Call Setup (Q.931)

PPP for interface encapsulation, can run LAPB, X.25, HDLC

ISDN Protocols

E – ISDN on existing telephone network

I - Specify concepts terminology and services

Q – specifies signaling and switching.

Which of the following best describe X.21bis?

Physical layer protocol used in X.25

When would you use ISDN?

To support applications requiring high-speed voice, video, and data communications.

How many frame relay encapsulation types are available with Cisco routers?

Two, Cisco and IETF

How many LMI types are available?

Three, Cisco, ANSI, and Q.933a.

Regarding frame relay, which of the following statements are true?

You must use IETF encapsulation if connecting to non-Cisco equipment.

What is the default LMI type?

Cisco

Which of the following uses a PVC at layer 2?

Frame Relay

Which ISDN protocol prefix specifies switching?

Q

If you wanted to view the DLCI and line speed for your frame relay network, which command would you use?

Sh int

Which of the following are valid WAN terms?

DTE, CPE, demarc, and DCE

What does the ISDN Basic Rate Interface provide?

Two 64-Kbps B channels and one 16 Kpbs D channel.

What is true about frame relay DLCI?

DLCI identifies a logical connection between DTE devices.

Which ISDN protocol specifies basic telephone service?

E

What does ISDN PRI support?

23 B channels and one 64 Kbps D channel

Which command will list all configured PVC’s and DLCI’s?

Sh frame pvc

What is the default encapsulation on point-to-point links between two Cisco routers?

HDLC

What are HDLC transfer modes used for?

Media Access

What is true when using DDR?

You must use static routing.

What information is provided by the LMI?

The status of virtual circuits, the current DLCI values, and the global or local significance of the DLCI values.

What ISDN protocol specifies concepts, terminology and services?

I

Which access configuration allows only traffic from network 172.16.0.0 to enter int s0?

Access-list 10 permit 172.16.0.0 0.0.255.255, int s0, ip access-group 10 in

If you want to capture IPX access lists being hit, what command parameter do you add to your extended IPX access list?

Log

In an IP access list, you want to refer to host 172.16.50.1.  What mask would you use to make the list as specific as possible?

0.0.0.0

IP Classfull routers assume that they know the entire network and will toss packets that don’t know.  IE In situation where a router knows all 155.16.1-10.x network and it finds a packet for 155.16.12.x will assume it knows all these networks and throws packets.

IP Classless network is set to be cheap.  This will cover bases and push any unknown packets to the default route.

NVRAM – sh config is your backup copy.  Wr erase kills backup, reload will load the empty config into RAM and enter the initial config dialog.

RAM – wr t shows the current config in RAM.

<CTL-BR> in first 60 seconds to break into box.

Administratively down means the shut command is set.  Down means that Carrier Detect is not running but not shut.  Line Protocol down means Layer 2, Data Link is not framing, no clock, circuit not running.

Run wr t will place config into RAM, wr mem will place in NVRAM and can power cycle and keep the config.  Usually want to do this at a new box and troubleshooting so that when you power cycle you don’t lose changes no put into NVRAM.

“ipx routing” will enable ipx. Point to interface (e0 – LAN interface) and type “ipx network 3”, and point to s0 – WAN and type “ipx network 5”.  “sh ipx route” to look at the routing.

Straight through ethernet cable for AUI port to hub.  Cross-over cable to daisy chain hubs, or have an MDI port.  Hub will cross cable internally by default.

IPX 802.3, 802.2 will talk to each other over routers because Layers 1 and 2 are rebuilt by routers and have their headers matched up.

Novell		Cisco

Eth_II 		ARPA 		EthV2

Eth_802.2	SAP  		802.2

Eth_802.3	Novell-Ether  	802.3 RAW

Eth_SNAP	SNAP

To define range for host addresses based on subnet mask.

128	64	32	16	8	4	2	1

Determine mask, hop range from the left side of the mask barrier and add up.  Will not be able to use the top 2 for host ID’s nor the 1st number.  This would be the range for host ID’s, and the subnet number counts up from the left side of the mask.

RIP 2 supports VLSM, but not version 1.  “router rip” at global config mode goes to routing config.  Now we have to tell it which network to advertise.  “network 159.16.0.0”  or for IGRP run “router igrp 1” with the autonomous system number.  Then “network 159.16.0.0” and this will past routing updates.

“clear int s0” to reset the port etc.

Case sensitive on router names and commands.

Apple requires removal of the network information in order to change the zone names.  IE remove cable-range and zone name, allow routers to remove from route table. 10 – 20 minutes for normal networks.

Address Gleaning is acquiring his node address in apple.  Sends out apple arps for node address, and randomly looks for a node address.  Safer to run 30-40 nodes per apple cable-range.

Apple looks for RTMP for its routing and announces routes every 20 secs, VERY chatty.

IP RIP is 30 secs, IPX RIP is 60 secs.

Novell looks for RIP, NLSP.

IP looks for external protocols RIP, OSPF, IGRP, EIGRP, or IS-IS.

Can do global config parameters at an interface prompt.

Do route summarization with OSPF using the area parameter so that you can save memory.  This allows you to keep detail info at backbone.  Point all routers to area 0 and only have their default gateway to the network in their local table.  Then overhead of all the subnetworks in area 0 is held local to the area 0 router.

“sh ip OSPF” to see the routing protocol for IP running OSPF.  Stabilize OSPF by setting a loopback interface with an IP address on it.  “int loopback 0” <R> <ip address> <subnet mask>”  Also route summarization gives OSPF stability.  Dead Time starts at 40 seconds, below 30 or down to 10, you are dropping tons of frames.

“undebug all” to kill all outside debugging that has been turned on.

“debug icmp” to run only on a protocol.

Telnet in and want to “debug ip icmp” will give you all the error messages.  Will need to turn on “terminal monitor” to dump to screen.

Telnet uses port 23.  Can telnet into another router via its IP address or Name.Can telnet into another router via its IP address or Name.

“ip host <routername> <ip address 1> <ip address 2>” will set a local host table to resolve the hostname without needing to type the IP.