1900 – Basic Properties

2 100BaseTX ports

1024 addresses

LED is Green during normal Operation

The 232 console connector is on the rear panel of the catalyst switch

Good for creating several workgroups with 10Mbps clients and 100Mbps servers

2820 – Basic Properties

24 10BaseT ports

Expansion Slot LED green/off is running POST

FDDI Modules: Fiber DAS, Fiber SAS, UTP SAS

100Base TX Module =one switched or eight shared 100-Mbps ports

Four VLANs

Tidbits:

If destination address on same port as source address packet is filtered

If you are experience FCS or Alignment errors switch to Store-and-Forward.

Secure Port = 132 addresses

To double performance of 100Mbps server use FULL-DUPLEX

Source Port Filtering restricts what addresses on what ports

Configuration of Catalyst 1900 and 2820

Menus:

| Menu | Purpose |
| --- | --- |
| Main Menu | Network Management |
| System Config Menu | Switching mode options |
| Network Mgmt Menu | IP configuration |
| SNMP Config Menu | Read and write community strings |
| Multi-cast Registration menu | Register a Multicast address |

In-band mgmt requires an IP Address

Out-of-band mgmt = console port

Port monitor enables you to route a copy of incoming and outgoing port traffic to a monitor port for analysis.

Changes on 1900: take effect immediately, might not be permament for 30 sec, when an IP address is changes must reset the switch

Catalyst 3000 Switches

| Basic Properties |  |
| --- | --- |
| Input Buffer Per High-Speed Port | 256KB |
| total buffer space for temp packet storage between communicating ports | 512KB |
| Max Switched Ports | 24 |
| Max Mac Addresses with 4MB memory | 6000 |
| Max Mac Addresses stack of 8 cat 3000’s | 6000 |
| Max switched ports at 10-Mbps is stact of 8 cat 3000’s | 192 |
| Max bandwidth of Stackport Module | 280 Mbps |

1 port on WS-X3002 100BaseT modules

4 UTP 100VG AnyLAN port is MAX on Cat 3000

1 Meter is Max Length of SCSI-2 cable for Cat 3000’s

Catalyst Matrix = 8 SCSI-2 Ports

Configuration Catalyst 3000 Switches

Switch Info Screen = set port duplex mode

Auto Switch Mode allows to switch between cut-through and store-and-forward

There are 4 filter types

VLAN Admin Config Screen -> Exam details of a Domain and make changes

The full/half duplex dip switches are over-ridden if set to software control

Troubleshooting 1900 and 2820

Port stat LED flash GREEN/OFF = device is communicating with switch

Possible cause of FCS and Alignment Errors = switch port at Full , attached device at Half-duplex

Turning of Cat switch during upgrade could corrupt firmware

CAT 2820 firmware upgrade not effect FDDI module firmware

Upgrade firmware via : tftp client, tftp server, serial connection (X-modem)

Cross over cable = when transceiver is used or hub marked with an ‘X’

## Catalyst 5000 Processors

| NMP - Network Management Process | General control of hardware, config, and Diagnostics Executes Separate instance of STP pre VLAN SNMP agent is Here |
| --- | --- |
| MCP - Master Communication Processor | On Supervisor Engine Communication between the NMP and LCP Supports download of run-time code |
| LCP - Line Control Processor | 8051 processor It processes info from MCP (using SCP) Execute boot code in ROM |
| SAINT ASIC (Ethernet) | Permits use of half-duplex and full-duplex Encapsulation of Ethernet frame before crossing bus (12 bytes of VLAN ID, port origin, FCS) Each port has its own ASIC 192KB of dedicated RAM Performs ISL Encapsulation |
| SAGE ASIC (FDDI, ATM…) | Same as SAINT but non-Ethernet |
| SAMBA ASIC | On both line modules and supervisor module On Supervisor = MASTER (support 13 line cards) On line card = SLAVE (support 48 ports) Supports broadcast suppression Counters gather statistics |
| EARL - Encoded Address Recognition Logic | Learns network addresses Associates Port, VLAN, and MAC Controls Address Aging (300 seconds) Determines where to send frame LTL helps EARL find destination Port CBL = Color Blocking Logic , Block VLANS, STP Blocking |
| PHEONIX ASIC - Supervisor III | Enable 3.6-GBPS Crossbar Fabric Layer 3 Switching with EARL II Cisco IOS Software Modular Uplink Support |

Flow Through the Switch

- A ports DMA controller stores a frames in buffer

SAINT ASIC requests Permission from BUS Arbiter

Bus Arbiter Grants Permission

Frame IS tramitted on BUS

All ports receive it

EARL tells non-destination ports to flush it

Others forward it